3GPP TSG-RAN WG2 Meeting #117 	R2-2204203

# eMeeting, 21st February – 3rd March, 2022

Title:	LS on UE capabilities for NR QoE

Response to: 	-

Release:	Rel-17

Work Item:	NR_QoE-Core

Source:	RAN2

To:	SA4, CT1

Cc:

Contact Person:

Name:	Jun Chen

E-mail Address:	jun.chen@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 agreed on the following optional UE AS layer capabilities for NR QoE:

- qoe-Streaming-MeasReport-r17: Indicates whether the UE supports NR QoE Measurement Collection for streaming services.

- qoe-MTSI-MeasReport-r17: Indicates whether the UE supports NR QoE Measurement Collection for MTSI services.

- qoe-VR-MeasReport-r17: Indicates whether the UE supports NR QoE Measurement Collection for VR services.

- ran-VisibleQoE-Streaming-MeasReport-r17: Indicates whether the UE supports RAN visible QoE Measurement Collection for streaming services.

- ran-VisibleQoE-VR-MeasReport-r17: Indicates whether the UE supports RAN visible QoE Measurement Collection for VR services.

- ul-MeasurementReportAppLayer-Seg-r17: Indicates whether the UE supports RRC segmentation of the MeasurementReportAppLayer message in UL.

Furthermore, RAN2 made the following assumptions on the support of the AS layer capabilities for QoE:

- An AS layer capability related to a service type for QoE will be indicated to network only if the UE supports the concerned capability in the application layer.

- How AS layer obtains the concerned application layer capability is based on UE implementation (with no AS specification impact).

2. Actions:

To SA4 and CT1:

ACTION: 	RAN2 respectfully asks SA4 and CT1 to provide feedback whether the RAN2 assumption can be confirmed.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #118-e 	16 - 27 May 2022	Electronic

TSG-RAN WG2 Meeting #119 	August 2022