3GPP TSG-RAN WG2 Meeting #117-e                                  R2-2202240

Online, February 21 – March 3, 2022

Agenda item:		8.3.3

Source:				Samsung

Title:					Finalizing NW switching with leaving from RRC_CONNECTED

Document for:		Discussion & Decision

1	Introduction

This contribution first tries to conclude remaining stage 3 open issues on network switching with leaving from RRC_CONNECTED for MUSIM purposes:

- Clarification on initiation of UE assistance information procedure for leaving RRC connection

- Condition of stopping the configured wait timer

Then our views are provided on raised open issues in [Post116bis-e][202]:

- FFS UE behavior when request of leaving RRC Connected is triggered for MUSIM or Power saving, but there is an ongoing procedure for Power saving or MUSIM

- Whether busy indication is supported by network or not should be indicated to UE?

2	Discussion

2.1	Remaining stage 3 open issues

When UE requests network to be released from RRC_CONNECTED for MUSIM purpose, network typically honors UE's request to leave and release it as soon as possible. Thus RAN2 agreed not to introduce a prohibit timer for network switching with leaving from RRC_CONNECTED:

4: Do not introduce a prohibit timer for RRC-level switching procedure for leaving RRC_CONNECTED state.

Upon initiation of UAI procedure for switching procedure for leaving, UE starts the configured wait timer so called 'musim-LeaveWithoutResponseTimer'. If network does not respond to it and such wait timer expires, the UE will enter RRC_IDLE autonomously. Since UAI message is transmitted over SRB1, it is a rare case for network not to receive it successfully. There is no need for UE to send multiple UAI messages while the configured wait timer is running.

Proposal 1: Clarify in the specification that the UE is allowed to report its preferred RRC state to network for MUSIM purpose once since it was configured to provide MUSIM assistance information for leaving RRC_CONNECTED.

At RAN2#116bis-e meeting, the following agreement was made:

5: The waiting timer for leaving RRC Connection state should be set to a finite value which can allow the UE to switch in a reasonable time and not delay the urgent procedures on the other NW. Network controls whether UE is allowed to use leaving RRC connection for MUSIM purposes.

Our understanding on the yellow-marked agreement is that network has a final decision whether UE is eventually allowed to leave RRC connection or not. Upon reception of preferred RRC state for MUSIM purpose, network is is NOT mandated to send the RRCRelease message to UE. If needed, network may release configuration of MUSIM assistance information for leaving RRC_CONNECTED (e.g. musim-LeaveAssistanceConfig) to UE. In this case, UE needs to stop the configured wait timer if running; otherwise, UE goes to RRC_IDLE autonomously according to [1].

Proposal 2: UE stops the configured wait timer (e.g. musim-LeaveWithoutResponseTimer), if running if musim-LeaveAssistanceConfig is set to release.

Note that draft TPs for Proposals 1-2 are provided in the Conclusion section.

2.2	Raised open issues in [Post116bis-e][202]

In [2] it is raised whether to specify UE behavior when request of leaving RRC Connected is triggered for MUSIM or Power saving, but there is an ongoing procedure for Power saving or MUSIM.

It is worthy to re-iterate that it's up to network whether UE is finally allowed to leave RRC connection or not. From a network perspective it does not matter whether 1) UE initiates UAI procedure for leaving for MUSIM while UAI message for leaving power saving was already sent to network (and vice versa) or 2) UE includes both preferred RRC states for MUSIM and power saving in a single UAI message. Thus there is no need to restrict UE implementation in the concerned scenario. It is anyway a rare case without any critical issue so no further optimization is pursued at this late stage.

Proposal 3: RAN2 does not specify any UE behavior on the interaction between power saving and MUSIM for leaving RRC connection.

It was suggested in [2] that network needs to indicate UE whether NAS busy indication is supported or not via system information. We think it is not needed because of the following reasons:

- UE can determine whether the current cell can support paging cause feature upon reception of Uu paging message

- Prerequisite of sending NAS busy indication to network A is UE has registered onto network B. According to TS 23.502 [3], AMF will indicate 'reject paging request supported' and/or 'paging restriction supported' in the Registration Accept message.

Proposal 4: Do not introduce an indication in system information to indicate whether NAS busy indication is supported or not.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals.

Proposal 1: Clarify in the specification that the UE is allowed to report its preferred RRC state to network for MUSIM purpose once since it was configured to provide MUSIM assistance information for leaving RRC_CONNECTED.

Proposal 2: UE stops the configured wait timer (e.g. musim-LeaveWithoutResponseTimer), if running if musim-LeaveAssistanceConfig is set to release.

Proposal 3: RAN2 does not specify any UE behavior on the interaction between power saving and MUSIM for leaving RRC connection.

Proposal 4: Do not introduce an indication in system information to indicate whether NAS busy indication is supported or not.

Draft TP for Proposal 1

#### 5.7.4.2  Initiation

…

Upon initiating the procedure, the UE shall:

…

1>  if configured to provide MUSIM assistance information for leaving RRC_CONNECTED:

2>  if the UE did not transmit a UEAssistanceInformation message with musim-PreferredRRC-State since it was configured to provide MUSIM assistance information for leaving RRC_CONNECTED and the UE needs to leave RRC_CONNECTED state:

3>  initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide MUSIM assistance information;

3>  start the timer T3xx, if configured, with the timer value set to the musim-LeaveWithoutResponseTimer;

Draft TP for Proposal 2

#### 5.3.5.9	Other configuration

The UE shall:

…

1>	if the received otherConfig includes the musim-LeaveAssistanceConfig:

2>	if musim-LeaveAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED in accordance with 5.7.4:

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED in accordance with 5.7.4 and stop the timer T3xx, if running:

4	Reference

[1] R2-2201997,	Running NR RRC CR for MUSIM	vivo

[2] R2-2201999,	[Post116bis-e][202][MUSIM] Open issues for MUSIM (vivo)

[3] 3GPP TS 23.502,	Procedures for the 5G System (5GS); Stage 2; Release 17