3GPP TSG-RAN WG2 #117	Tdoc R2-2202266

Electronic meeting, 2022-02-17 – 2022-03-03

Agenda Item:	8.12.2.1.1

Source:	Ericsson

Title:	Details on RRM relaxation

Document for:	Discussion, Decision

# 1	Introduction

The RedCap WID includes the following objective:

- Specify support for the following RRM measurement relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected [RAN2, RAN4]: - Specify measurement (RSRP/RSRQ) based stationarity criterion and not-at-cell-edge criterion [RAN2] - Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation. - Specify UE requirements for RRM measurement relaxation [RAN4] - No RRM measurement relaxations are specified for the serving cell.

Most of the work is completed but at least one issue for RAN2 to discuss remains. In this paper we present our suggestion for a way forward.

# 2	Discussion

Late in the discussion RAN2#116 RAN2 agreed to add a new parameter “combineRelaxedMeasCondition2”. The implementation of this parameter was discussed in the running CR for 38.304 after RAN2#116-bis. In this paper we clarify our view on how to implement the parameter.

There is a corresponding parameter for Rel-16, “combineRelaxedMeasCondition”,

-	if both lowMobilityEvaluation and cellEdgeEvaluation are configured:

-	if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and

-	if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; and

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	for any intra-frequency, NR inter-frequency, or inter-RAT frequency, if less than 1 hour has passed since measurements of corresponding frequency cell(s) for cell reselection were last performed:

-	the UE may choose not to perform measurement for measurements on this frequency cell(s);

-	else:

-	if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell, and the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; or,

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	if combineRelaxedMeasCondition is not configured:

-	the UE may choose to perform relaxed measurements for intra-frequency cells, NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to relaxation methods in clauses 4.2.2.9, 4.2.2.10, and 4.2.2.11 in TS 38.133 [8];

-	if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	the UE may choose to perform relaxed measurement for NR inter-frequency cells of higher priority, or inter-RAT frequency cells of higher priority according to relaxation methods in clauses 4.2.2.10, and 4.2.2.11 in TS 38.133 [8];

In Rel-16 the following can happen when combineRelaxedMeasCondition is configured. If both the low mobility criterion and the not-at-cell-edge criterion are configured and fulfilled, the maximum configured relaxation is achieved. As long as both criteria are configured and met the setting of combineRelaxedMeasCondition does not play any role. However, if both criteria are configured and only one of them met, while combineRelaxedMeasCondition is configured, there will be no relaxation.

In Rel-16, if configured combineRelaxedMeasCondition only affects the outcome when both criteria are configured and only one of them is met.

-	if both stationaryMobilityEvaluation and cellEdgeEvaluationWhileStationary are configured:

-	if combineRelaxedMeasCondition2 is configured:

-	if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP-Stationary after (re-)selecting a new cell; and

-	if the relaxed measurement criterion in clause 5.2.4.9.Y is fulfilled:

-	the UE may choose to perform relaxed measurements for [TBD] according to relaxation methods in clauses [TBD];

-	else:

-	if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP-Stationary after (re-)selecting a new cell; and

-	if the relaxed measurement criterion in clause 5.2.4.9.X is fulfilled for a period of TSearchDeltaP-Stationary:

-	the UE may choose to perform relaxed measurements for [TBD] according to relaxation methods in clauses [TBD];

The proposed specification text from the Rel-17 running CR seems to provide another outcome than Rel-16 since relaxation since the 2 different criteria are evaluated just once per IF-condition  (if combineRelaxedMeasCondition2 is configured). Thus, the outcomes resulting from the Rel-16 specification will not occur.

The proposal in the running CR appears to give a different result than what the corresponding Rel-16 text gives.

A straightforward solution is to reuse the specification approach from Rel-16.

Reuse the specification approach from Rel-16 for combined relaxed measurement condition.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reuse the specification approach from Rel-16 for combined relaxed measurement condition.

# 4	References

- R17 RedCap: Revised WID in RP-211574

- 38.304 CR 3GPP TSG-RAN2 Meeting #116bis-e	R2-220xxxx Online, 2022-01-17 - 2022-01-25

- R2-2201889 – Open Issue list for 304 Redcap.

- R2-210xxx	Report of 3GPP TSG RAN WG2 meeting #115-e Online 9 - 27 August 2021

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification. 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR;

# 5	Text proposal

Here is a text proposal to implement the proposal above.

-	if both stationaryMobilityEvaluation and cellEdgeEvaluationWhileStationary are configured:

-	if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP-Stationary after (re-)selecting a new cell; and

-	if the relaxed measurement criterion in clause 5.2.4.9.Y is fulfilled:

-	the UE may choose to perform relaxed measurements for [TBD] according to relaxation methods in clauses [TBD];

-	else:

-	if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP-Stationary after (re-)selecting a new cell; and

-	if the relaxed measurement criterion in clause 5.2.4.9.X is fulfilled for a period of TSearchDeltaP-Stationary:

-	if combineRelaxedMeasCondition2 is not configured:

-	the UE may choose to perform relaxed measurements for [TBD] according to relaxation methods in clauses [TBD];