3GPP TSG-RAN WG2 Meeting #117-e     			          	                           R2-2202618

Electronic Meeting, 21 February – 3 March, 2022

Agenda item:	8.8.3

Source:	CMCC

Title:	Discussion on open issues for slice based RACH configuration

WID/SID:	NR_slice

Document for:	Discussion and Decision

Introduction

This contribution will focus on the remaining issues related to slice-based RACH configuration.

Discussion

Whether the UE can be configured dedicated RACH resources

RAN2 made the following agreements in RAN2#113bis-e[1], whether the RACH prioritization parameters can be configured in dedicated RRC signalling is FFS, and connected mode is down prioritized in this WI.

- RAN2#113bis-e: scalingFactorBI and powerRampingStepHighPriority can be configured at least in SIB (FFS for dedicated RRC signalling). RAN2 will prioritize the discussion for slice specific RACH for IDLE and INACTIVE mode. And CONNECTED mode is down prioritized and can be considered if time allows. Slice specific RACH (including RACH isolation and RACH prioritization) is only applied for CBRA but not for CFRA.

And there is also an open issue 1.5 in the open issue list: Whether to support dedicated RACH resources and RACH prioritization parameters in dedicated signalling.

Our intention is that the current slice based RACH configuration in RACH-ConfigCommon could be naturally configured in either broadcast signalling for PCell and dedicated signalling for other serving cells, which follows the legacy principles. However, we observed that majority view in last meeting contributions is that RRC CONNECTED mode is not considered and not to support dedicated signalling for slice specific RACH configuration. Considering this is not an essential function, we are ready to follow the majority view for closing this WI on time and this issue can be considered in future release.

Proposal 1: For OI 1.5, R17 will not support dedicated slice based RACH resources and RACH prioritization parameters in RRC signalling for CONNECTED mode.

Conclusion

Here are the proposals for slice-based RACH.

Proposal 1: For OI 1.5, R17 will not support dedicated slice based RACH resources and RACH prioritization parameters in RRC signalling for CONNECTED mode.

References

- Report of 3GPP TSG RAN WG2 meeting #113bis-e