3GPP TSG-RAN WG2 Meeting #117	R2-2202768

Electronic, February 21 – March 3, 2022	resubmission of R2-2201216

Agenda item:	8.3.3

Source:	Sharp

Title:	RRC Connection release request procedure for MUSIM and power saving

WID/SID:	LTE_NR_MUSIM

Document for:	Discussion and Decision

1	Introduction

In this discussion paper, we discuss the issue related to RRC Connection release request procedure for MUSIM and power saving.

2 Discussion

In the latest TS38.331 running CR for MUSIM [1], the request of leaving RRC_CONNECTED procedure for power saving purpose and for MUSIM are captured independently.

TS38.331 Running NR RRC CR for MUSIM 5.7.4.2 Initiation [omit] A UE capable of providing assistance information to transition out of RRC_CONNECTED state may initiate the procedure if it was configured to do so, upon determining that it prefers to transition out of RRC_CONNECTED state, or upon change of its preferred RRC state. [omit] A UE capable of providing MUSIM assistance information may initiate the procedure if it was configured to do so, upon determining that it needs to leave RRC_CONNECTED state, or upon determining it needs the gaps, or upon change of the gap information without leaving RRC_CONNECTED state. Upon initiating the procedure, the UE shall: 1> if configured to provide its release preference and timer T346f is not running: 2> if the UE determines that it would prefer to transition out of RRC_CONNECTED state; or 2> if the UE is configured with connectedReporting and the UE determines that it would prefer to revert an earlier indication to transition out of RRC_CONNECTED state: 3> start timer T346f with the timer value set to the releasePreferenceProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the release preference; [omit] 1> if configured to provide MUSIM assistance information for leaving RRC_CONNECTED: 2> if the UE needs to leave RRC_CONNECTED state: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide MUSIM assistance information; 3> start the timer T3xx, if configured, with the timer value set to the musim-LeaveWithoutResponseTimer; 1> if configured to provide MUSIM assistance information without leaving RRC_CONNECTED and timer T3yy is not running: 2> if the UE needs the MUSIM gap or if the current MUSIM assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide MUSIM assistance information; 3> start the timer T3yy with the timer value set to the musim-PreferenceProhibitTimer; Editor's note: FFS if UE is allowed to update UAI message after the UE performs cell reselection in NW B or after the UE performs handover in NW A

Based on the Running CR, we have the following observation:

Observation 1 The request of leaving RRC_CONNECTED procedure for power saving purpose and for MUSIM are independent.

There are some differences between the request of leaving RRC_CONNECTED procedure for power saving purpose and for MUSIM. If the request of leaving RRC_CONNECTED procedure was initiated for power saving purpose, network may not release the RRC connection, for example, because there are still DL data for the UE. In this case, UE will not release the RRC connection until RRCRelease message is received. If the request of leaving RRC_CONNECTED procedure is initiated for MUSIM purpose, UE is expected to be released from the RRC connection and will release the RRC connection even without the response message from the network when the configured timer T3xx expires.

Observation 2 For the request of leaving RRC_CONNECTED procedure for power saving, network may not release the RRC connection and UE will not release the RRC connection until a RRCRelease message is received.

Observation 3 For the request of leaving RRC_CONNECTED procedure for MUSIM, UE will release the RRC connection when the configured timer T3xx expires.

Let’s consider the situation that when UE initiate a request of leaving RRC_CONNECTION procedure for MUSIM purpose (i.e. for that UE needs to switch to network B) but there is an on-going request of leaving RRC_CONNECTION procedure for Power saving purpose (e.g., UE has no UL data for transmission). In this case, according to current CR, UE can still initiate a request of leaving RRC_CONNECTION procedure for MUSIM.

Observation 4 The request of leaving RRC_CONNECTED procedure for MUSIM can be initiated when there is an on-going request of leaving RRC_CONNECTED procedure for Power saving.

The similar situation happens when UE initiate a request of leaving RRC_CONNECTION procedure for power saving purpose (e.g. for that UE has no more UL data) but there is an on-going request of leaving RRC_CONNECTION procedure for MUSIM. The request of leaving RRC_CONNECTED procedure for power saving can also be initiated when there is an on-going for request of leaving RRC_CONNECTED procedure for MUSIM

Observation 5 The request of leaving RRC_CONNECTED procedure for power saving can also be initiated when there is an on-going request of leaving RRC_CONNECTED procedure for MUSIM.

Based on observation 2, for the case that UE needs to leave RRC_CONNECTED for MUSIM, the request of leaving RRC_CONNECTED procedure for MUSIM should be initiated even if UE has already initiated the request of leaving RRC_CONNECTED procedure for power saving.

Proposal 1 The request of leaving RRC_CONNECTED procedure for MUSIM should be initiated even if UE has already initiated the request of leaving RRC_CONNECTED procedure for power saving.

But things is different when there is an on-going request of leaving RRC_CONNECTION procedure for MUSIM and request of leaving RRC_CONNECTION procedure for power saving is triggered. In this case, considering that UE has already indicate its preference of release the RRC_Connection (for MUSIM) and the RRC_Connection will be released by NW or when the configured timer T3xx expires. There is no need to indicate its preference of release the RRC_Connection for power saving to reduce signalling overhead.

Proposal 2 The request of leaving RRC_CONNECTED procedure for power saving should not be initiated if UE has already initiated the request of leaving RRC_CONNECTED procedure for MUSIM.

The TP for proposal 2 is showed in Annex.

3 Summary

In this section, we summarized our observation and proposal as showed below:

Observation 1 The request of leaving RRC_CONNECTED procedure for power saving purpose and for MUSIM are independent.

Observation 2 For the request of leaving RRC_CONNECTED procedure for power saving, network may not release the RRC connection and UE will not release the RRC connection until a RRCRelease message is received.

Observation 3 For the request of leaving RRC_CONNECTED procedure for MUSIM, UE will release the RRC connection when the configured timer T3xx expires.

Observation 4 The request of leaving RRC_CONNECTED procedure for MUSIM can be initiated when there is an on-going request of leaving RRC_CONNECTED procedure for Power saving.

Observation 5 The request of leaving RRC_CONNECTED procedure for power saving can also be initiated when there is an on-going request of leaving RRC_CONNECTED procedure for MUSIM.

Proposal 1 The request of leaving RRC_CONNECTED procedure for MUSIM should be initiated even if UE has already initiated the request of leaving RRC_CONNECTED procedure for power saving.

Proposal 2 The request of leaving RRC_CONNECTED procedure for power saving should not be initiated if UE has already initiated the request of leaving RRC_CONNECTED procedure for MUSIM.

4 References

[1] [Post116-e][233][MUSIM] Running NR RRC CR for MUSIM (vivo)

Annex(draft TP)

TS38.331-g70

5.7.4.2	Initiation

[omit]

1>	if configured to provide its release preference and timer T346f is not running and timer T3xx is not running (if configured):

2>	if the UE determines that it would prefer to transition out of RRC_CONNECTED state; or

2>	if the UE is configured with connectedReporting and the UE determines that it would prefer to revert an earlier indication to transition out of RRC_CONNECTED state:

3>	start timer T346f with the timer value set to the releasePreferenceProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the release preference;