3GPP TSG-RAN WG2 Meeting #117	R2-2202770

Electronic, February 21 – March 3, 2022

Agenda item:	8.3.3

Source:	Sharp

Title:	Stop using of MUSIM Gap requested to be released

WID/SID:	LTE_NR_MUSIM

Document for:	Discussion and Decision

1	Introduction

RAN2 had following agreement on configuration and release of the switching gap [1-3]:

- RAN2-115e Agreements - Periodic/Aperiodic/autonomous Gap configuration and activation 8: The switching gap configuration will explicitly provide the gap starting position (e.g. offset value or start SFN and subframe explicitly), gap length and gap repetition period. 10: Switching Gaps (of any type) are configured or released by RRC signalling (e.g. RRCReconfiguration message) in Rel-17. FFS if gap can be released autonomously by UE after N repetitions. Gap configuration assistance information 16 UE is allowed to include assistance information for setup or release of gaps for both 1) periodic gaps and 2) aperiodic gap in one UEAssistanceInformation Msg. 18 To report the assistance information, the UE maps the timing info of the Gap on the network B to the network A and reports the mapped timing info to the network A. 20 For the gap assistance information, the Gap start time, Duration of the gap and gap repetition period (for periodic) may be included. FFS is other information is included (e.g. gap purpose).

- RAN2-116e agreement 6: FFS how UE indicates release of gap pattern.

- RAN2-116bis-e agreement Stage-3 details for gap configuration (e.g AddModReleaseList, gap id, gap modification) are postponed for now (pending the general MG discussion). Can consider P8/P9 as starting point from MUSIM perspective. - Proposal 8: [17/20] Adopt the list with ToAddModList/ToReleaseList for the scheduling gap configuration Proposal 9: [15/19] Introduce gap ID in RRCReconfiguration message for MUSIM to identify each configured gap, and support modification or release of gaps via gap ID.

In this discussion paper, we discuss the issue related to when UE should stop use of the MUSIM gap that requested to be released.

2 Discussion

For UE with MUSIM, UE can request for no more than three scheduling gaps in NW A for NW B. UE does not expect to be scheduled by NW A during the scheduling gaps configured by NW A towards NW B. In the scheduling gap duration, UE will switching to NW B and does not monitor the PDCCH in NW A.

According to the offline discussion [4] and the agreement [3], majorities support that scheduling gap should be configured and released by gNB. But after sending the UAI message to request release of a scheduling gap, when the UE should stop using of the scheduling gap has not been discussed yet.

Following two alternatives can be considered:

Alt.1: UE stops using the scheduling gap if a release indication is received from the network.

Alt.2: UE stops using the scheduling gap upon sending the UAI message used to request release of the scheduling gap.

The Scheduling gap is no more needed

UAI (request to release the Scheduling gap)

Scheduling gap

UE cannot be scheduled during this Scheduling gap

RRCReconfiguration (release of the Scheduling gap)

Time

Fig. 1

With Alt.1, UE cannot be scheduled during the scheduling gap even if it is no more needed, as showed in Fig.1, which will reduce the throughput of current network.

Observation 1 UE cannot be scheduled during the scheduling gap requested to be released if UE stop using the gap pattern only when a release indication is received from the network, which will reduce the throughput of current network.

On the contrary, with Alt.2, UE can be scheduled during the scheduling gap after sending the UAI message used to request release of the scheduling gap, which can improve the throughput of current network. So,

Proposal 1: UE should stop using of a scheduling gap when a UAI message used to request release of the scheduling gap is transmitted.

The TP for the proposal 1 is in Annex.

3 Summary

In this section, we summarized our proposals and observation as showed below:

Observation 1 UE cannot be scheduled during the gap pattern requested to be released if UE stop using the gap pattern only when a release indication is received from the network, which will reduce the throughput of current network.

Proposal 1: UE should stop using of a scheduling gap when a UAI message used to request release of the scheduling gap is transmitted.

4 References

[1] RAN2#115e chairman’s note.

[2] RAN2#116e chairman’s note.

[3] RAN2#116bis-e chairman’s note.

[4] R2-2201706 [AT116bis-e][231][MUSIM] MUSIM gap details

Annex (draft TP)

TS38.321-g70

5.14	Handling of measurement gaps and Scheduling gaps

During a measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5], or

During a Scheduling gap that does not request to be released, the MAC entity shall, on all the Serving Cell(s):

1>	not perform the transmission of HARQ feedback, SR, and CSI;

1>	not report SRS;

1>	not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2;

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running:

2>	monitor the PDCCH as specified in clauses 5.1.4 and 5.1.5.

1>	else:

2>	not monitor the PDCCH;

2>	not receive on DL-SCH.