3GPP TSG-RAN WG2 Meeting #118-e	R2-2204627

eLocation, 9th – 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | xxx | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE location in Non-Terrestrial Networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Corrections of UE location aspect for Non-Terrestrial Networks |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolving Editor’s notes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of UE location in NTN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.14.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: Stage 2 CR creation |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Changes Begin >>>>>>>>>>>>>>>>>>>>

## 16.14	Non-Terrestrial Networks

### 16.14.1	Overview

The Figure 16.14.1-1 below illustrates an example of a Non-Terrestrial Network (NTN) providing non-terrestrial NR access to the UE by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload.

Figure 16.14.1-1: Overall illustration of an NTN

NOTE 1:	Figure 16.14.1-1 illustrates an NTN; RAN4 aspects are out of scope.

The NTN payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The following connectivity is supported by the NTN payload:

-	A gNB may serve multiple NTN payloads;

-	An NTN payload may be served by multiple gNBs.

NOTE 2:	In this release, the NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

For NTN, the following applies in addition to Network Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 16.14.5.

Non-Geosynchronous orbit (NGSO) includes Low Earth Orbit at altitude approximately between 300 km and 1500 km and Medium Earth Orbit at altitude approximately between 7000 km and 25000 km.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the gNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while gNB operating with GSO satellite can provide Earth fixed cell coverage.

In this release, the UE supporting NTN is GNSS-capable.

In the case of NGSO, service link switch refers to a change of serving satellite.

The support for Non-Terrestrial Networks (NTNs) is facilitated by the mechanisms described in the following clauses.

### 16.14.2	User Plane aspects

The UE may be configured to report the UE's Timing Advance:

-	during Random Access procedure in Idle/Inactive state;

-	in connected mode:

-	using event-triggered reporting;

-	for RRC re-establishment procedure, if an indication is broadcasted by the target cell's SI;

-	for handover, the UE should trigger TA report if the target cell indicates this in the handover command.

To accommodate the long propagation delay, Use Plane procedures are adapted as follow:

-	For downlink, HARQ feedback can be enabled or disabled per HARQ process;

-	For uplink, the UE can be configured with a HARQ mode A or B per HARQ process;

-	Maximum number of HARQ processes is extended to 32;

-	The value ranges of MAC (i.e. sr-ProhibitTimer and configuredGrantTimer), RLC (i.e. t-Reassembly) and PDCP (i.e. discardTimer and t-reordering) layer timers are extended.

NOTE:	It is up to network implementation to ensure proper configuration of HARQ feedback (i.e. enabled or disabled) for HARQ processes used by an SPS configuration and of HARQ mode for HARQ processes used by a CG configuration.

If a logical channel is configured with allowedHARQ-mode, it can only be mapped to a HARQ process with the same HARQ mode.

Impact on timing aspects:

To accommodate the long propagation delays, several NR timings involving DL-UL timing interaction are enhanced by the support of two scheduling offsets: Koffset and kmac.

The timing relationships that need to be modified for NTN using Koffset are summarized as follows:

-	The transmission timing of DCI scheduled PUSCH, including channel state information (CSI) transmission on PUSCH;

-	The transmission timing of random access response (RAR) grant or fallbackRAR grant scheduled PUSCH;

-	The timing of the first PUSCH transmission opportunity in type-2 configured grant;

-	The transmission timing of HARQ-ACK on physical uplink control channel (PUCCH), including HARQ-ACK on PUCCH to message B (MsgB) in 2-step random access;

-	The transmission timing of PDCCH ordered physical random access channel (PRACH);

-	The timing of the adjustment of uplink transmission timing upon reception of a corresponding timing advance command;

-	The transmission timing of aperiodic sounding reference signal (SRS);

-	The CSI reference resource timing.

Figure 16.14.2-1 is an illustration of the transmission timing of DCI scheduled PUSCH, the slot allocated for the PUSCH can be modified to be n+K2+Koffset. Note for this example the subcarrier spacing (SCS) value of the downlink is supposed to be the same as that of the uplink.

For initial access, the information of Koffset is carried in system information. Update of Koffset after initial access is supported. The UE-specific Koffset can be provided and updated by the network with MAC CE.

Figure 16.14.2-1: Timing relationship between UL and DL for PUSCH transmission

kmac is a scheduling offset supported in NTN for MAC CE timing relationships enhancement. It is provided by the network if downlink and uplink frame timing are not aligned at gNB. It is needed for UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH. The Kmac is also used in the beam failure recovery, where after a PRACH transmission in uplink slot n the UE monitors the corresponding PDCCH starting from downlink slot "n + kmac + 4" within a corresponding RAR window.

If a UE is provided with a kmac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+kmac, where µ is the SCS configuration for the PUCCH. MAC CE timing relationship enhancement with kmac is illustrated in Figure 16.14.2-2.

Figure 16.14.2-2: MAC CE timing relationship enhancement with Kmac

Timing pre-compensation at the UE:

To accommodate the long propagation delays experienced in NTN on both service link and feeder link, the UE should be able to perform time pre-compensation for all its uplink transmissions; including PRACH preamble transmissions and uplink transmissions during the RRC_CONNECTED state. To do such pre-compensation, the UE is assisted by its GNSS and by the network which periodically broadcasts assistance information including serving satellite ephemeris as well as higher layer Common-TA-related parameters, where the latter may be used to calculate the common RTD e.g. delay on the feeder link.

Release-17 specified the following formula for TA calculation that shall be applied by NTN UEs for PRACH preamble transmission and in RRC_CONNECTED state:

TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUE×Tc

Where:

-	NTA and NTA,offset were already specified in [TS 38.213] [TS 38.211] as part of the existing TA Control;

-	NTA,adjcommon is network-controlled common TA, and may include any timing offset considered necessary by the network (e.g. feeder link delay). It is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured, otherwise NTA,adjcommon=0;

-	NTA,adjUE is UE self-estimated TA to pre-compensate for the service link delay. It is computed by the UE based on UE position and serving satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0;

-	Tc is the NR basic time unit [TS 38.211].

Figure 16.14.2-3: Uplink/Downlink Radio Frame Timing at the UE

Frequency pre-compensation at the UE:

The UE shall be capable of using its acquired GNSS position and serving satellite ephemeris information (when provided by the network) to calculate frequency pre-compensation to counter shift the instantaneous Doppler shift experienced on the service link.

While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE, the management of Doppler shift experienced over the feeder link as well as any transponder frequency error whether it is introduced in Downlink or Uplink is left to the network implementation without any specification impacts in Release 17.

### 16.14.3	Mobility and State transition

#### 16.14.3.1	Mobility in RRC_IDLE and RRC_INACTIVE

The same principles as described in 9.2.1 apply to mobility in RRC_IDLE for NTN and the same principles as described in 9.2.2 apply to mobility in RRC_INACTIVE for NTN unless hereunder specified.

The network may broadcast multiple Tracking Area Codes (TAC) per PLMN in a NR NTN cell. A TAC change in the System Information is under network control, i.e. it may not be exactly synchronised with real-time illumination of beams on ground.

The UE can determine the network type (Terrestrial or non-terrestrial) implicitly by the existence of scheduling information of SIB19 in SIB1.

Editor's note: Non-NTN capable UE can be prevented from accessing an NTN cell in Rel-17. FFS how.

The NTN ephemeris is divided into serving cell's satellite ephemeris and neighbouring cell's satellite ephemeris.

At least in the quasi-earth fixed cell scenario, UE can perform time-based and location-based cell selection /reselection:

-	The timing and location information associated to a cell are provided via system information;

-	Timing information refers to the time when the serving cell is going to stop serving a geographical area;

-	Location information refers to the reference location of serving or neighbouring cells.

Location information may be used to assist cell reselection in NTN with for example a condition based on the distance between UE and the reference location of the serving cell and/or neighbour cells.

UE may support mobility between radio access technologies based on different orbit (GSO, NGSO at different altitude).

#### 16.14.3.2	Mobility in RRC_CONNECTED

##### 16.14.3.2.1	Handover

The same principle as described in 9.2.3.2 applies unless hereunder specified:

During mobility between NTN and Terrestrial Network, a UE is not required to connect to both NTN and Terrestrial Network at the same time.

NOTE:	NTN-Terrestrial Network hand-over refers to mobility in both directions, i.e. from NTN to Terrestrial Network (hand-in) and from Terrestrial Network to NTN (hand-out).

Editor's note: FFS details for NTN-TN hand-over impact on RRC procedure.

DAPS handover is not supported for NTN in this release of the specification.

UE may support mobility between radio access technologies based on different orbit (GSO, NGSO at different altitude).

##### 16.14.3.2.2	Conditional Handover

The same principle as described in 9.2.3.4 applies to NTN.

Editor's note: FFS details for NTN-TN CHO impact on RRC procedure.

NTN supports the following additional triggering conditions upon which UE may execute CHO to a candidate cell, as defined in TS 38.331 [12]:

-	event A4;

-	A time-based trigger condition;

-	A location-based trigger condition.

A time-based or a location-based trigger condition is always configured together with the measurement-based trigger conditions (CHO events A4). Location is defined by the distance between UE and a reference location. Time is defined by the time between T1 and T2, where T1 is an absolute time value and T2 is a duration started at T1.

#### 16.14.3.3	Measurements

The same principle as described in 9.2.4 applies to measurements in NTN unless hereunder specified.

The network can configure:

-	multiple SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities using propagation delay difference, feeder link delay as well as serving/neighbour satellite cell ephemeris;

-	measurement gaps using the same propagation delay difference as computed for SMTC.

The adjustment of SMTCs is possible under network control for connected mode and under UE control based on UE location information and ephemeris for idle/inactive modes.

### 16.14.4	Switch over

#### 16.14.4.1	Definitions

A feeder link switch over is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switch over is a Transport Network Layer procedure.

Both hard and soft feeder link switch over are applicable to NTN.

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload only connect to one NTN Gateway at any given time i.e. a radio link interruption may occur during the transition between the feeder links.

#### 16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported.

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.The mapping between Cell Identities and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location info received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Pre-configuration of special mapped cell identifiers may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

### 16.14.6	AMF (Re-)Selection by gNB

The gNB implements the NAS Node Selection Function specified in TS 38.410 [16].

For a RRC_CONNECTED UE, when the gNB is configured to ensure that the UE connects to an AMF that serves the country in which the UE is located. If the gNB detects that the UE is in a different country to that served by the serving AMF, then it should perform an NG handover to change to an appropriate AMF, or initiate an UE Context Release Request procedure towards the serving AMF (in which case the AMF may decide to de-register the UE).

### 16.14.7	O&M Requirements

The following NTN related parameters shall be provided by O&M to the gNB providing non-terrestrial NR access:

-	Ephemeris information describing the orbital trajectory information or coordinates for the NTN vehicles. This information is provided on a regular basis or upon demand to the gNB;

-	Two different sets of ephemeris format shall be supported:

-	Set 1: Satellite position and velocity state vectors:

-	Position;

-	Velocity.

-	Set 2: At least the following parameters in orbital parameter ephemeris format, as specified in NIMA TR 8350.2 [51]:

-	Semi-major axis;

-	Eccentricity;

-	Argument of periapsis;

-	Longitude of ascending node;

-	Inclination;

-	Mean anomaly at epoch time to.

-	The explicit epoch time associated to ephemeris data;

-	The location of the NTN-Gateways;

NOTE 1:	The ephemeris of the satellites and the location of the NTN-Gateways, are used at least for the Uplink timing and frequency synchronization. It may also be used for the random access and the mobility management purposes.

-	Additional information to enable gNB operation for feeder/service link switch overs.

NOTE 2:	The NTN related parameters provided by O&M to the gNB may depend on the type of supported service links e.g. earth fixed beams, quasi earth fixed beams, earth moving beams, etc.

### 16.14.8	UE location aspects

Upon network request, after AS security in connected mode is established, a UE can report coarse UE location information (the most Significant Bits of its GNSS coordinates corresponding to an accuracy around 2km level) to the NG-RAN without receiving any prior explicit user consent. if "user consent" is available at the UE, the UE will report the coarse UE location information. Else, the UE will respond "no coarse GNSS location available". Periodic location reporting can be configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED.

END