3GPP TSG-RAN WG2 Meeting #118 electronic                                            R2-2204699

Online, May 9th –20th, 2022

Source:	CATT

Title:	Discussion on the positioning MG activation/deactivation MAC CE

Agenda Item:	6.11.2.1

Document for:	Discussion and Decision

1	Introduction

In this document, we will discuss on the remaining FFS on the positioning MG activation/deactivation request MAC CE, as well as the positioning MG activation/deactivation command MAC CE, , and we also give the corresponding draft CRs in [1].

2	Discussion

The positioning MGs are per UE configured via MeasGapConfig within the RRCReconfiguration message, and then based on UE request via positioning MG activation/deactivation request MAC CE, or LMF request, those pre-configured positioning MGs are activated/deactivated via positioning MG activation/deactivation command MAC CE by the serving gNB. However, the uplink positioning MG activation/deactivation request MAC CE, as well as the downlink positioning MG activation/deactivation command MAC CE are still missing in the TS38.321 [2].

#### 6.1.3.40 Positioning Measurement Gap Activation/Deactivation Request MAC CE The Positioning Measurement Gap Activation/deactivation request MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-2b. Editor's NOTE: FFS the format of the MAC CE and the fields within the MAC CE. #### 6.1.3.41 Positioning Measurement Gap Activation/Deactivation Command MAC CE The Positioning Measurement Gap Activation/Deactivation Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. Editor's NOTE: FFS the format of the MAC CE and the fields within the MAC CE.

Observation 1: The format of the positioning MG activation/deactivation request/command MAC CE and the fields within the MAC CE are still FFS.

The positioning MGs are pre-configured at per UE level, and each pre-configured positioning MG is identified with a unique ID. Further, according to the latest RAN1 parameters [3], the maximum number of positioning MGs that can be configured is 16. Thus, the positioning MG activation/deactivation request/command MAC CE should include the ID of the pre-configured positioning MGs which length is 4 bits, as well as the corresponding indication of the activation/deactivation status which length is 1 bit.

Proposal 1: The positioning MG activation/deactivation request/command MAC CE should include the following fields:

- ID of the pre-configured positioning MGs, which length is 4 bits;

- The indication of the activation/deactivation status of the ID, which length is 1 bit.

The corresponding draft CR on the detailed format of the positioning MG activation/deactivation request MAC CE, as well as the positioning MG activation/deactivation command MAC CE can be seen in the [1].

Proposal 2: RAN2 to agree the draft CR [1] on the positioning MG activation/deactivation request MAC CE, as well as the positioning MG activation/deactivation command MAC CE.

3	Conclusion

In the previous sections, we made the following observations and proposals:

Observation 1: The format of the positioning MG activation/deactivation request/command MAC CE and the fields within the MAC CE are still FFS.

Proposal 1: The positioning MG activation/deactivation request/command MAC CE should include the following fields:

- ID of the pre-configured positioning MGs, which length is 4 bits;

- The indication of the activation/deactivation status of the ID, which length is 1 bit.

Proposal 2: RAN2 to agree the TP in the annex on the positioning MG activation/deactivation request MAC CE, as well as the positioning MG activation/deactivation command MAC CE.

4	Reference

[1] R2-2204700, Correction on the positioning MG activation deactivation MAC CE, CATT, CR, Rel-17, F, TS38.321, 17.0.0, 1229  -F  NR_pos_enh-Core.

[2] 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[3] R1-2202759 Consolidated higher layers parameter list for Rel-17 NR.xlsx.