3GPP TSG-RAN WG2 Meeting #118-e                                                    R2-2204765

Electronic, 9th – 20th May, 2022

Source:	CATT

Title:	[C122]Conditions of RemoteUEInformationSidelink Transmission

Agenda Item:	6.7.2.1‎

Document for:	Discussion

Introduction

This Tdoc is to further discuss the issue which was raised in the SL relay-specific RILs C122 of the “38.331 Rel17 ASN1 review Ph1 v76” document.  In this contribution, we will discuss conditions of RemoteUEInformationSidelink message transmission.

Discussion

In [1], the description of actions related to transmission of RemoteUEInformationSidelink message is below:

5.8.9.8.2 Actions related to transmission of RemoteUEInformationSidelink message The L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE shall: 1> if the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1: 2> include sl-Requested-SI-List in the RemoteUEInformationSidelink to indicate the requested SIB(s); 1> set sl-PagingInfo-RemoteUE as follows: 2> if the L2 U2N Remote UE is in RRC_IDLE: 3> include ng-5G-S-TMSI in the sl-PagingIdentity-RemoteUE; 3> set UE specific DRX cycle to the value of UE specific Uu DRX cycle configured by upper layer in the sl-PagingCycle-RemoteUE; 2> else if the L2 U2N Remote UE is in RRC_INACTIVE: 3> include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentity-RemoteUE; 3> set UE specific DRX cycle to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RAN) in the sl-PagingCycle-RemoteUE; 1> submit the RemoteUEInformationSidelink message to lower layers for transmission; When entering RRC_CONNECTED, if L2 U2N remote UE had send sl-Requested-SI-List and sl-PagingInfo-RemoteUE, the L2 U2N Remote UE shall: 1> set the sl-Requested-SI-List to the value release; 1> set the sl-PagingInfo-RemoteUE to the value release; 1> submit the RemoteUEInformationSidelink message to lower layers for transmission;

The L2 U2N Remote UE sends RemoteUEInformationSidelink message for SIB request when it is in RRC_IDLE or RRC_INACTIVE and has not stored a valid version of one or several SIB(s).

However, the L2 U2N Remote UE sends RemoteUEInformationSidelink message for paging forward request only when it is in RRC_IDLE or RRC_INACTIVE. It means the RRC_IDLE/RRC_INACTIVE remote UE will transmit RemoteUEInformationSidelink message to the relay UE to request paging forward again and again. Obviously, this situation should be avoided. Therefore, the conditions of RemoteUEInformationSidelink message transmission should be clarified in 5.8.9.8.2. To avoid repetitive paging forward request, the condition “if the UE has not sent sl-PagingInfo-RemoteUE to the current relay UE” can be added.

Proposal 1: Add “if the UE has not sent sl-PagingInfo-RemoteUE to the current relay UE” as the condition of transmission RemoteUEInformationSidelink message to the relay UE to request paging forward.

Furthermore, for the case that the L2 U2N remote UE shall send RemoteUEInformationSidelink message for paging or SIB forward release, it should be restricted that the remote UE had sent paging or SIB forward request to the current relay UE. The case that the L2 U2N remote UE had sent paging or SIB forward request to the previous relay UE should be excluded.

Proposal 2: Clarify when the L2 U2N remote UE entering RRC_CONNECTED, it shall transmit RemoteUEInformationSidelink message to the relay UE if it had sent paging or SIB forward request to the current relay UE.

In addition, the RemoteUEInformationSidelink message may include sl-Requested-SI-List and/or sl-PagingInfo-RemoteUE to release paging or SIB forward. The cases that the remote UE only request paging forward or SIB forward should be considered.

Proposal 3: When L2 U2N remote UE enters RRC_CONNECTED, it may send release of sl-Requested-SI-List and/or sl-PagingInfo-RemoteUE.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Add “if the UE has not sent sl-PagingInfo-RemoteUE to the current relay UE” as the condition of transmission RemoteUEInformationSidelink message to the relay UE to request paging forward.

Proposal 2: Clarify when the L2 U2N remote UE entering RRC_CONNECTED, it shall transmit RemoteUEInformationSidelink message to the relay UE if it had sent paging or SIB forward request to the current relay UE.

Proposal 3: When L2 U2N remote UE enters RRC_CONNECTED, it may send release of sl-Requested-SI-List and/or sl-PagingInfo-RemoteUE.

TP corresponding to Proposal 1, Proposal 2 and Proposal 3 is in Annex.

Reference

- 3GPP TS 38.331 v17.0.0

Annex

* * * Start of Text Proposal * * *

5.8.9.8.2	Actions related to transmission of RemoteUEInformationSidelink message

The L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE shall:

1>	if the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1:

2>	include sl-Requested-SI-List in the RemoteUEInformationSidelink to indicate the requested SIB(s);

1>	if the UE has not sent sl-PagingInfo-RemoteUE to the current relay UE:

2> set sl-PagingInfo-RemoteUE as follows:

3> if the L2 U2N Remote UE is in RRC_IDLE:

4> include ng-5G-S-TMSI in the sl-PagingIdentity-RemoteUE;

4> set UE specific DRX cycle to the value of UE specific Uu DRX cycle configured by upper layer in the sl-PagingCycle-RemoteUE;

3> else if the L2 U2N Remote UE is in RRC_INACTIVE:

4> include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentity-RemoteUE;

4> set UE specific DRX cycle to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RAN) in the sl-PagingCycle-RemoteUE;

1> submit the RemoteUEInformationSidelink message to lower layers for transmission if RemoteUEInformationSidelink message is needed;

When entering RRC_CONNECTED, if L2 U2N remote UE had sent sl-Requested-SI-List and/or sl-PagingInfo-RemoteUE to the current relay UE, the L2 U2N Remote UE shall:

1>	if L2 U2N remote UE had sent sl-Requested-SI-List to the current relay UE:

2> set the sl-Requested-SI-List to the value release;

1>	if L2 U2N remote UE had sent sl-PagingInfo-RemoteUE to the current relay UE:

2> set the sl-PagingInfo-RemoteUE to the value release;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission if RemoteUEInformationSidelink message is needed;

* * * End of Text Proposal * * *