3GPP TSG-RAN WG2 Meeting #118-e                                                     R2-2204766

Electronic, 9th - 20th May, 2022

Source:	CATT

Title:	Discussion on the LCIDs of SL-SCH for Uu Logical Channels of Remote UE

Agenda Item:	6.7.2.1‎

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the value of the LCIDs of Uu logical channels of remote UE.

Discussion

For L2 U2N relay, both Uu and PC5 logical channels of remote UE are mapped to the SL-SCH at PC5 hop. However, there wasn’t LCID definition for Uu logical channels for SL-SCH.

In TS 38.321, as shown in the following Table 6.2.1-1, the value of LCID of CCCH for DL-SCH and UL-SCH is fixed to 0, and the values of LCID of DCCH and DTCH for DL-SCH and UL-SCH are configurable from 1 to 32. Similar as LCID for DL-SCH and UL-SCH, the value of LCID of CCCH for SL-SCH should be fixed. And the value of LCID of DCCH and DTCH for SL-SCH should be configurable.

Table 6.2.1-1 Values of LCID for DL-SCH (TS38.321)

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel of DCCH, DTCH and multicast MTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35–46 | Reserved |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.4-1 Values of LCID for SL-SCH (TS38.321)

| Index | LCID values |
| --- | --- |
| 0 | SCCH carrying PC5-S messages that are not protected |
| 1 | SCCH carrying PC5-S messages "Direct Security Mode Command" and "Direct Security Mode Complete" |
| 2 | SCCH carrying other PC5-S messages that are protected |
| 3 | SCCH carrying PC5-RRC messages |
| 4–19 | Identity of the logical channel |
| 20–58 | Reserved |
| 59 | Sidelink Inter-UE Coordination Request |
| 60 | Sidelink Inter-UE Coordination Information |
| 61 | Sidelink DRX Command |
| 62 | Sidelink CSI Reporting |
| 63 | Padding |

For defining the LCID value of CCCH, it needs to consider the existing values of LCID for SL-SCH. Since RAN2 had agreed to use a specific LCID for discovery, it may be 58. Hence, the value of LCID of CCCH for SL-SCH can be fixed to 57.

Proposal 1: The value of LCID used for CCCH in SL-SCH can be fixed to 57.

In above Table 6.2.4-1, values of 4-19 are used for identity of the logical channel. There are two interpretions for it:

- Interpretion 1: values of 4-19 are used for STCH;

- Interpretion 2: values of 4-19 are used for any logical channel without fixed value.

It is not needed to use different value range of LCID for the logical channels without fixed value, they can share the same value range. Hence, interpretion 2 is better.

Proposal 2: STCH, DCCH and DTCH share the same LCID value range in SL-SCH.

For interpretion 2, it needs to extend the values of LCID in order to identity of the logical channels. There are two options for extension:

- Option 1: values of 4-35 are used for STCH, DCCH and DTCH(i.e., maximum number of configured logical channel is 32);

- Option 2: values of 4-51 are used for STCH, DCCH and DTCH(i.e., maximum number of configured logical channel is 16+32=48).

Since STCH, DCCH and DTCH share the values, it is unnecessary to define a wide range for it. Therefore, option 1 is prefered.

Proposal 3: The values of LCID of STCH, DCCH and DTCH for SL-SCH can be configured from 4 to 35.

Conclusion

Proposal 1: The value of LCID used for CCCH in SL-SCH can be fixed to 57.

Proposal 2: STCH, DCCH and DTCH share the same LCID value range in SL-SCH.

Proposal 3: The values of LCID of STCH, DCCH and DTCH for SL-SCH can be configured from 4 to 35.

The corresponding TP for proposal 1, proposal 2 and proposal 3 are shown in the Annex:

Reference

- 3GPP TS 38.321 v17.0.0

Annex

* * * Start of Text Proposal * * *

Table 6.2.4-1 Values of LCID for SL-SCH

| Index | LCID values |
| --- | --- |
| 0 | SCCH carrying PC5-S messages that are not protected |
| 1 | SCCH carrying PC5-S messages "Direct Security Mode Command" and "Direct Security Mode Complete" |
| 2 | SCCH carrying other PC5-S messages that are protected |
| 3 | SCCH carrying PC5-RRC messages |
| 4–35 | Identity of the logical channel of STCH, DCCH and DTCH |
| 36–56 | Reserved |
| 57 | CCCH |
| 58 | SCCH carrying discovery message |
| 59 | Sidelink Inter-UE Coordination Request |
| 60 | Sidelink Inter-UE Coordination Information |
| 61 | Sidelink DRX Command |
| 62 | Sidelink CSI Reporting |
| 63 | Padding |

* * * End of Text Proposal * * *