3GPP TSG-RAN WG2 Meeting #118 electronic	R2-2204797

Online, May 09-20, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0003 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the DL bearer mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 38.473, the CU need to configure the DU with DL mapping information via F1 signalling. And it states that the gNB-DU shall use the stored mapping information for the mapping of SRB/DRB data to Uu RLC channel, as specified in TS 38.351. TS 38.473 If the SRB Mapping Info IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the SRB Mapping Info IE for the SRB identified by the SRB ID IE and the Uu RLC channel identified by the SRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of SRB data to Uu RLC channel, as specified in TS 38.351[45]. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu RLC channel, as specified in TS 38.351[45].. However, in the latest TS 38.351, the mapping of SRB/DRB data to Uu RLC channel at gNB-DU is missing. As a reference, the procedure of mapping to BH RLC channel at IAB-donor DU is captured in BAP specification of IAB (i.e. TS 38.340). Similarly, it woud be better to also capture the mapping of SRB/DRB data to Uu RLC channel at gNB-DU in TS 38.351. |
| Summary of change: |  | - Add the reference of 38.473; - Add the SDAP relevant description for gNB-DU and in Figure 4.2.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how to utilize the SRB Mapping Info IE and DRB Mapping Info IE configured in F1 signalling and the DL mapping procedure is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.2, 4.5, 5.1.1, 5.1.2, 5.2.1, 5.2.1.1, 5.2.2, 5.2.3, 5.2.3.1, 5.2.3.2, 5.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NG Radio Access Network; Overall description".

[3]	3GPP TS 38.331: "NR Radio Resource Control (RRC); Protocol Specification".

[4]	3GPP TS 38.322: "NR Radio Link Control (RLC) protocol specification".

[5]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[6]	3GPP TS 38.473: "NG-RAN F1 application protocol (F1AP) protocol specification".

<<Next Change>>

4.2.2	SRAP entities

Figure 4.2.2-1 represents one possible structure for the SRAP sublayer. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

Figure 4.2.2-1: SRAP structure overview

On the U2N Relay UE, the SRAP sublayer contains one SRAP entity at Uu interface and a separate collocated SRAP entity at the PC5 interface. On the U2N Remote UE, the SRAP sublayer contains only one SRAP entity at the PC5 interface. On the gNB-DU, the SRAP sublayer contains only one SRAP entity at the Uu interface.

Each SRAP entity has a transmitting part and a receiving part. Across the PC5 interface, the transmitting part of the SRAP entity at the U2N Remote UE has a corresponding receiving part of an SRAP entity at the U2N Relay UE, and vice-versa. Across the Uu interface, the transmitting part of the SRAP entity at the U2N Relay UE has a corresponding receiving part of an SRAP entity at the gNB-DU, and vice-versa.

<omitted text>

<<Next Change>>

4.5	Configurations

The configuration of the SRAP entity for U2N Remote UE includes:

-	Mapping from a radio bearer identified by BEARER ID to egress PC5 RLC channel via RRC;

-	The local identity via RRC.

The configuration of the SRAP entity for U2N Relay UE includes:

-	The local identity for each U2N Remote UE via RRC;

-	Mapping from UE ID and BEARER ID to egress Uu RLC channel for each U2N Remote UE via RRC;

-	Mapping from UE ID and BEARER ID to egress PC5 RLC channel for each U2N Remote UE via RRC.

The configuration of the SRAP entity for gNB-DU includes:

-	Mapping from a radio bearer to egress Uu RLC channel via F1AP;

5	Procedures

5.1	SRAP entity handling

5.1.1	SRAP entity establishment

When upper layers request establishment of an SRAP entity, UE or gNB-DU shall:

-	establish an SRAP entity;

-	follow the procedures in clause 5.

5.1.2	SRAP entity release

When upper layers request release of an SRAP entity, UE or gNB-DU shall:

-	release the SRAP entity and the related SRAP configurations.

5.2	DL Data transfer

5.2.1	Transmitting operation of gNB-DU

The transmitting part of the SRAP entity on the Uu interface of gNB-DU can receive SRAP Data SDU from upper layer, and construct SRAP Data PDUs.

Upon receiving a SRAP SDU from upper layer, the transmitting part of the SRAP entity on the Uu interface shall:

-		Determine the UE ID and BEARER ID field in accordance with clause 5.2.1.1;

-		Construct an SRAP Data PDU by adding an SRAP header to the SRAP SDU, where the UE ID field and BEARER ID field is set to the determined value, in accordance with clause 6.2.2;

-	Determine the egress Uu RLC channel of the link with U2N Relay UE corresponding to SRB Mapping Info configured for the concerned SRB ID, or corresponding to DRB Mapping Info configured for the concerned DRB ID as specified in TS 38.473 [6];

-	Submit this SRAP Data PDU to the determined egress Uu RLC channel.

5.2.1.1	UE ID and BEARER ID field determination

For a SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID corresponding to Remote UE local ID, configured as specified in TS 38.473 [6];

-	Determine the BEARER ID corresponding to SRB identity for SRB (i.e., set the BEARER ID field to SRB ID), or corresponding to DRB identity minus 1 for DRB (i.e., set the BEARER ID field to DRB ID minus 1), from which the SRAP SDU is received, configured as specified in TS 38.473 [6].

5.2.2	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the Uu interface of U2N Relay UE shall:

-	deliver the SRAP Data Packet to the transmitting part of the collocated SRAP entity on the PC5 interface.

5.2.3	Transmitting operation of U2N Relay UE

The transmitting part of the SRAP entity on the PC5 interface of U2N Relay UE receives SRAP Data Packets from the receiving part of the SRAP entity on the Uu interface of the same U2N Relay UE.

When the transmitting part of the SRAP entity on the PC5 interface has an SRAP Data PDU to transmit, the transmitting part of the SRAP entity on the PC5 interface shall:

-	Determine the egress link in accordance with clause 5.2.3.1;

-	Determine the egress RLC channel in accordance with clause 5.2.3.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel of the determined egress link.

5.2.3.1	Egress link determination

For a SRAP Data PDU to be transmitted, SRAP entity shall:

-	if there is an entry in sl-SRAP-Config-Relay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU:

-	Determine the egress link on PC5 interface corresponding to sl-L2Identity-Remote configured for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

5.2.3.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if the BEARER ID of the SRAP Data PDU is 0:

-	Determine the egress PC5 RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3];

-	else if there is an entry in sl-SRAP-Config-Relay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU, which includes an sl-RemoteUE-RB-Identity that matches the SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (SRB and DRB are differentiated based on sl-Egress-RLC-Channel-Uu):

-	Determine the egress PC5 RLC channel in the determined egress link corresponding to sl-Egress-RLC-Channel-PC5 configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

5.2.4	Receiving operation of U2N Remote UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity shall:

-	remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to upper layer, i.e., PDCP layer (TS 38.323 [5]), entity corresponding to the BEARER ID of this SRAP Data PDU (SRB and DRB are differentiated based on sl-Egress-RLC-Channel-PC5).

<<Next Change>>

5.3	UL Data transfer

5.3.1	Transmitting operation of U2N Remote UE

The transmitting part of the SRAP entity on the PC5 interface of U2N Remote UE can receive SRAP Data SDU from upper layer, and construct SRAP Data PDUs as needed (see clause 4.2.2).

Upon receiving a SRAP SDU from upper layer, the transmitting part of the SRAP entity on the PC5 interface shall:

-	if the SRAP SDU is not for SRB0:

-	Determine the UE ID and BEARER ID field in accordance with clause 5.3.1.1;

-	Construct an SRAP Data PDU by adding an SRAP header to the SRAP SDU, where the UE ID field and BEARER ID field is set to the determined value, in accordance with clause 6.2.2;

-	Determine the egress RLC channel in accordance with clause 5.3.1.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

5.3.1.1	UE ID and BEARER ID field determination

For a SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID corresponding to sl-LocalIdentity, configured as specified in TS 38.331 [3];

-	Determine the BEARER ID corresponding to SRB identity for SRB (i.e., set the BEARER ID field to srb-Identity), or corresponding to DRB identity minus 1 for DRB (i.e., set the BEARER ID field to drb-Identity minus 1), from which the SRAP SDU is received, configured as specified in TS 38.331 [3].

5.3.1.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if the SRAP SDU is for SRB0:

-	Determine the egress PC5 RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3];

-	else if there is an entry in sl-SRAP-Config-Remote, whose sl-RemoteUE-RB-Identity matches the SRB identity or DRB identity of the SRAP Data PDU,:

-	Determine the egress PC5 RLC channel of the link with U2N Relay UE corresponding to sl-Egress-RLC-Channel-PC5 configured for the concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

5.3.2	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data Packet from lower layer, the receiving part of the SRAP entity on the PC5 interface shall:

-	deliver the SRAP Data Packet to the transmitting part of the collocated SRAP entity.

5.3.3	Transmitting operation of U2N Relay UE

The transmitting part of the SRAP entity on the Uu interface of U2N Relay UE can receive SRAP Data Packets from the receiving part of the SRAP entity on the PC5 interface of the same U2N Relay UE, and construct SRAP Data PDUs as needed (see clause 4.2.2).

Upon receiving SRAP Data packet from the collocated SRAP entity, the transmitting part of the SRAP entity on the Uu interface shall:

-	if the SRAP Data packet is received from SL-RLC0 as specified in TS 38.331 [3]:

-	Determine the UE ID and BEARER ID field in accordance with clause 5.3.3.1, for SRAP Data packet;

-	Construct an SRAP Data PDU by adding an SRAP header to the SRAP SDU, where the UE ID field and BEARER ID field is set to the determined value, in accordance with clause 6.2.2, for SRAP Data packet;

-	Determine the egress RLC channel in accordance with clause 5.3.3.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

5.3.3.1	UE ID and BEARER ID field determination

For an SRAP Data SDU received from SL-RLC0 as specified in TS 38.331 [3], the SRAP entity shall:

-	if there is an entry in sl-RemoteUE-ToAddModList, whose sl-L2Identity-Remote matches the Layer-2 ID of the remote UE from which the SRAP Data packet is received:

-	Determine the UE ID corresponding to sl-LocalIdentity configured for the concerned sl-L2Identity-Remote as specified in TS 38.331 [3];

-	Determine the BEARER ID as 0 (i.e., set BEARER ID field as 0), configured as specified in TS 38.331 [3].

5.3.3.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if there is an entry in sl-SRAP-Config-Relay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU, and which includes an sl-RemoteUE-RB-Identity matches SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (SRB and DRB are differentiated based on sl-Egress-RLC-Channel-PC5):

-	Determine the egress Uu RLC channel corresponding to sl-Egress-RLC-Channel-Uu configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

5.3.4	Receiving operation of gNB-DU

Upon receiving an SRAP Data Packet from lower layer, the receiving part of the SRAP entity on the Uu interface shall:

-	remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to upper layer.

<<End of Change>>