3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2204878

# Elbonia, 09 – 20 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3013 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Mobility History Information setting [H098][N094][N095][E121][H071][E122] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SONMDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| - Reason for change: |  | - RAN2 has agreed as Rel-17 MHI enhancement to generate MHI for PSCell in a nested structure, i.e. any VisitedPSCellInfo is appended in VisitedCellInfoList to the corresponding VisitedCellInfo. Thus, the PSCell entries should not be generated always by appending to the VisitedCellInfo. Only the UE capable of pscell-MHI-Report-r17 MHI, generates the VisitedPSCellInfo. VisitedPSCellInfo can contain only timeSpent entry whenever new PSCell or PCell is added. Upon ‘addition’ of a new PSCell, the UE can ‘include’ a field with a timer (but does not know the tme yet), while it can’t set its value. The time will be running till the PSCell change or release. - On the other hand, it can ‘set’ the PSCell id value (visitedPSCellId). ‘The oldest entry’ in the current specification isnt explictly distingushed for PCell and PSCell entries. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarifing the UE capable of pscell-MHI-Report-r17 MHI, generates the VisitedPSCellInfo. - Adding ‘timeSpent’ field setting for PSCell entry: at PSCell change or release(to indicate the actual time) - Adding ‘timeSpent’ field inclusion for PSCell entry at PSCell addition or PCell change (to indicate no PSCell time). - Removing the addition of PSCell entries under the RRC CONNECTED related procedures (as there should be no entry for PCells in RRC IDLE or RRC INACTIVE) - Setting no time for PSCell explicitly clarifies the time is set to ‘0’ - Impact analysis Impacted functionality: Mobility History Information Inter-operability: - If the network is implemented according to the CR and the UE is not the Mobility History Information report is ambiguous - If the UE is implemented according to the CR and the network is not the Mobility History Information report is ambiguous |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Mobility History Information feature is not implemented according to the agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 5.7.9	Mobility history information

#### 5.7.9.1	General

This procedure specifies how the mobility history information is stored by the UE, covering RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED.

#### 5.7.9.2	Initiation

If the UE supports storage of mobility history information, the UE shall:

1>	If the UE supports storage of PSCell mobility history information, upon addition of a PSCell:

2>	include a new visitedPSCellInfoList in VisitedCellInfo in variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following:

3>	include timeSpent in the visitedPSCellInfo;

3> set the field visitedCellId in the visitedPSCellInfo  according to following:

5>	if the global cell identity of the PSCell is available:

6>	include the global cell identity of that cell in the field visitedCellId of the entry;

5>	else:

6>	include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry;

1>	Upon change, or release of a PSCell or upon declaring failure in a PSCell (SCG RLF or SCG HOF) while being connected to the current PCell:

2>	set the field timeSpent of the current entry visitedPSCellInfo as the time spent in the PSCell, while being connected to current PCell;

2>	include a new entry in visitedPSCellInfoList of the variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following:

3>	if the global cell identity of the PSCell is available:

4>	include the global cell identity of that cell in the field visitedCellId of the entry;

3>	else:

4>	include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry;

3>	include new field timeSpent of the entry while being connected to the current PCell;

1>	Upon change of suitable cell, consisting of PCell in RRC_CONNECTED (for NR or E-UTRA cell) or serving cell in RRC_INACTIVE (for NR cell) or in RRC_IDLE (for NR or E-UTRA cell), to another NR or E-UTRA cell, or when entering any cell selection' state from 'camped normally' state in NR or LTE or when entering 'any cell selection' state from a suitable cell in RRC_CONNECTED state in NR or LTE:

2>	include an entry in visitedCellInfoList of the variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following:

3>	if the global cell identity of the previous PCell/serving cell is available:

4>	include the global cell identity of that cell in the field visitedCellId of the entry;

3>	else:

4>	include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry;

3>	set the field timeSpent of the entry as the time spent in the previous PCell/serving cell;

3>	if the UE continues to be connected to the same PSCell during the change of the PCell in RRC_CONNECTED:

4>	set the field timeSpent of the current entry visitedPSCellInfo as the time spent in the PSCell, while being connected to current PCell;

4>	include a new entry in visitedPSCellInfoList of the variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following:

5>	if the global cell identity of the PSCell is available:

6>	include the global cell identity of that cell in the field visitedCellId of the entry;

5>	else:

6>	include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry;

5>	include new  field timeSpent of the entry;

3>	if the UE was not configured with a PSCell at the time of change of PCell in RRC_CONNECTED:

4>	include an entry in visitedPSCellInfoList after removing the oldest entry, if necessary, according to the following;

5>	set the field timeSpent of the current entry to 0:

1>	upon entering 'camped normally' state in NR (in RRC_IDLE or RRC_INACTIVE) or E-UTRA (in RRC_IDLE) while previously in RRC_CONNECTED state NR or LTE while not connected to a PSCell:

1>	upon entering 'camped normally' state in NR (in RRC_IDLE or RRC_INACTIVE) or E-UTRA (in RRC_IDLE) while previously in 'any cell selection' state or 'camped on any cell' state in NR or LTE:

2>	include an entry in variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following:

3>	set the field timeSpent of the entry as the time spent in 'any cell selection' state and/or 'camped on any cell' state in NR or LTE.

End of Changes