3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2204880

Elbonia, 09 – 25 May 2022

Agenda item:	6.13.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarification on SHR for DAPS HO

WID/SID:	NR_ENDC_SONMDT_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

Release 17 has introduced the successful handover report at the request of RAN3 to log information regarding a near failure events experienced by the UE during a successful handover. Some clarification of the current specification text in 38.331 is needed in regard with DAPS HO.

# 2	SHR for DAPS HO

RAN2 has agreed that the SHR will only be generated and stored by the UE if it has been previously configured by the network and if certain triggering conditions have been met. These conditions were agreed in RAN2#113bis as follows:

At least the following triggering conditions are applied for generating an HO Success Report in the case that the HO succeeds:

a.    The UE logs the HO success report if, while doing HO, T310 value exceeds a threshold

b.    The UE logs the HO success report if, while doing HO, T312 value exceeds a threshold

c.    The UE logs the HO success report if, while doing HO, T304 exceeds a threshold

d.    In case of DAPS, if the UE gets an RLF in the source while doing DAPS.

This has been captured in the current specification 38.331 in the section 5.7.10.6	Actions for the successful handover report determination is as follows:

The UE shall:

1>	if the ratio between the value of the elapsed time of the timer T304 and the configured value of the timer T304, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 included in the successHO-Config received before executing the last reconfiguration with sync; or

1>	if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT310 included in the successHO-Config configured by the source PCell before executing the last reconfiguration with sync; or

1>	if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312 included in the successHO-Config configured by the source PCell before executing the last reconfiguration with sync; or

1>	if sourceDAPS-FailureReporting included in the successHO-Config before executing the last reconfiguration with sync is set to true and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

2>	store the successful handover information in VarSuccessHO-Report and determine the content in VarSuccessHO-Report as follows:

[…]

While it is clear that condition d) can only apply in the case of a DAPS HO, it is unclear whether the other conditions a) to c) also apply in this case.

Proposal 1: RAN2 to clarify if and which conditions based on timers T310, T312 and T304 also apply to DAPS HO in addition to the condition based on ‘RLF in the source cell while doing DAPS’.

# 3	Conclusion

This document has made the following proposal:

Proposal 1: RAN2 to clarify if and which conditions based on timers T310, T312 and T304 also apply to DAPS HO in addition to the condition based on ‘RLF in the source cell while doing DAPS’.