3GPP TSG-RAN WG2 Meeting #118-e 	      R2-2205147

E-meeting, 09 – 20 May 2022

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0455 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous eIAB corrections to 38.300 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correcting several omissions where the difference between IAB-MT and regular UE cases is not captured or is currently unclear. Adding explanations to clarify certain scenarios. Correcting several typos. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Proposing to revert to previous wording in 4.7.4.4, since the current text (‘perform RLF recovery AT another parent node’) is unclear and could imply e.g. DC operation. - Proposing to make it clear in 4.7.4.5 that the additional Tdelta is received via the collocated IAB-MT from the parent node, to avoid creating the confusion that the text may refer to the link towards the child node instead (since the spec says that the use of the Tdelta by the receiving node is to ‘control its downlink timing’ which may imply that the MAC-CE signalling is towards the child node of the node in question). - Correction made to make it clear that it is IAB-DU or IAB-donor-DU (and not the gNB, as per current spec text) which determines the desired Timing Advance setting and provides that to the IAB-MT (5.3.5.3). - LCG space extension for IAB-MTs introduced (10.4). - Clarification made that an IAB-MT can only independently request operation in Case-6 mode (for setting the IAB-MT transmission time to the transmission time of the IAB-DU) and not in other timing modes (10.9). Only Case-6 timing mode can be ‘required’ ( = requested by the IAB-MT). We do not need to name it (that is Stage-3 level of detail), but a clarification is needed that only a subset of timing modes is covered by this MAC-CE. - Various other clarifications made (10.9) - Several typos corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification from IAB-MT point of view. Lack of clarity and possibility for misinterpretation in some cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.4.4 Backhaul RLF recovery 4.7.4.5 OTA timing synchronization 5.3.5.3 Uplink timing control 10.4 Measurements to Support Scheduler Operation 10.9 IAB Resource Configuration |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | See Summary of change |
| --- | --- |

FIRST CHANGE

#### 4.7.4.4	Backhaul RLF Recovery

When the IAB-node using SA-mode declares RLF on the backhaul link, it can perform RLF recovery by migrating to another parent node underneath the same or a different IAB-donor-CU. In the latter case, the collocated IAB-DU and the IAB-DU(s) of its descendant node(s) may retain the F1 connectivity with the initial IAB-donor-CU in the same manner as for inter-donor partial migration.

The BH RLF recovery procedure for IAB are captured in TS 38.401 [4]. BH RLF declaration for IAB and aspects of RLF recovery by the IAB-MT are handled in clause 9.2.7 of the present document.

NEXT CHANGE

#### 4.7.4.5	OTA timing synchronization

An IAB-DU is subject to the same downlink timing alignment of a gNB. The IAB-DU may use the received downlink signal from a parent as a reference to control its downlink timing using TA in conjunction with an additional Tdelta parameter received by the collocated IAB-MT from the parent via MAC-CE.

NEXT CHANGE

#### 5.3.5.3	Uplink timing control

The gNB(/IAB-DU or IAB-donor-DU) determines the desired Timing Advance setting and provides that to the UE(/IAB-MT). The UE/IAB-MT uses the provided TA to determine its uplink transmit timing relative to the UE's/IAB-MTs observed downlink receive timing.

An IAB-node may support additional modes for uplink timing:

-	The IAB-MT uses the provided TA plus a provided additional offset to determine its uplink transmission timing, to facilitate parent node’s IAB-MT Rx / IAB-DU Rx multiplexing;

-	The IAB-MT aligns its uplink transmission timing to the IAB-DU downlink transmission timing, to facilitate IAB-MT Tx / IAB-DU Tx multiplexing.

The IAB-node uplink timing mode is indicated by the parent node via MAC-CE.

NEXT CHANGE

## 10.4	Measurements to Support Scheduler Operation

Measurement reports are required to enable the scheduler to operate in both uplink and downlink. These include transport volume and measurements of a UEs radio environment.

Uplink buffer status reports (BSR) are needed to provide support for QoS-aware packet scheduling. In NR, uplink buffer status reports refer to the data that is buffered in for a group of logical channels (LCG) in the UE. Eight LCGs and two formats are used for reporting in uplink, except for IAB-MT where up to 256 LCGs are supported:

-	A short format to report only one BSR (of one LCG);

-	A flexible long format to report several BSRs (up to all eight LCGs/up to all 256 LCGs for an IAB-MT).

Uplink buffer status reports are transmitted using MAC signalling. When a BSR is triggered (e.g. when new data arrives in the transmission buffers of the UE), a Scheduling Request (SR) can be transmitted by the UE (e.g. when no resources are available to transmit the BSR).

For IAB, the Pre-emptive BSR can be configured on the backhaul links. The Pre-emptive BSR is sent based on expected data rather than buffered data, as described in clause 4.7.3.3.

Power headroom reports (PHR) are needed to provide support for power-aware packet scheduling. In NR, three types of reporting are supported: a first one for PUSCH transmission, a second one for PUSCH and PUCCH transmission in an LTE Cell Group in EN-DC (see TS 37.340 [21]) and a third one for SRS transmission on SCells configured with SRS only. In case of CA, when no transmission takes place on an activated SCell, a reference power is used to provide a virtual report. To allow network to detect UL power reduction, the PHR reports may also contain Power Management Maximum Power Reduction (P-MPR, see TS 38.101-2 [35]) information that UE uses to ensure UE compliance with the Maximum Permissible Exposure (MPE) exposure regulation for FR2, which is set for limiting RF exposure on human body. Power headroom reports are transmitted using MAC signalling.

NEXT CHANGE

## 10.9	IAB Resource Configuration

If the IAB-DU and the IAB-MT of an IAB-node are subject to a half-duplex constraint, correct transmission/reception by one cannot be guaranteed during transmission/reception by the other and vice versa, e.g., when collocated and operating in the same frequency. If an IAB-node supports enhanced frequency or spatial multiplexing capabilities, additional multiplexing modes can be supported, i.e. simultaneous operation of IAB-MT Rx / IAB-DU Rx, IAB-MT Tx / IAB-DU Tx, IAB-MT Rx / IAB-DU Tx, IAB-MT Tx / IAB-DU Rx. An IAB-node can report its duplexing constraints between the IAB-MT and the IAB-DU via F1AP. An IAB-node can indicate via F1AP whether or not FDM is required for an enhanced multiplexing operation.

The scheduler on an IAB-DU or IAB-donor-DU complies with the gNB-DU resource configuration received via F1AP, which defines the usage of scheduling resources to account for the aforementioned duplexing constraint.

The resource configuration assigns an attribute of hard, soft or unavailable to each symbol of each DU cell. Transmission/reception can occur for symbols configured as hard, whereas scheduling cannot occur, except for some special cases, for symbols configures as unavailable. For symbols configured as soft, scheduling can occur conditionally on an explicit indication of availability by the parent node via DCI format 2_5, or on an implicit determination of availability by the IAB-node. The implicit determination of availability is determined by the IAB-node depending on whether or not the operation of the IAB-DU would have an impact on the collocated IAB-MT.

The resource configuration can be shared among neighbouring IAB-nodes and IAB-donors to facilitate interference management, dual connectivity, and enhanced multiplexing.

To facilitate transitioning from IAB-MT to IAB-DU operation and vice versa, guard symbols can be used at the beginning and/or the end of slots where the IAB-node switches operation from its IAB-DU to its IAB-MT function and operation from its IAB-MT to its IAB-DU function, to overcome potentially misaligned symbol boundaries between the IAB-MT domain and the IAB-DU domain (e.g., IAB-MT Rx boundaries are not aligned with the IAB-DU Tx boundaries). Specifically, an IAB-node can indicate to a parent node a number of desired guard symbols, while the parent node can indicate to the IAB-node the number of actually provided guard symbols for specific transitions (MT Rx to DU Tx, MT Rx to DU Rx, MT Tx to DU Tx, MT Tx to DU Rx, DU Rx to MT Tx, DU Rx to MT Rx, DU Tx to MT Tx, and DU Tx to MT Rx).

An IAB-node supporting enhanced multiplexing capabilities, i.e., IAB-MT Rx / IAB-DU Rx, IAB-MT Tx / IAB-DU Tx, IAB-MT Rx / IAB-DU Tx, IAB-MT Tx / IAB-DU Rx, can provide via MAC-CE to a parent node information to facilitate scheduling for enhanced multiplexing operation by the IAB-node, specifically:

-	recommended IAB-MT’s Tx/Rx beams;

-	desired IAB-MT Tx PSD range;

-	desired parent node’s IAB-DU Tx power adjustment;

-	required IAB-MT’s uplink transmission timing mode (for setting the IAB-MT transmission time to the transmission time of the IAB-DU).

Correspondingly, the parent node can provide via MAC-CE information to the IAB-node to facilitate enhanced multiplexing at the IAB-node and/or at the parent node:

-	restricted IAB-DU Tx beams;

-	actual parent node’s IAB-DU Tx power adjustment;

-	IAB-MT’s uplink transmission timing mode.