3GPP TSG-RAN WG2 Meeting #118-e	R2-2205431

# Electronic Meeting, 9th – 20th May, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0005 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the handling of unknown, unforeseen, and erroneous protocol data and other miscellaneous in SRAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-5-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The SRAP sublayer determines the UE ID and BEARER ID for packets received from collocated SRAP entity or from upper layer. However, in clause 4.4 of current spec, the SRAP sublayer function determination of UE ID and BEARER ID is only for packets received from SRAP entity. - The SRB/DRB is determined by the CHOICE of sl-RemoteUE-RB-Identity, which is the one associated with the received sl-Egress-RLC-Channel-PC5. However, it is not clear for SRB/DRB differentiate in the current spec. - For UL data transfer, U2N remote UE does not need to determine egress link, since there is only one PC5 link between relay UE and Remote UE for relaying. It should be clarified the egress RLC is in the link with Relay UE. - When the SRAP entity of relay UE receives SRAP Data SDU from SL-RLC0, it will determine the bearer ID as 0 as specified in TS 38.331, and no configuration is needed. - In handling of unknow, unforeseen and errorneous data in sction 5.4: - When the SRAP entity of relay/remote UE receives a SRAP Control PDU, the data shall be discarded since no SRAP control PDU is introduced in R17. This is mainly for the scenario of the future deploment that R17 UEs work together with the new UEs supporting control PDU. - When the SRAP entity of Relay UE receives SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2Identity-Remote of the ingress link, it shall be discarded since the local ID in the header is not for this Remote UE. This is to address the case one local ID is received from the incorrect remote UE. - Considering the direct-to-indirect path switch case, if the target relay UE is in IDLE/INACTIVE, the HO complete message on SRB1 from remote UE will be discarded per the current specification, since there is no sl-SRAP-Config-Relay configured yet. Therefore, current error handling shall not be applied to SRB1 messgae. - Some editorials exists. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify in clause 4.4 that the SRAP sublayer function is the determination of UE ID and BEARER ID. - Clarify that SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the sl-Egress-RLC-Channel-Uu. - In clause 5.3.1.2, modify that SRAP entity shall determine egress PC5 Relay RLC channel in the link with U2N Relay UE, for SRB0 case. - In 5.3.3.1, delete “, configured as specified in TS 38.331 [3]” - In clause 5.4, clarify that the current procedure for error handling is not applied for SRB1 message, and the Relay UE do not need to discard SRAP Data packet if the data is for SRB1 message. When the SRAP Control PDU or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2Identity-Remote of the ingress link is received by U2N Relay UE, the data should be discarded. - Fix the editorials. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - SRB/DRB differentiation is not clear. - In clause 5.3.1.2, the action of Remote UE’s SRAP is not correct since no egress link determination is specified for Remote UE. - No SRAP control PDU handling. No handling for the case the SRAP entity of Relay UE receives SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2Identity-Remote of the ingress link. Undisred packet discard can happen in direct-to-indirect path switching procedure for SRB1 message. - Editorials still exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 4.4, 4.5, 5.2, 5.3, 5.4, 6.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

# 4	General

## 4.1	Introduction

The objective is to describe the SRAP architecture and the SRAP entities from a functional point of view.

## 4.2	SRAP architecture

### 4.2.1	General

This clause describes a model of the SRAP, i.e., it does not specify or restrict implementations.

### 4.2.2	SRAP entities

Figure 4.2.2-1 represents one possible structure for the SRAP sublayer. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

Figure 4.2.2-1: SRAP structure overview

On the U2N Relay UE, the SRAP sublayer contains one SRAP entity at Uu interface and a separate collocated SRAP entity at the PC5 interface. On the U2N Remote UE, the SRAP sublayer contains only one SRAP entity at the PC5 interface.

Each SRAP entity has a transmitting part and a receiving part. Across the PC5 interface, the transmitting part of the SRAP entity at the U2N Remote UE has a corresponding receiving part of an SRAP entity at the U2N Relay UE, and vice-versa. Across the Uu interface, the transmitting part of the SRAP entity at the U2N Relay UE has a corresponding receiving part of an SRAP entity at the gNB, and vice-versa.

Figure 4.2.2-2 and Figure 4.2.2-3 represents the functional view of the SRAP entity for the SRAP sublayer at PC5 interface and at Uu interface respectively.

Figure 4.2.2-2: Example of functional view of SRAP sublayer at PC5 interface

Figure 4.2.2-3: Example of functional view of SRAP sublayer at Uu interface

In the example of Figure 4.2.2-2 and Figure 4.2.2-3, at relay UE:

-	The receiving part on the SRAP entity of Uu interface delivers SRAP PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the receiving part on the SRAP entity of PC5 interface delivers SRAP PDUs to the transmitting part on the collocated SRAP entity of Uu interface, except for data packet received from SL-RLC0 as specified in TS 38.331 [3]. As an alternative mode, the receiving part may deliver SRAP SDUs to the collocated transmitting part. When passing SRAP SDUs, the receiving part removes the SRAP header and the transmitting part adds the SRAP header with the same SRAP header content as carried on the SRAP PDU header prior to removal. Passing SRAP SDUs in this manner is therefore functionally equivalent to passing SRAP PDUs, in implementation. The following specification therefore refers to the passing of SRAP Data Packets in supporting the alternative mode.

-	For data packet received from SL-RLC0 as specified in TS 38.331 [3], the receiving part on the SRAP entity of PC5 interface delivers SRAP SDUs to the collocated transmitting part on the collocated SRAP entity of Uu interface, and the transmitting part adds the SRAP header in accordance with clause 5.3.3.

## 4.3	Services

### 4.3.1	Services provided to upper layers

The following services are provided by the SRAP sublayer to upper layers:

-	Data transfer.

### 4.3.2	Services expected from lower layers

An SRAP sublayer expects the following services from lower layers per RLC entity (for a detailed description see TS 38.322 [4]):

-	Acknowledged data transfer service;

-	Unacknowledged data transfer service.

## 4.4	Functions

The SRAP sublayer supports the following functions:

-	Data transfer;

-	Determination of UE ID and BEARER ID for packets;

-	Determination of egress link;

-	Determination of egress RLC channel.

## 4.5	Configurations

The configuration of the SRAP entity for U2N Remote UE includes:

-	Mapping from a radio bearer identified by BEARER ID to egress PC5 Relay RLC channel via RRC;

-	The local identity via RRC.

The configuration of the SRAP entity for U2N Relay UE includes:

-	The local identity for each U2N Remote UE via RRC;

-	Mapping from UE ID and BEARER ID to egress Uu Relay RLC channel for each U2N Remote UE via RRC;

-	Mapping from UE ID and BEARER ID to egress PC5 Relay RLC channel for each U2N Remote UE via RRC.

# 5	Procedures

## 5.1	SRAP entity handling

### 5.1.1	SRAP entity establishment

When upper layers request establishment of an SRAP entity, UE shall:

-	establish an SRAP entity;

-	follow the procedures in clause 5.

### 5.1.2	SRAP entity release

When upper layers request release of an SRAP entity, UE shall:

-	release the SRAP entity and the related SRAP configurations.

## 5.2	DL Data transfer

### 5.2.1	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the Uu interface of U2N Relay UE shall:

-	deliver the SRAP Data Packet to the transmitting part of the collocated SRAP entity on the PC5 interface.

### 5.2.2	Transmitting operation of U2N Relay UE

The transmitting part of the SRAP entity on the PC5 interface of U2N Relay UE receives SRAP Data Packets from the receiving part of the SRAP entity on the Uu interface of the same U2N Relay UE.

When the transmitting part of the SRAP entity on the PC5 interface has an SRAP Data PDU to transmit, the transmitting part of the SRAP entity on the PC5 interface shall:

-	Determine the egress link in accordance with clause 5.2.2.1;

-	Determine the egress RLC channel in accordance with clause 5.2.2.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel of the determined egress link.

#### 5.2.2.1	Egress link determination

For a SRAP Data PDU to be transmitted, SRAP entity shall:

-	if there is an entry in sl-SRAP-Config-Relay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU:

-	Determine the egress link on PC5 interface corresponding to sl-L2Identity-Remote configured for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

#### 5.2.2.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if the BEARER ID of the SRAP Data PDU is 0:

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3];

-	else if there is an entry in sl-SRAP-Config-Relay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU, which includes an sl-RemoteUE-RB-Identity that matches the SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the sl-Egress-RLC-Channel-Uu):

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to sl-Egress-RLC-Channel-PC5 configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

### 5.2.3	Receiving operation of U2N Remote UE

Upon receiving an SRAP Data PDU/packet from lower layer, the receiving part of the SRAP entity shall:

-	remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to upper layer entity corresponding to the BEARER ID of this SRAP Data PDU (SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the sl-Egress-RLC-Channel-PC5).

## 5.3	UL Data transfer

### 5.3.1	Transmitting operation of U2N Remote UE

The transmitting part of the SRAP entity on the PC5 interface of U2N Remote UE can receive SRAP Data SDU from upper layer, and construct SRAP Data PDUs as needed (see clause 4.2.2).

Upon receiving a SRAP SDU from upper layer, the transmitting part of the SRAP entity on the PC5 interface shall:

-	if the SRAP SDU is not for SRB0:

-	Determine the UE ID and BEARER ID field in accordance with clause 5.3.1.1;

-	Construct an SRAP Data PDU by adding an SRAP header to the SRAP SDU, where the UE ID field and BEARER ID field is set to the determined value, in accordance with clause 6.2.2;

-	Determine the egress RLC channel in accordance with clause 5.3.1.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

#### 5.3.1.1	UE ID and BEARER ID field determination

For a SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID corresponding to sl-LocalIdentity, configured as specified in TS 38.331 [3];

-	Determine the BEARER ID corresponding to SRB identity for SRB (i.e., set the BEARER ID field to srb-Identity), or corresponding to DRB identity minus 1 for DRB (i.e., set the BEARER ID field to drb-Identity minus 1), from which the SRAP SDU is received, configured as specified in TS 38.331 [3].

#### 5.3.1.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if the SRAP SDU is for SRB0:

-	Determine the egress PC5 Relay RLC channel in the link with U2N Relay UE  corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3];

-	else if there is an entry in sl-SRAP-Config-Remote, whose sl-RemoteUE-RB-Identity matches the SRB identity or DRB identity of the SRAP Data PDU,:

-	Determine the egress PC5 Relay RLC channel of the link with U2N Relay UE corresponding to sl-Egress-RLC-Channel-PC5 configured for the concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

### 5.3.2	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data Packet from lower layer, the receiving part of the SRAP entity on the PC5 interface shall:

-	deliver the SRAP Data Packet to the transmitting part of the collocated SRAP entity.

### 5.3.3	Transmitting operation of U2N Relay UE

The transmitting part of the SRAP entity on the Uu interface of U2N Relay UE can receive SRAP Data Packets from the receiving part of the SRAP entity on the PC5 interface of the same U2N Relay UE, and construct SRAP Data PDUs as needed (see clause 4.2.2).

Upon receiving SRAP Data packet from the collocated SRAP entity, the transmitting part of the SRAP entity on the Uu interface shall:

-	if the SRAP Data packet is received from SL-RLC0 as specified in TS 38.331 [3]:

-	Determine the UE ID and BEARER ID field in accordance with clause 5.3.3.1, for SRAP Data packet;

-	Construct an SRAP Data PDU by adding an SRAP header to the SRAP SDU, where the UE ID field and BEARER ID field is set to the determined value, in accordance with clause 6.2.2, for SRAP Data packet;

-	Determine the egress RLC channel in accordance with clause 5.3.3.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

#### 5.3.3.1	UE ID and BEARER ID field determination

For an SRAP Data SDU received from SL-RLC0 as specified in TS 38.331 [3], the SRAP entity shall:

-	if there is an entry in sl-RemoteUE-ToAddModList, whose sl-L2Identity-Remote matches the Layer-2 ID of the remote UE from which the SRAP Data packet is received:

-	Determine the UE ID corresponding to sl-LocalIdentity configured for the concerned sl-L2Identity-Remote as specified in TS 38.331 [3];

-	Determine the BEARER ID as 0 (i.e., set BEARER ID field as 0).

#### 5.3.3.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if there is an entry in sl-SRAP-Config-Relay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU, and which includes an sl-RemoteUE-RB-Identity matches SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the sl-Egress-RLC-Channel-PC5):

-	Determine the egress Uu Relay RLC channel corresponding to sl-Egress-RLC-Channel-Uu configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

## 5.4	Handling of unknown, unforeseen, and erroneous protocol data

When a SRAP Data PDU that contains a UE ID or BEARER ID which is not included in sl-SRAP-Config-Remote (for Remote UE) or sl-SRAP-Config-Relay (for Relay UE) is received not from SL-RLC1 as specified in TS 38.331 [3], or when a SRAP Control PDU is received, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2Identity-Remote of the ingress link is received by U2N Relay UE, the SRAP entity shall:

-	discard the received SRAP Data PDU.

# 6	Protocol data units, formats, and parameters

## 6.1	Protocol data units

### 6.1.1	Data PDU

The SRAP Data PDU is used to convey one of the following in addition to the PDU header:

-	upper layer data.

## 6.2	Formats

### 6.2.1	General

An SRAP PDU is a bit string that is byte aligned (i.e. multiple of 8 bits) in length. The formats of SRAP PDUs are described in clause 6.2.2 and their parameters are described in clause 6.3.

### 6.2.2	Data PDU

Figure 6.2.2-1 shows the format of the SRAP Data PDU.

Figure 6.2.2-1: SRAP Data PDU format

## 6.3	Parameters

### 6.3.1	General

If not otherwise mentioned in the definition of each field then the bits in the parameters shall be interpreted as follows: the left most bit string is the first and most significant and the right most bit is the last and least significant bit.

Unless otherwise mentioned, integers are encoded in standard binary encoding for unsigned integers. In all cases the bits appear ordered from MSB to LSB when read in the PDU.

### 6.3.2	UE ID

Length: 8 bits.

This field carries local identity of U2N Remote UE.

### 6.3.3	BEARER ID

Length: 5 bits.

This field carries Uu radio bearer identity for U2N Remote UE.

### 6.3.4	Data

Length: Variable

This field carries the SRAP SDU (i.e. PDCP PDU).

### 6.3.5	R

Length: 1 bit

Reserved. In this release, reserved bits shall be set to 0. Reserved bits shall be ignored by the receiver.

### 6.3.6	D/C

Length: 1 bit

This field indicates whether the corresponding SRAP PDU is an SRAP Data PDU or an SRAP Control PDU (not used in this release).

Table 6.3.6-1: D/C field

| Bit | Description |
| --- | --- |
| 0 | SRAP Data PDU |
| 1 | SRAP Control PDU (not used in this release) |

END OF CHANGE