3GPP TSG-RAN WG2 #118	R2-2205443

Electronical meeting, 9 May – 20 May 2022

Agenda Item:	6.14.3 Corrections

Source:	Ericsson

Title:	Discussion on RIL issues H054 and H094

Document for:	Discussion, Decision

# 1	Introduction

The work item for QoE measurements in NR is declared completed. However, some RIL issues have been added to the RRC CR during ASN.1 review. This contribution addresses RIL issues discussed in the RAN2#118 pre-meeting discussion of the RRC CR, H054 and H094.

# 2	Discussion

## 2.1	RIL issue H054

The following RIL issue was added during the ASN.1 review:

[RIL]: H054 [Delegate]: Huawei (Jun) [WI]: QOE [Class]: 1 [Status]: ToDo [TDoc]: None [Proposed Conclusion]: v31

[Description]: The discard of the oversized measurement report is missing.

[Proposed Change]:

2>	 If the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5]:

3> if the RRC message segmentation is enabled based on the field rrc-SegAllowed received in appLayerMeasConfig:

4> initiate the UL message segment transfer procedure as specified in clause 5.7.7;

3> else:

4> discard the RRC message.

2> else:

3> submit the MeasurementReportAppLayer   message to lower layers for transmission upon which the procedure ends.

[Comments]: [Ericsson]: Corrected in WI CR.

The issue is related to that there is no procedure text related to the handling of messages larger than the maximum size of 16 segments of each 9000 bytes, i.e. totally 144 000 bytes. The issue was first corrected, but later it was commented by one company that the change is not needed, whereas another company supported to clarify the case. In the legacy text, there is no procedure text for the case of oversized messages, so it seems reasonable to not introduce it for the MeasurementReportAppLayer message either. In addition, all different failure cases are normally not described in RRC. Therefore, it is proposed that the change is not added in the correction CR for QoE.

The handling of MeasurementReportAppLayer message larger than 144 000 bytes is not described in the RRC specification.

## 2.2	RIL issue H094

Another question brought up during the pre-meeting discussion is why the IE type for pauseReporting, transmissionOfSessionStartStop and reportPlayOutDelayForMediaStartup have been changed from BOOLEAN to ENUMERATED {true, false}. This is related to RIL issue H094:

[RIL]: H094 [Delegate]: Huawei (Dawid) [WI]: QOE [Class]: 2 [Status]: ToDo [TDoc]: None [Proposed Conclusion]: v127

[Description]: ]: Since this parameter is mandatory, it has to be sent whenever the QoE configuration is modified. This results in the UE forwarding it to app layer, even though the value has not changed.

[Proposed Change]: Make this parameter optional with NEED M.

[Comments]: [Ericsson]: Corrected in WI CR, and also the parameters pauseReporting and reportInitialPlayoutDelay.

The types were changed from BOOLEAN to ENUMERATED in order to have the IEs OPTIONAL. If the parameters are always signalled they will always be forwarded to the application layer which sems to be unnecessary. RAN2 is proposed to discuss and agree on this issue.

The parameters for pauseReporting, transmissionOfSessionStartStop and reportPlayOutDelayForMediaStartup are defined as ENUMERATED {true, false} OPTIONAL.

# Conclusion

RAN2 is kindly asked to discuss the following proposals:

Proposal 1	The handling of MeasurementReportAppLayer message larger than 144 000 bytes is not described in the RRC specification.

Proposal 2	The parameters for pauseReporting, transmissionOfSessionStartStop and reportPlayOutDelayForMediaStartup are defined as ENUMERATED {true, false} OPTIONAL.

# References

TR 38.890, Study on NR QoE management and optimizations for diverse services, Ericsson

RP-210913, NR QoE management and optimizations for diverse services, China Unicom