3GPP TSG-RAN WG2 #118-e	R2-2205488

E-meeting, 9 May – 20 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | draftCR | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Positioning Integrity parameter table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Rel-17 Positioning WI, positioning integrity was introduced and its corresponding parameters are listed, but some are not aligned with the actual contents and corresponding reference equations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For orbit row and integrity bounds (Mean) column, add “mean orbit error and mean orbit rate error” since these parameters are explicitly configured in corresponding LPP field as of mean value. So no need to use the equation. Remove “calculated according to equation 8.1.1a-3” - For orbit row and integrity bound (StdDev) column, add “variance orbit error, and variance orbit rate error” due to the same reason as above case. But variance not standard deviation is used for Orbit error / error rate in the corresponding LPP field. Therefore, adding “(using this values for deriving StdDev)” - In Residual Risk column, these information is only signalled for SSR Orbit error field in current LPP. All the other error sources such as clock, code bias, phase bias would not use this residual risk. For this case, table needs to be updated to isolate this only into the Orbit row. - For clock row and integrity bounds (Mean) column, Mean Clock Residual Error Vector isn’t in the corresponding LPP field, but two other parameters, i.e., Mean Clock Error and Mean Clock Rate Error. So correct this. - For clock row and integrity bounds (StdDev), there is missing parameter Standard Deviation Clock Rate Error which is signalled in the corresponding LPP field. Impact analysis Impacted functionality: Positioning integrity signalling and calculation Inter-operability: If only the network is implemented according to the CR, interoperability problems are not foreseen. If only the UE is implemented according to the CR, interoperability problems are not foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong interpretation happens for signalling and calculating positioning integrity results |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 8.1.2.1b	Mapping of integrity parameters

Table 8.1.2.1b-1 shows the mapping between the integrity fields and the SSR assistance data according to the Integrity Principle of Operation (Clause 8.1.1a). The corresponding field descriptions for each of the field names listed in Table 8.1.2.1b-1 are specified under Clause 6.5.2.2 of TS 37.355 (LPP).

Table 8.1.2.1b-1: Mapping of Integrity Parameters

| Error | GNSS Assistance Data | Integrity Fields |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Integrity Alerts |  | Integrity Bounds (Mean) |  | Integrity Bounds (StdDev) |  | Residual Risks |  | Integrity Correlation Times |  |
| Orbit | SSR Orbit Corrections | Real-Time Integrity (see Section 8.1.2.1.8) | Mean Orbit Error Mean Orbit Rate Error | Variance Orbit Error Variance Orbit Rate Error (using this values for deriving StdDev) | Probability of Onset of Constellation Fault Probability of Onset of Satellite Fault Mean Constellation Fault Duration Mean Satellite Fault Duration | Orbit Range Error Correlation Time Orbit Range Rate Error Correlation Time |
| Clock |  | SSR Clock Corrections |  |  |  | Mean Clock Error Mean Clock Rate Error |  | Standard Deviation Clock Error Standard Deviation Clock Rate Error |  |  |  | Clock Range Error Correlation Time Clock Range Rate Error Correlation Time |  |
| Code Bias |  | SSR Code Bias |  |  |  | Mean Code Bias Error Mean Code Bias Rate Error |  | Standard Deviation Code Bias Error Standard Deviation Code Bias Rate Error |  |  |  |  |  |
| Phase Bias |  | SSR Phase Bias |  |  |  | Mean Phase Bias Error Mean Phase Bias Rate Error |  | Standard Deviation Phase Bias Error Standard Deviation Phase Bias Rate Error |  |  |  |  |  |
| Ionosphere |  | SSR STEC Correction |  | Ionosphere DNU |  | Mean Ionospherre Error Mean Ionospherre Rate Error |  | Standard Deviation Ionosphere Error Standard Deviation Ionosphere Rate Error |  | Probability of Onset of Ionosphere Fault Mean Ionosphere Fault Duration |  | Ionosphere Range Error Correlation Time Ionosphere Range Rate Error Correlation Time |  |
| Troposphere Vertical Hydro Static Delay | SSR Gridded Corrections | Troposphere DNU | Mean Troposphere Vertical Hydro Static Delay Error Mean Troposphere Vertical Hydro Static Delay Rate Error | Standard Deviation Troposphere Vertical Hydro Static Delay Error Standard Deviation Troposphere Vertical Hydro Static Delay Rate Error | Probability of Onset of Troposphere Fault Mean Troposphere Fault Duration | Troposphere Range Error Correlation Time Troposphere Range Rate Error Correlation Time |
| TroposphereVertical WetDelay |  |  | Mean Troposphere Vertical Wet Delay Error Mean Troposphere Vertical Wet Delay Rate Error | Standard Deviation Troposphere Vertical Wet Delay Error Standard Deviation Troposphere Vertical Wet Delay Rate Error |  |  |

END OF CHANGE