3GPP TSG-RAN WG2 #118-e	R2-2205521

E-meeting, 9 May – 20 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | draftCR | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclusion of IABOtherInformation message in RRC Transfer procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In IAB WI, there is an IABOtherInformation message transfer procedure for handling IP address allocation request and report ones already allocated in RRC layer. This should be captured in RRC Transfer procedure when MRDC is configured for IAB node and IABOtherInformation message is transferred to the donor node with encapsulated in ULTransferMRDC message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding IABOtherInformation message in RRC Transfer procedure. Impact analysis Impacted functionality: IAB Other Information procedure in MRDC configured IAB node Inter-operability: If only the network is implemented according to the CR, interoperability problems are not foreseen. If only the UE is implemented according to the CR, interoperability problems are not foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IP address allocation request/ report for IAB DU cannot be delivered to the target donor node for IAB operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 10.10.2	MR-DC with 5GC

The RRC Transfer procedure is used to deliver an RRC message, encapsulated in a PDCP PDU between the MN and the SN (and vice versa) so that it may be forwarded to/from the UE using split SRB. The RRC transfer procedure is also used for:

-	providing a SN measurement report, failure information report, SN UE assistance information, CPC execution completion, or IABOtherInformation from the UE to the SN;

-	providing MCG failure information from the UE to the MN via the SN and an RRC reconfiguration, or release, or an inter-RAT handover command from the MN to the UE via the SN.

Additional details of the RRC transfer procedure are defined in TS 38.423 [5].

Split SRB:

Figure 10.10.2-1: RRC Transfer procedure for split SRB (DL operation)

Figure 10.10.2-1 shows an example signaling flow for DL RRC Transfer in case of the split SRB:

1.	The MN, when it decides to use the split SRBs, starts the procedure by initiating the RRC Transfer procedure. The MN encapsulates the RRC message in a PDCP PDU and ciphers with own keys.

NOTE:	The usage of the split SRBs shall be indicated in the Secondary Node Addition procedure or Modification procedure.

2.	The SN forwards the RRC message to the UE.

3.	The SN may send PDCP delivery acknowledgement of the RRC message forwarded in step 2.

Figure 10.10.2-2: RRC Transfer procedure for split SRB (UL operation)

Figure 10.10.2-2 shows an example signaling flow for UL RRC Transfer in case of the split SRB:

1.	When the UE provides response to the RRC message, it sends it to the SN.

2.	The SN initiates the RRC Transfer procedure, in which it transfers the received PDCP PDU with encapsulated RRC message.

SN measurement report, failure information report, SN UE assistance information, CPC execution completion, or IABOtherInformation:

Figure 10.10.2-3: RRC Transfer procedure for SN measurement report, failure information report, SN UE assistance information, CPC execution completion, or IABOtherInformation

Figure 10.10.2-3 shows an example signaling flow for RRC Transfer in case of the forwarding of the SN measurement report, failure information report, SN UE assistance information, CPC execution completion, or IABOtherInformation from the UE:

1.	When the UE sends an SN measurement report, failure information report, SN UE assistance information, CPC execution completion, or IABOtherInformation it sends it to the MN in a container called ULInformationTransferMRDC as specified in TS 38.331 [4].

2.	The MN initiates the RRC Transfer procedure, in which it transfers the received SN measurement report, failure information, SN UE assistance information, CPC execution completion, or IABOtherInformation as an octet string.

MCG failure information and RRC Reconfiguration / RRC Release / inter-RAT handover command over SRB3:

Figure 10.10.2-4: RRC Transfer procedure for MCG failure information

Figure 10.10.2-4 shows an example signaling flow for RRC Transfer in case of the forwarding of the MCG failure information from the UE:

1.	When the UE sends MCGFailureInformation over SRB3, it sends it to the SN in a container called ULInformationTransferMRDC as specified in TS 38.331 [4].

2.	The SN initiates the RRC Transfer procedure, in which it transfers the received MCGFailureInformation as an octet string.

3.	The MN initiates the RRC Transfer procedure, in which it transfers the RRCConnectionReconfiguration, or RRCReconfiguration, or RRCConnectionRelease, or RRCRelease, or MobilityFromNRCommand, or MobilityFromEUTRACommand as an octet string.

4.	The SN sends the received RRC message to the UE in a container called DLInformationTransferMRDC, as specified in TS 38.331 [4].

END OF CHANGE