3GPP TSG-RAN WG2 Meeting #118-e	R2-2205523

Online, May 9 – 20, 2022

Agenda Item:	6.12.2.2.2

Souce:	MediaTek Inc.

Title:	SIB validity with eDRX

Document for:	Discussion and decision

# 1 Introduction

Extended DRX has been introduced as part of the Rel-17 RedCap WI. There have been several agreements regarding the validity of system information. However, the impact of SIB validity time is still outstanding in RAN2.

In this contribution, we discuss the impact of eDRX on the SIB validity time and propose a way forward.

# 2 Discussion

Currently in NR, the UE deletes any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid, according to the clause below from TS 38.331 [1]:

##### 5.2.2.2.1 SIB validity The UE shall: 1> delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

In LTE, for NB-IoT UEs, the validity time has been extended to 24 hours. For eMTC UEs (BL UEs or UEs in CE), a new optional field si-ValidityTime is introduced in SIB1. If the parameter is set to TRUE, the validity time of 3h applies, otherwise 24h applies [2].

#### 5.2.1.3 System information validity and notification of changes Additionally, for other than BL UEs or UEs in CE or NB-IoT UEs, the UE considers stored system information to be invalid after 3 hours from the moment it was successfully confirmed as valid, unless specified otherwise. BL UE or UE in CE considers stored system information to be invalid after 24 hours from the moment it was successfully confirmed as valid, unless the UE is configured by parameter si-ValidityTime to consider stored system information to be invalid 3 hours after validity confirmation. NB-IoT UE considers stored system information to be invalid after 24 hours from the moment it was successfully confirmed as valid.

si-ValidityTime-r13 ENUMERATED {true} OPTIONAL, -- Need OP si-ValidityTime Indicates system information validity timer. If set to TRUE, the timer is set to 3h, otherwise the timer is set to 24h.

In NR, for the UEs that are configured with eDRX, a similar mechanism is needed. Otherwise, if the UE is configured with a large eDRX cycle (e.g., 10485.76 s or 2.91 hours), the UE will have to re-acquire SI at every eDRX cycle, which will negatively affect UE power savings.

Observation 1: If the legacy validity time of 3h is used for UEs configured with large eDRX cycles, there will be a negative impact on power savings, which will reduce the benefit of eDRX.

We therefore propose that a similar approach as eMTC is introduced to NR. By default, a UE configured with eDRX can consider SIB to be invalid 24h after confirmed as valid. If the network expects the SIB updates to happen rather frequently, it can indicate this using SI (e.g., SIB1) that instructs a UE configured with eDRX to consider the SIB to be invalid after 3h. This gives enough flexibility to the network to configure SIB validity for eDRX UEs.

Proposal 1: By default, UEs configured with eDRX delete any stored version of a SIB after 24 hours from the moment it was successfully confirmed as valid.

Proposal 2: An optional field si-ValidityTime is introduced in SIB1. If the field is set to true, UEs configured with eDRX delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid.

A TP is provided in the Annex to implement the required changes.

Proposal 3: Adopt the text proposal included in the Annex of this document.

# 3 Conclusion

In this contribution, we discuss a remaining issue on SIB validity with eDRX and have the following observation and proposals:

Observation 1: If the legacy validity time of 3h is used for UEs configured with large eDRX cycles, there will be a negative impact on power savings, which will reduce the benefit of eDRX.

Proposal 1: By default, UEs configured with eDRX delete any stored version of a SIB after 24 hours from the moment it was successfully confirmed as valid.

Proposal 2: An optional field si-ValidityTime is introduced in SIB1. If the field is set to true, UEs configured with eDRX delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid.

Proposal 3: Adopt the text proposal included in the Annex of this document.

# 4 References

- TS 38.331

- TS 36.331

# 5 Annex

First change

##### 5.2.2.2.1	SIB validity

The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification, upon receiving request (e.g., a positioning request) from upper layers; and whenever the UE does not have a valid version of a stored SIB or posSIB or a valid version of a requested SIB.

When the UE acquires a MIB or a SIB1 or an SI message in a serving cell as described in clause 5.2.2.3, and if the UE stores the acquired SIB, then the UE shall store the associated areaScope, if present, the first PLMN-Identity in the PLMN-IdentityInfoList for non-NPN-only cells or the first NPN identity (SNPN identity in case of SNPN, or PNI-NPN identity in case of PNI-NPN) in the NPN-IdentityInfoList for NPN-only cells, the cellIdentity, the systemInformationAreaID, if present, and the valueTag, if present, as indicated in the si-SchedulingInfo for the SIB. If the UE stores the acquired posSIB, then the UE shall store the associated areaScope, if present, the cellIdentity, the systemInformationAreaID, if present, the valueTag, if provided in assistanceDataSIB-Element, and the expirationTime if provided in assistanceDataSIB-Element. The UE may use a valid stored version of the SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication. The valueTag and expirationTime for posSIB is optionally provided in assistanceDataSIB-Element, as specified in TS 37.355 [49].

NOTE:	The storage and management of the stored SIBs in addition to the SIBs valid for the current serving cell is left to UE implementation.

The UE shall:

1>	if the UE is configured with eDRX:

2>	if field si-ValidityTime is signalled in SIB1:

3>	delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

2>	else:

3>	delete any stored version of a SIB after 24 hours from the moment it was successfully confirmed as valid;

1>	else:

2>	delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

1>	for each stored version of a SIB:

2>	if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the UE is NPN capable and the cell is an NPN-only cell:

4>	if the first NPN identity included in the NPN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

5>	consider the stored SIB as valid for the cell;

3>	else if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the UE is NPN capable and the cell is an NPN-only cell:

4>	if the first NPN identity in the NPN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

5>	consider the stored SIB as valid for the cell;

3>	else if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

1>	for each stored version of a posSIB:

2>	if the areaScope is associated and its value for the stored version of the posSIB is the same as the value received in the posSIB-MappingInfo for that posSIB from the serving cell and the systemInformationAreaID included in the si-SchedulingInfo is identical to the systemInformationAreaID associated with the stored version of that posSIB:

3>	if the valueTag (see TS 37.355 [49]) for the posSIB received from the serving cell is identical to the valueTag associated with the stored version of that posSIB; or if the expirationTime (see TS 37.355 [49]) associated with the stored posSIB has not been expired:

4>	consider the stored posSIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the posSIB and the areaScope value is not included in the posSIB-MappingInfo for that posSIB from the serving cell and the cellIdentity for the posSIB received from the serving cell is identical to the cellIdentity associated with the stored version of that posSIB:

3>	if the valueTag (see TS 37.355 [49]) for the posSIB received from the serving cell is identical to the valueTag associated with the stored version of that posSIB; or if the expirationTime (see TS 37.355 [49]) associated with the stored posSIB has not been expired:

4>	consider the stored posSIB as valid for the cell;

Next change

### 6.2.2	Message definitions

< unmodified Subclauses removed>

#### –	SIB1

SIB1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. It also contains radio resource configuration information that is common for all UEs and barring information applied to the unified access control.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SIB1 message

-- ASN1START

-- TAG-SIB1-START

SIB1 ::=        SEQUENCE {

cellSelectionInfo                   SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinOffset                    INTEGER (1..8)                                              OPTIONAL,   -- Need S

q-RxLevMinSUL                       Q-RxLevMin                                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                   OPTIONAL,   -- Need S

q-QualMinOffset                     INTEGER (1..8)                                              OPTIONAL    -- Need S

}                                                                                                   OPTIONAL,   -- Cond Standalone

cellAccessRelatedInfo               CellAccessRelatedInfo,

connEstFailureControl               ConnEstFailureControl                                           OPTIONAL,   -- Need R

si-SchedulingInfo                   SI-SchedulingInfo                                               OPTIONAL,   -- Need R

servingCellConfigCommon             ServingCellConfigCommonSIB                                      OPTIONAL,   -- Need R

ims-EmergencySupport                ENUMERATED {true}                                               OPTIONAL,   -- Need R

eCallOverIMS-Support                ENUMERATED {true}                                               OPTIONAL,   -- Need R

ue-TimersAndConstants               UE-TimersAndConstants                                           OPTIONAL,   -- Need R

uac-BarringInfo                     SEQUENCE {

uac-BarringForCommon                UAC-BarringPerCatList                                           OPTIONAL,   -- Need S

uac-BarringPerPLMN-List             UAC-BarringPerPLMN-List                                         OPTIONAL,   -- Need S

uac-BarringInfoSetList              UAC-BarringInfoSetList,

uac-AccessCategory1-SelectionAssistanceInfo CHOICE {

plmnCommon                           UAC-AccessCategory1-SelectionAssistanceInfo,

individualPLMNList                   SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AccessCategory1-SelectionAssistanceInfo

}                                                                                                   OPTIONAL    -- Need S

}                                                                                                   OPTIONAL,   -- Need R

useFullResumeID                     ENUMERATED {true}                                               OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                                                    OPTIONAL,

nonCriticalExtension                SIB1-v1610-IEs                                                  OPTIONAL

}

SIB1-v1610-IEs ::=               SEQUENCE {

idleModeMeasurementsEUTRA-r16    ENUMERATED{true}                                                   OPTIONAL,  -- Need R

idleModeMeasurementsNR-r16       ENUMERATED{true}                                                   OPTIONAL,  -- Need R

posSI-SchedulingInfo-r16         PosSI-SchedulingInfo-r16                                           OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v1630-IEs                                                     OPTIONAL

}

SIB1-v1630-IEs ::=               SEQUENCE {

uac-BarringInfo-v1630            SEQUENCE {

uac-AC1-SelectAssistInfo-r16     SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AC1-SelectAssistInfo-r16

}                                                                                                   OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v1700-IEs                                                     OPTIONAL

}

SIB1-v1700-IEs ::=               SEQUENCE {

hsdn-Cell-r17                        ENUMERATED {true}                                              OPTIONAL,  -- Need R

ue-TimersAndConstants-RemoteUE-r17   UE-TimersAndConstants-RemoteUE-r17                             OPTIONAL,  -- Need R

uac-BarringInfo-v1700                SEQUENCE {

uac-BarringInfoSetList-v1700         UAC-BarringInfoSetList-v1700                               OPTIONAL   -- Cond MINT

}                                                                                                   OPTIONAL,  -- Need R

sdt-ConfigCommon-r17                 SDT-ConfigCommonSIB-r17                                        OPTIONAL,  -- Need R

featurePriorities-r17        SEQUENCE {

redCapPriority-r17           FeaturePriority-r17                                                OPTIONAL,

slicingPriority-r17          FeaturePriority-r17                                                OPTIONAL,

ce-Priority-r17              FeaturePriority-r17                                                OPTIONAL,

sdt-Priority-r17             FeaturePriority-r17                                                OPTIONAL,

...

}                                                                                                   OPTIONAL,  -- Need R

si-SchedulingInfo-v1700      SI-SchedulingInfo-v1700                                                OPTIONAL,  -- Need R

hyperSFN-r17                 BIT STRING (SIZE (10))                                                 OPTIONAL,  -- Need R

eDRX-Allowed-r17             ENUMERATED {true}                                                       OPTIONAL,  -- Need R

-- FFS whether eDRX-allowed is one or two bits, i.e. separate for idle/inactive

halfDuplexRedCapAllowed-r17  ENUMERATED {true}                                                       OPTIONAL,  -- Need R

-- FFS whether halfDuplexRedCapAllowed is kept, remove also from related procedure

cellBarredRedCap-r17         SEQUENCE {

cellBarredRedCap1Rx-r17      ENUMERATED {barred, notBarred},

cellBarredRedCap2Rx-r17      ENUMERATED {barred, notBarred}

}                                                                                                   OPTIONAL,  -- Need R

intraFreqReselectionRedCap-r17 ENUMERATED {allowed, notAllowed}                                     OPTIONAL,  -- Need S

nonCriticalExtension         SIB1-v17xx-IEs                                                         OPTIONAL

}

SIB1-v17xx-IEs ::=               SEQUENCE {

si-ValidityTime-r17            ENUMERATED {true}                                                    OPTIONAL   -- Need R

}

UAC-AccessCategory1-SelectionAssistanceInfo ::=    ENUMERATED {a, b, c}

UAC-AC1-SelectAssistInfo-r16 ::=     ENUMERATED {a, b, c, notConfigured}

SDT-ConfigCommonSIB-r17 ::=          SEQUENCE {

sdt-RSRP-Threshold-r17               RSRP-Range,

sdt-LogicalChannelSR-DelayTimer-r17  ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}  OPTIONAL, -- Need R

sdt-DataVolumeThreshold-r17          ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000,

byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000},

t319a-r17                            ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000,

ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

...

}

FeaturePriority-r17 ::= INTEGER (0..15)

-- TAG-SIB1-STOP

-- ASN1STOP

SIB1 field descriptions

cellBarredRedCap1Rx Value barred means that the cell is barred for a RedCap UE with 1 Rx branch, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs.

cellBarredRedCap2Rx Value barred means that the cell is barred for a RedCap UE with 2 Rx branches, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs.

cellSelectionInfo Parameters for cell selection related to the serving cell.

eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services as defined in TS 23.501 [32]. If absent, eCall over IMS is not supported by the network in the cell.

eDRX-Allowed The presence of this field indicates that extended DRX is allowed in the cell for UEs in RRC_IDLE and in RRC_INACTIVE. The UE shall stop using extended DRX if eDRX-Allowed is not present.

featurePriorities Indicates priorities for features, such as RedCap, Slicing, etc. These priorities are used to determine which FeatureCombinationPreambles the UE shall use when a feature maps to more than one FeatureCombinationPreambles, as specified in TS 38.321 [3]. A lower value means a higher priority. The network does not signal the same priority for more than one feature. The network signals a priority for all feature that map to at least one FeatureCombinationPreambles.

halfDuplexRedCap-Allowed FFS: The presence of this field indicates whether the cell supports half-duplex FDD RedCap UEs.

hsdn-Cell This field indicates this is a HSDN cell as specified in TS 38.304 [20].

hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around.

idleModeMeasurementsEUTRA This field indicates that a UE that is configured for EUTRA idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform EUTRA idle/inactive measurements.

idleModeMeasurementsNR This field indicates that a UE that is configured for NR idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform NR idle/inactive measurements.

ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode. If absent, IMS emergency call is not supported by the network in the cell for UEs in limited service mode.

intraFreqReselectionRedCap Controls cell selection/reselection to intra-frequency cells for RedCap UEs when this cell is barred, or treated as barred by the RedCap UE, as specified in TS 38.304 [20]. If not present, a RedCap UE treats the cell as barred, i.e.,the UE considers that the cell does not support RedCap.

q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for serving cell. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-QualMinOffset Parameter "Qqualminoffset" in TS 38.304 [20]. Actual value Qqualminoffset = field value [dB]. If the field is absent, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell.

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for serving cell.

q-RxLevMinOffset Parameter "Qrxlevminoffset" in TS 38.304 [20]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell.

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20], applicable for serving cell.

si-ValidityTime Indicates system information validity timer. If set to TRUE, the timer is set to 3h, otherwise the timer is set to 24h.

sdt-RSRP-Threshold RSRP threshold for UE to determine whether to perform SDT procedure, as specified in TS 38.321 [3].

sdt-DataVolumeThreshold Data volume threshold used to determine whether SDT can be initiated, as specified in TS 38.321 [3]. Value byte32 corresponds to 32 bytes, value byte100 corresponds to 100 bytes, and so on.

sdt-LogicalChannelSR-DelayTimer The value of logicalChannelSR-DelayTimer applied during SDT for logical channels configured with SDT, as specified in TS 38.321 [3]. Value in number of subframes. Value sf20 corresponds to 20 subframes, sf40 corresponds to 40 subframes, and so on. If this field is not configured, then logicalChannelSR-DelayTimer is not applied for SDT logical channels.

servingCellConfigCommon Configuration of the serving cell.

t319a Initial value of the timer T319a. Value ms100 corresponds to 100 milliseconds, value ms200 corresponds to 200 milliseconds and so on.

uac-AccessCategory1-SelectionAssistanceInfo Information used to determine whether Access Category 1 applies to the UE, as defined in TS 22.261 [25]. If plmnCommon is chosen, the UAC-AccessCategory1-SelectionAssistanceInfo is applicable to all the PLMNs and SNPNs in plmn-IdentityInfoList and npn-IdentityInfoList. If individualPLMNList is chosen, the 1st entry in the list corresponds to the first network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList, the 2nd entry in the list corresponds to the second network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList and so on. If uac-AC1-SelectAssistInfo-r16 is present, the UE shall ignore the uac-AccessCategory1-SelectionAssistanceInfo.

uac-AC1-SelectAssistInfo Information used to determine whether Access Category 1 applies to the UE, as defined in TS 22.261 [25]. The 1st entry in the list corresponds to the first network within all of the PLMNs and SNPNs across the plmn-IdentityList and npn-IdentityInfoList, the 2nd entry in the list corresponds to the second network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList and so on. Value notConfigured indicates that Access Category1 is not configured for the corresponding PLMN/SNPN.

uac-BarringForCommon Common access control parameters for each access category. Common values are used for all PLMNs/SNPNs, unless overwritten by the PLMN/SNPN specific configuration provided in uac-BarringPerPLMN-List. The parameters are specified by providing an index to the set of configurations (uac-BarringInfoSetList). UE behaviour upon absence of this field is specified in clause 5.3.14.2.

ue-TimersAndConstants Timer and constant values to be used by the UE. The cell operating as PCell always provides this field.

useFullResumeID Indicates which resume identifier and Resume request message should be used. UE uses fullI-RNTI and RRCResumeRequest1 if the field is present, or shortI-RNTI and RRCResumeRequest if the field is absent.

| Conditional Presence | Explanation |
| --- | --- |
| MINT | The field is optionally present, Need R, in a cell that provides a configuration for disaster roaming, otherwise it is absent. |
| Standalone | The field is mandatory present in a cell that supports standalone operation, otherwise it is absent. |

End of change