3GPP TSG-RAN2 Meeting #118-e	R2-2205752

eMeeting, 9-20 May 2022

Agenda Item:	6.9.4 UE capabilities

Source: 	Ericsson

Title:  	UE capabilities for UE power saving

Document for:	Discussion and Decision

# Introduction

The following topics are discussed in this contribution:

- AS and NAS UE capability for subgrouping

- Support for TRS/CSI-RS in idle/inactive and PEI

# Background

AS and NAS UE capability for subgrouping

Concerning subgroup capability RAN2 agreed:

- A PEI-capable UE must support at least UEID-based subgrouping method.

- RAN2 confirms that “PEI without subgrouping” can be implemented by configuring PEI plus UEID subgrouping with one subgroup.

- “PEI without subgrouping” can be configured by only one method.

It is optional, with UE capability signalling, for the UE to support subgrouping with Paging. If the UE supports subgrouping, it supports UEID-based subgrouping, signalled via AS capability (pei-SubgroupingSupportBandList capability per supported band in UERadioPagingInformation):

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| pei-SubgroupingSupportBandList-r17 Indicates whether the UE supports receiving paging early indication and UE subgrouping indication with UEID based subgrouping in DCI format 2_7 as specified in TS38.304 [21] for the bands corresponding to the supportedBandListNR. | UE | No | No | No |

The UE can in addition optionally support CN based subgrouping, signalled via NAS capability.

# Discussion

AS and NAS UE capability for subgrouping

During initial registration, but also mobility and periodic registration, the UE can set the "NR paging subgroup support indication" (NR-PSSI) bit in the 5GMM capability to indicate that it supports CN-based subgrouping. When the UE sets the NR-PSSI bit, the UE shall also indicate to support UEID based subgrouping:

Observation 1: The UE shall indicate AS capability for subgrouping when the UE indicates NAS capability for subgrouping.

Observation 2: When the UE supports subgrouping the UE also supports PEI, i.e. subgroup info can only be sent via PEI.

Support for TRS/CSI-RS in idle/inactive and PEI

The L1 availability indication can be send via PEI (DCI format 2-7) or Paging PDCCH (DCI format 1-0). RAN1 is considering to make support for L1 availability via PEI mandatory when the UE supports TRS/CSI-RS in idle/inactive [1]:

TRS occasions for idle/inactive UEs

1. Support reading TRS configuration from SIB

[2. Support receiving L1 indication for TRS availability]

TRS/CSI-RS in idle/inactive is an optional feature without capability signalling, captured in chapter 5 of 38.306:

TRS occasions for idle mode and RRC_INACTIVE UEs It is optional for UE to support reading TRS configuration from SIB and receiving L1 indication for TRS availability

It is not clear what the outcome of the RAN1 discussion will be:

- If the UE supports TRS in idle/inactive, does the UE support L1 indication via PEI and/or Paging PDCCH? And if via PEI, does this imply that the UE also support subgrouping via PEI?

The outcome of the RAN1 discussion may also impact the gNB, i.e. if the gNB supports TRS in idle/inactive it also has to support L1 availability signalling and PEI:

Proposal 1: Wait for RAN1 decision concerning PEI and TRS/CSI-RS in idle/inactive.

# Summary

RAN2 is kindly asked to discuss UE capabilities for UE power saving:

Observation 1: The UE shall indicate AS capability for subgrouping when the UE indicates NAS capability for subgrouping.

Observation 2: When the UE supports subgrouping the UE also supports PEI, i.e. subgroup info can only be sent via PEI.

Proposal 1: Wait for RAN1 decision concerning PEI and TRS/CSI-RS in idle/inactive.

# References

- R1-2202927, LS on updated Rel-17 RAN1 UE features list for NR, RAN1#108-e