3GPP TSG-RAN WG2 Meeting #118-e	R2-2205987

# Electronic Meeting, 9 – 20 May, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3146 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Uu threshold handling when configured with measurements of L2 U2N Relay UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2022-5-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the current version of TS 38.331, the Remote UE can receive/ transmit discovery message to evaluate AS layer conditions, i.e., SD-RSRP only when the Uu threshold conditions are met, as stated in clause 5.8.15. --------------------------------------------TS 38.331------------------------------------------ ### 5.8.15 NR sidelink U2N Remote UE operation #### 5.8.15.1 General This procedure is used by a UE supporting NR sidelink U2N Remote UE operationconfigured by upper layers to receive/ transmit NR sidelink discovery message to evaluate AS layer conditions. #### 5.8.15.2 NR Sidelink U2N Remote UE threshold conditions A UE capable of NR sidelink U2N Remote UE operation shall: 1> if the threshold conditions specified in this clause were not met: 2> if threshHighRemote is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRemote by hystMaxRemote if configured, or 2> if the UE has no suitable cell: 3> consider the threshold conditions to be met (entry); 1> else: 2> if the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshHighRemote if configured: 3> consider the threshold conditions not to be met (leave);. On the other hand, a UE should follow dedicated configuration received from gNB, that is a UE should perform Relay UE measurement if configured. However, it is not specified in the current spec whether the Remote UE should perform Relay UE measurement if it receives the measurement configuration for relay discovery but the Uu threshold conditions are not met. |
| Summary of change: |  | Clarify that Remote UE shall ignore the Uu threshold conditions if it is configrued with the measurement objects associated with Relay. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality SL relay Inter-operability: If the network is implemented according to this CR while the UE is not, the remote UE cannot perform measurement and reporting on Relay UE(s) according to network configuration. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE actions specified in the spec TS 38.331 conflicts in case Remote UE receives the measurement configuration for relay discovery but the Uu threshold conditions for discovery are not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.8.15.2	NR Sidelink U2N Remote UE threshold conditions

A UE capable of NR sidelink U2N Remote UE operation shall:

1>	if the threshold conditions specified in this clause were not met:

2>	if threshHighRemote is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRemote by hystMaxRemote if configured, or

2> if the UE has no suitable cell:

3>	consider the threshold conditions to be met (entry);

2> if the UE is configured with NR sidelink measurements of L2 U2N Relay UEs:

3>	consider the threshold conditions to be met (entry);

1>	else:

2>	if the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshHighRemote if configured:

3>	consider the threshold conditions not to be met (leave);

END OF CHANGE