3GPP TSG RAN WG2#118-e	R2-2206752

e-Meeting, 9th - 20th May, 2022

3GPP TSG-RAN WG4 Meeting #103-e	R4-2210601

Electronic Meeting, 9 – 20 May, 2022

Title:	Further reply LS on condition for PRS measurement outside the MG

Response to:	R1-2112883

Release:	Release 17

Work Item:	NR_pos_enh-Core

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact Person:

#### Name:	Li Zhang

E-mail Address:	zhangli164@huawei.com

Attachments: -

1. Overall Description:

RAN4 has provided initial response on the condition for PRS measurement outside the MG in R4-2206981. In RAN4#103-e, RAN4 further discussed the condition and would like to provide the following information.

- The applicability condition on Rx timing difference between the serving cell and a neighbor cell/TRP for PRS measurements within a PPW is max ∆T≤THR, where - ∆T is the time difference between the start of a slot containing PRS from the neighbor cell/TRP and the start of the closest slot from the serving cell, , and - the range of ∆T is determined by the expected RSTD and expected RSTD uncertainty in the assistance data. - THR is the threshold, which is up to UE capability (as already informed in R4-2206981) - The candidate values for THR include {CP length, ¼ symbol length, ½ symbol length, ½ slot length}

RAN4 respectfully asks RAN1 and RAN2 to take the above information into account in the future work for PRS measurement outside MG.

2. Actions:

To RAN2 and RAN1:

RAN4 respectfully asks RAN2 and RAN1 to take the above information into account.

3. Date of Next TSG-RAN4 Meetings:

RAN WG4 Meeting #103-e	May 16 – May 27, 2022	Electronic Meeting

RAN WG4 Meeting #104	August 22 – August 26, 2022	Toulouse, France