- 3GPP TSG RAN WG2 Meeting #118e      	                         R2-220xxxx
E-Conference, 9th – 20th May 2022

Agenda item:	6.12.2.1

Source:	ZTE Corporation

Title:	[AT118-e][105][RedCap] NCD-SSB aspects

WID/SID:	NR_redcap-Core – Release 17

Document for:	Discussion and Decision

# Introduction

This document summarizes the following offline discussion.

[Pre118-e][105][RedCap] NCD-SSB aspects (ZTE)

Initial scope: Continue the discussion on NCD-SSB aspects, based on R2-2206143, including a possible reply LS to R2-2204486 (RAN4 LS on NCD-SSB issues)

Initial intended outcome: Summary of the offline discussion with e.g.:

         Text/proposals for a possible reply LS to R2-2204486

         List of proposals for agreement (if any)

         List of proposals that require online discussions

         List of proposals that should not be pursued (if any)

Deadline (for companies' feedback): Tuesday 2022-05-10 0800 UTC

Deadline (for rapporteur's summary in R2-2206195): Tuesday 2022-05-10 1000 UTC

# Contact information

| Company | Name and email address |
| --- | --- |
| ZTE | LiuJing (liu.jing30@zte.com.cn) |
| MediaTek | Pradeep Jose (pradeep dot jose at mediatek dot com) |
| Qualcomm | Linhai He (linhaihe@qti.qualcomm.com) |
| Apple | Naveen Palle (naveen.palle@apple.com) |
| NEC | Hisashi Futaki (hisashi.futaki @ nec.com) |
| Samsung | Jaehyuk Jang (jack.jang@samsung.com) |
| InterDigital | Keiichi Kubota (keiichi.kubota@interdigital.com) |
| Huawei, HiSilicon | Yulong (shiyulong5@huawei.com) |
| vivo | Chenli5g@vivo.com |
| DENSO | Hideaki Takahashi (hideaki.takahashi.j6e@jp.denso.com) |
| Intel | Yi. Guo (yi.guo@intel.com) |
| OPPO | Haitao Li (lihaitao@oppo.com) |

# Discussion

## Serving Cell measurements

For serving cell measurements, as indicated in AI summary [1], the discussion focus on following two questions:

- Q1: Whether the UE can be configured to perform serving cell measurements on CD-SSB when its active BWP is associated with NCD-SSB?

- Q2: How to configure the UE to perform serving cell measurements (i.e. signalling design)?

And following proposals are provided in AI summary.

Proposal 1: To discuss whether UE can be configured to perform serving cell measurements on CD-SSB when the UE’s active BWP is associated with NCD-SSB. Proposal 2.1: If answers “Yes” to P1, to select one of the following options: - Solution A-1: Reuse existing servingCellMO IE. - Solution A-2: Optionally configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE if BWP-DownlinkDedicated contains nonCellDefiningSSB-r17. If the field is present, the UE shall use this servingCellMO, otherwise, the UE uses legacy servingCellMO. Proposal 2.2: If answers “No” to P1, to select one of the following options: - Solution B-1: Add servingCellMO-List-NCD-r17 in ServingCellConfig, each entry associated to the one NCD-SSB MO, the UE selects corresponding servingCellMO upon BWP switching. - Solution B-2: Configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE, the field is configured in the BWP if BWP-DownlinkDedicated contains nonCellDefiningSSB-r17.

In this section, companies are invited to express your views to the two questions and above proposals:

Q1: In case a RedCap UE’s activate BWP contains only NCD-SSB, whether the UE can be configured to perform serving cell measurements on CD-SSB?

(E.g. when all neighbour cell measurements are performed on CD-SSB and gap is already configured)

| Company | Yes or No | Comments |
| --- | --- | --- |
| Vodafone | Yes | In case UE’s active BWP only contains NCD-SSB, serving cell measurement can be performed on either NCD-SSB or CD-SSB |
| ZTE | Yes | If not all neighbour cells have deployed NCD-SSB, the network will configure the UE to measure CD-SSB frequency for triggering handover, and gap will be provided because CD-SSB is outside UE’s activate BWP (if the UE does not report ”no-gap” for intra-frequency in NeedForGap). In this case, the network may expect the UE to also perform serving cell measurements on CD-SSB, similar to Rel-15 UEs. So the spec should allow such flexibility. |
| MediaTek | No | The reason for introducing the NCD-SSB concept was so that the UE can use the NCD-SSB for those purposes that the CD-SSB would normally be used for, when operating in a BWP that only includes the NCD-SSB. In this case, if we require the UE to monitor the CD-SSB instead of the NCD-SSB, then we defeat the purpose of the introduction of the NCD-SSB altogether! [Rapp-ZTE]: We would like to clarify the discussion here only relates to RRM measurements, it does not impact other physical operations, so for RLM/BFD, QCL...etc, the UE should still use NCD-SSB. This is similar to Rel-15 operation, e.g. when UE’s active BWP does not contain SSB, the UE uses other RS (i.e. CSI-RS) for RLM/BFD and QCL, but the UE can still perform RRM measurements on SSB with gap assisted. [MTK2] The point of introducing NCD-SSB was so that it can be used as an alternative to the CD-SSB. Now you’re suggesting that it’s a partial alternative, which can be used in some cases (e.g. RLM) and not in others (e.g. RRM). Why? What is there to gain with such a hybrid approach? As far as we can see, such a hybrid approach only adds specification and implementation confusion, with no clear benefit. |
| Qualcomm | See comment | In typical deployment scenarios, if Redcap UE’s dedicated BWP does contain a SSB (does not matter whether it is CD-SSB or NCD-SSB), then its serving cell measurements should be performed based on this particular SSB, to avoid measurement gaps. In some deployment scenarios, e.g. UE’s dedicated BWP contains NCD-SSB but no intra-frequency neighbor cells are configured with SSB on the same frequency as NCD-SSB in UE’s serving cell, it is understandable that network may want to configure serving cell measurements based on CD-SSB instead of NCD-SSB via measurement gaps. But we are not sure how likely this scenario is in actual deployment… We are fine to leave this issue to network configuration, as long as serving cell measurements in the both types of scenarios above can be efficiently performed. That is why we think the configuration of servingCellMO should be BWP specific. |
| Apple | Neutral | While we agree with MediaTek’s view on NCD-SSB standing in for CD-SSB, we also see that this is NW deployment option. We are ok to go with majority. |
| NEC | Yes | We expect there could be the case where some of neighbour cells do not support NCD-SSB, although this would not be the typical deployment. We also assume, as exemplified by ZTE, that from RRM measurement point of view, the scenario is similar to serving cell measurement on SSB, while the active BWP does not contain the SSB in Rel-15. |
| Samsung | Yes | Assuming a gap is configured for the measurement. |
| Interdigital | Yes | We share Qualcomm view that RedCap UE performs serving cell measurement based on the configured SSB and CD-SSB measurement makes sense for the case that the neighbouring cells aren’t configured with any SSB on the same frequency as the NCD-SSB of the current serving cell. Thus we should allow NW to configure CD-SSB to measure for the serving cell measurement. |
| Huawei, HiSilicon | Yes | Agree with Qualcomm that we should give the NW configuration flexibility. |
| Vivo | See comment | In our understanding, NCD-SSB could be used the same as CD-SSB during measurement. In this way, serving cell measurement should be performed always based on CD-SSB. From UE point of view, it is not reasonable to always switch to BWP with CD-SSB for serving cell measurement. Considering this question is asked from network configuration point of view, we assume it is possible that UE could be configured to perform serving cell measurements on CD-SSB in case a RedCap UE’s active BWP contains only NCD-SSB. But we would like to clarify more about the motivation or scenario. As rapporteur mentioned, it is possible when not all neighbour cells have deployed NCD-SSB. The question is whether this scenario is typical. We think it is possible at the early stage of deployment, but it would be a corner case in mature deployment. |
| DENSO | Yes | Agree with Qualcomm to allow the flexible NW configuration. |
| Intel | Yes | We do not see the problem to do so. |
| OPPO | Yes | Agree with Qualcomm. |

If answers “Yes” to Q1, companies are invited to provide your views to below solutions:

    - Solution A-1: Reuse existing servingCellMO IE (based on the assumption that RRCReconfiguration is always needed (e.g. to reconfigure UE CBW) when switching from a BWP associated with CD-SSB and a BWP associated with NCD-SSB).

    - Solution A-2: Optionally configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE when the BWP-DownlinkDedicated contains nonCellDefiningSSB-r17. If the field is present, the UE shall use this servingCellMO, otherwise, the UE uses legacy servingCellMO.

    - Solution A-3: Optionally configures servingCellMO-List-NCD-r17 in ServingCellConfig, each entry associated to the one NCD-SSB MO, the UE selects corresponding servingCellMO upon BWP switching. For a NCD-SSB contained in UE’s activate BWP, if corresponding MO is NOT included in servingCellMO-List-NCD-r17, the UE shall use legacy servingCellMO.

Different from the original proposal 2.1 in AI summary, rapporteur has added Solution A-3 based on received offline comments. The solution A-3 is similar to Solution B-1, but it supports optional configuration.

Q2: If answers “Yes” to Q1, which solution do you prefer?

| Company | Solution A-1 or Solution A-2 or Solution A-3 | Comments |
| --- | --- | --- |
| Vodafone | Solution A-1 |  |
| ZTE | Prefer Solution A-1, can accept Solution A-2 if majority supports | As we mentioned in R2-2204547, we think in real deployment, the network will not deploy two SSBs within 20MHz, so for RedCap UE, switching from a BWP containing CD-SSB to another BWP containing NCD-SSB always requires RRCReconfiguration, because the UE specific channel bandwidth needs to be reconfigured. So the network is able to reconfigure existing servingCellMO field in the same RRC message. Thus we think Solution A-1 is sufficient for RedCap UEs in Rel-17. However, if most companies want to consider the case of deploying two SSBs within 20MHz, we can accept Solution A-2. We don’t prefer Solution A-3, because the UE is required to compare the NCD-SSB frequency (indicated in BWP-DownlinkDedicated) with the multiple Mos provided by the servingCellMO-List to determine the applicable serving cell MO, so it is more complex than Solution A-2. |
| MediaTek | None of the above | The NCD-SSB must always be used for serving cell measurements |
| Qualcomm | A-2 | We are the proponent of the proposal. The main motivation is to enable more flexibility for network in configuring serving cell measurements when UE’s dedicated BWP contains NCD-SSB. As we have clarified in our reply to Q1, it may not be an efficient approach if we mandate serving cell measurements only on a specific type of SSB. Hence it is better to allow servingCellMO to be BWP specific. With Solution A-1, network may have to RRC reconfigure servingCellMO whenever UE switches its BWP. Although RRC Reconfig and BWP switch may not need to have the same timeline, that is still a significant overhead, which should be avoided. |
| NEC | Solution A-1 | With the same reason as ZTE that the network will not normally configure CD-SSB and NCD-SSB within 20 MHz, A-1 would be flexible enough. |
| Samsung | A-1 | - |
| Interdigital | A-2 | We can avoid RRCReconfiguration per BWP switch by A-2 and so we can take advantage of the existing BWP switch mechanism via lower layer signalling + it can cover the case the fallback to default BWP upon the inactive timer expiry. |
| Huawei, HiSilicon | Support A-2/3, but not fine with A-1 | Minor wording update to A-2: “when UE is on this BWP, If the field is present, the UE shall use this servingCellMO, otherwise, the UE uses legacy servingCellMO.” On the comments from ZTE “because the UE specific channel bandwidth needs to be reconfigure”, the downlinkChannelBW-PerSCS-List can be configured larger than 20M. This is the carrier BW of NW, just configuring the UE specific value, which has nothing to do with UE capaiblity. NCD-SSB will lose its benefits if it requires RRC reconfiguration upon DCI based BWP switching, as commented by Qualcomm. |
| vivo | None | We think it’s straightforward that UE is configured to perform serving cell measurements on NCD-SSB in case a RedCap UE’s active BWP contains only NCD-SSB. In this case, there is no need to configure an optional indication in BWP-DownlinkDedicated. On the other hand, if the UE is configured to perform serving cell measurements on CD-SSB, existing servingCellMO could be used to configure the UE performs serving cell measurement based on the CD-SSB. With this, there is no need to configure separate servingCellMO. |
| DENSO | A-1 | Agree with ZTE if the carrier bandwidth is 20 MHz. On the other hand, if the carrier bandwidth is 100 MHz and a 20 MHz portion of the carrier bandwidth is used for RedCap UE, there might be the scenario that NW may transmit multiple NCD-SSB, as well as CD-SSB. Even with that, we tend to agree that reconfiguration is sufficient for the initial release of RedCap. |
| Intel | Solution A-1 or Solution A-3 | To our understanding, solution A-1 is the default solution (i.e. aligned with current specification). A-3 is kind of optimization, but we see benefits on this. So we would also be fine with it. |
| OPPO | Solution A-1 |  |

If answers “No” to Q1, companies are invited to provide your views to below solutions:

    - Solution B-1: Add servingCellMO-List-NCD-r17 in ServingCellConfig, each entry associated to the one NCD-SSB MO, the UE selects corresponding servingCellMO upon BWP switching.

      - Note: for all configured NCD-SSB, the corresponding MO must be included in servingCellMO-List-NCD-r17.

    - Solution B-2: Configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE, the field is configured in the BWP if BWP-DownlinkDedicated contains nonCellDefiningSSB-r17.

      - Note: for each BWP configured with NCD-SSB, its BWP-sepecific servingCellMO field must be included and set to the corresponding NCD-SSB MO.

Please note, the difference between Solution B-1 and Solution A-3 (or Solution B-2 and Solution A-2) is that, the NCD-SSB MO must be configured as serving cell MO, to ensure the UE always perform serving cell measurements on NCD-SSB when its active BWP contains NCD-SSB.

Q3: If answers “No” to Q1, which solution do you prefer?

| Company | Solution B-1 or Solution B-2 | Comments |
| --- | --- | --- |
| MediaTek | B-1 (open to B-2 as well) | We prefer B-1 as it’s a natural extension of the current serving cell MO signaling. |
| vivo | B-1 | B-1 is most straightforward solution. |

## Neighbour cell measurements

For neighbour cell measurements, the following proposal is provided in AI summary:

Proposal 4: To discuss if there is any problem in applying existing neighbour cell measurements framework to RedCap UEs.

To facilitate the discussion on neighbour cell measurements, the corresponding proposals and rapporteur analysis are kept as below:

| Source | Related proposals |
| --- | --- |
| HW [R2-2205038] | Proposal 3a: When RedCap UE is on the DL BWP with CD-SSB: - UE only performs the measurement on the measurement object associated with CD-SSB (for both serving and neighbor cell measurement); - UE does not perform the measurement on the measurement object associated with the NCD-SSBs configured to its serving cell. Proposal 3b: When RedCap UE is on the DL BWP with NCD-SSB: - UE at least performs the measurement on the measurement object associated with NCD-SSB in its active DL BWP, if configured (for both serving and potential neighbor cell measurement); - UE does not perform the measurement on the measurement object associated with other NCD-SSBs, which is configured to its serving cell but is not on its active DL BWP. - UE may in addition perform the measurement on the measurement object associated with CD-SSB, based on the NW indication, which is a 1bit indication per DL BWP. Proposal 3c: When UE is on the DL BWP without any SSBs: - UE only performs the measurement on the measurement object associated with CD-SSB (for both serving and neighbor cell measurement); - UE does not perform the measurement on the measurement object associated with the NCD-SSBs configured its serving cell. |
| Qualcomm [R2-2204812] | Proposal 4. If UE’s active DL BWP is configured with a BWP-specific servingCellMO, network can include in the BWP-DownlinkDedicated IE the measurement object(s) whose frequency(s) should be changed to the frequency of the servingCellMO in this BWP. |

In general, above proposals are discussing how to perform neighbour cell measurements when the SSB used for serving cell measurements is changed. If legacy principle is followed, then the UE is required to measure the Mos that are associated with measID, and for Mos that need gap assistance but gap is not configured, the UE will not measure those Mos. It is up to the network to decide which target frequencies should be measured and the network can configure Mos accordingly.

Rapporteur understands above proposals want to introduce some enhancements so that MO reconfiguration can be avoid. However, rapporteur also see some problem (or limitation) of above proposals. For instance, even if RedCap UE is on the DL BWP with CD-SSB, the network may want the UE to also measure NCD-SSB frequency (with gap assistance), so that the network knows whether it can trigger BWP addition/release, the spec should not disallow the network to configure inter-frequency measurements on NCD-SSB.

During this offline, companies are invited to show your view on neighbour cell measurements.

Q6: Do companies think anything needs to be fixed or updated for neighbour cell measurements for RedCap Ues?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | So far, no | We think the existing RRM principle is quite flexible for neighbour cell measurements, for instance: 1. The UE is required to measure neighbour cells when the MO is associated with measID. (If network wants the UE to only perform serving cell on a frequency, the network can configure a MO without associating to any reportConfig and measID) 2. For gap-assisted measurements, the UE only perform the measurement when gap is configured. So if the network wants the UE to stop measuring the neighbour cells when the SSB is outside UE’s active BWP, the network can configure measurements without gap configuration. So far, we think the network has flexibility in configuring neighbour cell measurement, but we are open to hear other companies’ views. |
| MediaTek | No | This can be left to NW implementation with appropriate neighbour cell MO configuration |
| Qualcomm | Yes | The following example can be used to illustrate the motivation and need for some enhancements to the existing framework, if servingCellMO is BWP-specific: In this example, UE is configured with two Mos, one on each type of SSB. Suppose UE switches from BWP#1 to BWP#2 and hence also switches its serving cell measurement from CD-SSB to NCD-SSB configured in BWP#2. However, UE’s neighbor cell measurements are still based on MO#1. Hence network needs a way to signal UE to change those measurements to NCD-SSB (if network thinks this change is necessary). One may argue that no enhancement is needed in this case as network can RRC Reconfig UE’s neighbor cell measurements. But the issue here is the same as the one we discussed in Q1 for serving cell measurement. It is more efficient if neighbor cell measurements, just like serving cell measurements, are switched when BWP switch results in a new type of SSB to measure. Please note that in this proposal, network still has full control whether neighbor cell or serving cell measurements should switch BWP. For example, if network does not include any MO under BWP-DedidcatedDownlink, then no neighbor cell measurements need to be switched. |
| Apple | So far no | Similart views as ZTE, but we understand that this depends on outcome of prop1/2 (Q1/Q2/Q3) |
| NEC | So far no | We still do not see need of specification change to introduce something more in Rel-17. |
| Samsung | No | Share the view with many others. |
| Interdigital | Yes | Share Qualcomm view. |
| Huawei, HiSlicon | Yes | Agree with Qualcomm that the motivation is to avoid the MO/measurement gap reconfiguration via RRC reconfiguration upon DCI triggered BWP switching. This indeed depends on the agreements from Q1-3. At least, we should discuss/clarify the neighbor cell measurement behaviors if RAN2 agree the BWP specific serving cell MO. As to the rapporteur comments, we believe the inter-frequency MO will not overlap with the NCD-SSB of serving cell’s BWPs typically. |
| vivo | No | The current measurement configuration is enough for neighbour cell measurements, and there is no need to introduce additional enhancements for neighbour cell measurements for RedCap UEs. It should be up to network configuration how to configure the measurement on neighboring cells. |
| DENSO | so far no | Similar to Apple’s view, is it related to Q1 to Q3. |
| Intel | No |  |
| OPPO | No |  |

## About reply LS to R2-2204486

### Definition of intra-frequency measurements

The discussion relates to the following questions from RAN4 in LS R2-2204486:

Measurement related: - Is it possible to configure CD-SSB and/or multiple NCD-SSB(s) for serving cell measurements? - If yes, is it feasible from RAN2’s signalling design perspective to explicitly indicate which SSB is the reference SSB to define intra-frequency measurement?

And the following proposal is provided in AI summary.

Proposal 3: For measurement related questions in R2-2204486, reply to RAN4 with the following RAN2 understandings: - Either CD-SSB or NCD-SSB can be configured for serving cell measurements, not both. - The SSB used for serving cell measurements may be changed upon BWP switching. - The outcome of Proposal 1. - From RAN2 signalling point of view, it is feasible to explicitly indicate which SSB is the reference SSB to define intra-frequency measurements. But the indicated SSB may be different from the SSB that UE performs serving cell measurements on. So RAN2 expects further confirmation from RAN4 before introducing such signalling.

As indicated in AI summary, there are two different understandings on how to define the intra-frequency measurements:

- Understanding-1: The SSB used by the UE to perform serving cell measurements is reference SSB;

- Understanding-2: The reference SSB is explicitly configured by the network, it can be different from the SSB that UE performs serving cell measurements.

Although RAN4 did not ask RAN2 how to define the intra-frequency measurements, to avoid misunderstanding, it is better to clarify RAN2 understandings as much as possible. So the first three bullets in Proposal 3 are provided regarding to RAN4’s first question.

- Is it possible to configure CD-SSB and/or multiple NCD-SSB(s) for serving cell measurements?

Companies are invited to show your views on the content/proposals for a possible reply LS to R2-2204486.

Q4: For the first measurement related question, do companies agree to reply RAN4 with below RAN2 understandings?

- Either CD-SSB or NCD-SSB can be configured for serving cell measurements, not both.

- The SSB used for serving cell measurements may be changed upon BWP switching.

- The outcome of Proposal 1.

| Company | Yes or No | Comments |
| --- | --- | --- |
| Vodafone | yes | Should follow the outcome from proposal 1 from section 3.1 in this document. |
| ZTE | Yes | Strictly speaking, the second bullet also relates to the discussion of Q1. If solution A-1 is selected, then the second bullet is not valid anymore. In addition, we can also mention that a RedCap UE may be configured with multiple NCD-SSBs, but one BWP can contain at least one NCD-SSB. |
| MediaTek | Partially | We have to clarify that ‘not both’ is BWP specific and not cell specific. For example, with solutions A2, A3, B1 and B2 in section 3.1, the RRC configuration can include CD and multiple NCD SSBs for a given cell, but when operating in a BWP only one of the configured SSBs is used for serving cell measurements. So, we propose the following modification: - CD-SSB and/or multiple NCD-SSBs can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements within a configured BWP. - The SSB used for serving cell measurements may be changed upon BWP switching. - The outcome of Proposal 1. |
| Qualcomm | Yes | We support the clarification to the first bullet proposed by MediaTek |
| Apple | Yes, but with the clarification by MediaTek |  |
| NEC | Yes | It would be good to confirm this after concluding the Q1/P1. |
| Samsung | Yes | We also support the clarification from MediaTek. |
| Interdigital | Yes | We support the clarification proposed by MediaTek. |
| Huawei, HiSilicon | Yes, with the update | We support the intention from MediaTek. We suggest to minor update - CD-SSB and/or multiple NCD-SSBs can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements within an active configured BWP. |
| vivo | Yes | - Agree the clarification proposed by MediaTek. 2nd bullet could be also updated based on the output of above questions. |
| DENSO | Yes | Agree on MediaTek’s clarification |
| Intel | Yes | Agree the proposed change from MediaTek. |
| OPPO | Yes | Agree on MediaTek’s clarification |

Regarding the second question from RAN4, RAN4 asks about the feasibility of explicit indication. As pointed out in AI summary, by using RRC signalling, the indicated referenced SSB can be different from the SSB that used for serving cell measurements, because BWP can be changed via DCI. Then both RAN2 and RAN4 may need more discussion on how to interpret UE gap capability.

So in Proposal 3, rapporteur suggests to highlight this aspect, companies are invited to show your views for the possible reply LS to R2-2204486.

Q5: For the second measurement related question, do companies agree to reply RAN4 with below RAN2 understandings?

- From RAN2 signalling point of view, it is feasible to explicitly indicate which SSB is the reference SSB to define intra-frequency measurements. But the indicated SSB may be different from the SSB that UE performs serving cell measurements on. So RAN2 expects further confirmation from RAN4 before introducing such signalling.

| Company | Yes or No | Comments |
| --- | --- | --- |
| Vodafone | yes | I think it is good to have the clarification before final answer. |
| ZTE | Yes | We have some concern on the RRC indicated reference SSB. The definition of intra-frequency measurement will be used to determine UE’s gap requirement, if the SSB used for actual serving cell measurements can be different from the configured reference SSB, we are afraid mismatch will happen between the network and the UE. For instance, the UE reports “no gap” for intra-frequency measurements in NeedForGap reporting. Network has set reference SSB to CD-SSB, but at the moment the UE is performing serving cell measurements on NCD-SSB, then for neighbour cell measurements on CD-SSB frequency, the network will assume gap is not needed. But this may not be true, because from UE RF point of view, measuring cell on CD-SSB frequency may need RF retuning. |
| MediaTek | No | The purpose of defining intra and inter frequencies is to know which set of UE capabilities and RAN4 requirements apply. For example, measurement gaps are not needed for intra frequency measurements but are needed for inter frequency measurements. If we change the definition altogether, such that the ‘intra frequency’ is different from the frequency of the SSB that the UE performs cell measurements on, we end up with a situation where intra frequency measurements would REQUIRE measurement gaps, while inter frequency measurements would NOT REQUIRE measurement gaps! Such an approach will create a mess out of the specifications and we’ll be dealing with the fallout of this mess for the next two years… We therefore propose the following response: From RAN2 signalling point of view, the NW and the UE always know which SSB is the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB can be used to define intra-frequency measurements. This reference SSB may change with BWP switching. [Rapp-ZTE] On behalf of ZTE, we have the same understanding that the SSB used for serving cell measurements should be regarded as “reference SSB”, so we are fine with the proposed response. |
| Qualcomm | No | We can’t agreement with this statement “But the indicated SSB may be different from the SSB that UE performs serving cell measurements on”. In our view, we do not need to change the current understanding that the frequency used in serving cell measurement should be the one used to define intra- vs inter-frequency. We can only need to update the definition that the frequency used in serving cell measurements can be either CD-SSB or NCD-SSB. |
| Apple | No | We have the same view as Qualcomm and Mediatek, and just the “which” SSB needs discussion, and based on this the existing intra/inter definition can be reused. We are ok with the wording suggested by Mediatek. |
| Samsung | No | Proposal from MediaTek looks good to us. |
| Interdigital | No | MediaTek’s proposal is fine for us. |
| Huawei, HiSilicon | See comment | In any case, the 1st sentence is correct, we can always just confirm the feasibility “From RAN2 signalling point of view, it is feasible to explicitly indicate which SSB is the reference SSB to define intra-frequency measurements.” And, the first part from MTK wording is also reasonable “From RAN2 signalling point of view, the NW and the UE always know which SSB is the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB can be used to define intra-frequency measurements.” So, the key point/controversial part is on whether “This reference SSB may change with BWP switching.” To be honest, this is actually R4 discussion. For sure, RAN2 can also conclude on this and send the agreement back to R4 to help R4 discussion. But, if RAN2 has no consensus, we’d better not to touch this and send the LS immediately to ask for R4 to give the feedback/confirmation by the end of meeting. The most important thing is we need to conclude this in May meeting, since there is RAN2 impact anyway. So, we suggestion the proposal as: Pa: reply RAN4 with below RAN2 understandings: - From RAN2 signalling point of view, it is feasible to explicitly indicate which SSB is the reference SSB to define intra-frequency measurements - From RAN2 signalling point of view, the NW and the UE always know which SSB is the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB can be used to define intra-frequency measurements. Pb: RAN2 to discuss online on whether this reference SSB may change with BWP switching.. |
| Vivo | No | As we prefer the Understanding-1 above, we don’t see any motivation that network indicates UE to perform serving cell measurement on one SSB, while indicating a different SSB as reference SSB. If the network thinks the NCD-SSB in the active BWP is not suitable to be set as reference SSB, it could indicate UE to perform serving cell measurement on CD-SSB as legacy, rather than introducing an explicit indication. |
| DENSO | No | Same view as MediaTek, Qualcomm and Apple. There shouldn’t be mismatch between NW and UE as to which SSB is used for the serving cell measurement. We’re fine with MediaTek’s text proposal. |
| Intel | No | Agree the proposed change from MediaTek. |
| OPPO | No | MediaTek’s proposal is fine for us. |

### Handover to BWP without CD-SSB

The discussion relates to the following questions from RAN4 in L;)S R2-2204486:

Handover related: - Are the following Handover scenarios valid from RAN2’s perspective? - Handover to a target cell’s specific Redcap BWP associated with NCD-SSB directly other than to the initial BWP associated with CD-SSB - Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH - the specific Redcap BWP associated with NCD-SSB - the specific Redcap BWP without presence of NCD-SSB

And following proposals are provided in AI summary:

Proposal 5: RAN2 confirms that RedCap UE can be configured to handover directly to a target cell’s specific Redcap BWP associated with an NCD-SSB, i.e. the UE directly sync to the NCD-SSB and perform RACH on that BWP. Proposal 5.1: In handover command, if the first active BWP is associated with NCD-SSB, the smtc field included reconfigurationWithSync is configured according to the NCD-SSB of target cell. Proposal 6: To discuss whether handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH can be supported.

In this section, companies are invited to show your views on the support of scenario 1 and scenario 2 separately.

Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB directly other than to the initial BWP associated with CD-SSB

Q7: From RAN2 perspective, do companies think Scenario 1 can be supported (i.e. the UE can directly sync to the NCD-SSB and perform RACH on that BWP)?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Vodafone | No; Depending on assumption | Vodafone (we don’t see a significant gain on supporting this scenario, under the assumption that CORESET#0 is not in the NCD-SSB, leading to the reconfiguration to happen anyway to obtain information to perform RACH), but if anyone could confirm that our assumption is correct this would be helpful. |
| ZTE | Yes | NCD-SSB also provides SFN/subframe information, so it is feasible to sync to NCD-SSB to obtain DL timing, SIB1 can be provided via dedicated signalling. The drawback of scenario 1 is that, NCD-SSB may have larger transmission periodicity, so it may cause longer handover delay. However, according to RAN4 requirements, if the target cell is known (e.g. based on RRM in source cell), the value of Tsearch is 0ms. So we think the drawback is negligible in case of measurement based handover. |
| MediaTek | Yes |  |
| Qualcomm | Yes | We think it is technically feasible. And it enables faster and more efficient handover, as compared with alternative options (e.g. handover to default initial BWP and then switch to RedCap-specific BWP to RACH). So we think it should be supported. |
| Apple | Yes | We agree. Also this implies that UE doesn’t have to do anything (per spec) with CD-SSB as part of handover in such a case. |
| NEC | Yes | We think it is feasible to support the scenario 1, while it would be good to raise potential delay as explained by ZTE and confirm with RAN4. |
| Samsung | Yes | - |
| Interdigital | Yes | It’s feasible and so Yes. |
| Huawei, HiSilicon | Yes, but | We are fine to compromise. But, we need to add the clarification that: The UE can also directly sync to the CD-SSB and perform RACH on that BWP, as supported in legacy. We don’t want to mandate UE to directly HO to NCD-SSB. That’s also to allow NW to choose HO to CD-SSB rather than NCD-SSB, if the delay is considered as an issue, as commented by ZTE. |
| Vivo | Yes | This is one of the intentions for NCD-SSB in connected mode. It could be helpful to avoid unnecessary RF retuning from UE side. Regarding the potential delay mentioned by ZTE, we assume it depends on which scenarios. There is no need to inform them in LS, as it is RAN4 work scope. |
| DENSO | Yes | Agree that it is feasible and more efficient than the other alternative, even with the delay due to longer SSB periodicity. |
| Intel | Yes | We do not see the problem to do this. |
| OPPO | Yes |  |

If answers “Yes” to Q7, please also provide your views on how to set the “smtc” field in handover command.

Q8: If answers “Yes” to Q7, do companies agree that in handover command, if the first active BWP is associated with NCD-SSB, the smtc field included reconfigurationWithSync is configured according to the NCD-SSB of target cell.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | “smtc” field is introduced to assist the UE to search target cell. To support scenario 1, we think the simplest way is to reuse this field and associate it to NCD-SSB when first active BWP is associated with NCD-SSB, it’s better to make sure the network and the UE have the same understanding, so IoT problem won’t occur. So if this can be agreed, we suggest to make following update to the field description. Smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR Pcell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR Pcell change, the smtc is based on the timing reference of (source) Pcell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If both this field and targetCellSMTC-SCG are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. For RedCap UE, if the first active DL BWP indicated in this RRC message is configured with nonCellDefiningSSB-r17, the UE assumes this field is configured according to the NCD-SSB indicated by nonCellDefiningSSB-r17, otherwise, the UE assumes this field is configured according to the CD-SSB of target cell. |
| MediaTek | Yes |  |
| Qualcomm | Yes |  |
| Apple | Yes |  |
| NEC | Yes |  |
| Samsung | Yes | - |
| Interdigital | Yes |  |
| Huawei, HiSilicon | Disagree | smtc is mainly to help UE to find the SSB fast. But, for NCD-SSB, NW already provide the frequency and periodicity in nonCellDefiningSSB. So, UE already have the full information to find the NCD-SSB. Introducing smtc for NCD-SSB is enhancement. Reusing the smtc field included reconfigurationWithSync, which is supposed to be for CD-SSB, is not acceptable. Introducing this will has impact to the case of “HO to CD-SSB”. It means we cannot support the below case in scenario 2, where NW may want to use smtc to configure UE to find the CD-SSB for DL sync, rather than NCD-SSB. “Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH  the specific Redcap BWP associated with NCD-SSB”. |
| vivo | Yes | We have agreed before: For connected mode operation if NCD-SSB is configured in a dedicated DL BWP whose paired UL BWP is configured with RACH-ConfigDedicated, RACH-ConfigCommon or BeamFailureRecovery Config, SSB in that RACH configuration (e.g., in CFRA-SSB-Resource IE or in PRACH-ResourceDedicatedBFR IE) refers to the NCD-SSB configured in that DL BWP. Hence, if the target cell’s specific Redcap BWP is associated with an NCD-SSB, SSB in rach-ConfigDedicated and smtc within reconfigurationWithSync should also refer to the NCD-SSB, then UE could perform RACH on NCD-SSB. |
| DENSO | Yes |  |
| Intel | Yes | Make sense to us. |
| OPPO | Yes |  |

For handover scenario 2:

Scenario 2: Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH

    - the specific Redcap BWP associated with NCD-SSB

    - the specific Redcap BWP without presence of NCD-SSB

Q9: From RAN2 perspective, do companies think Scenario 2 can be supported (i.e. the UE first sync to the CD-SSB and then autonomously switch to first active BWP to perform RACH)?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Vodafone | Yes | Please see comments to Q7 |
| ZTE | Prefer No | Scenario 2 is more complex than Scenario 1. If scenario 1 can be supported, there seems no additional benefit to support scenario 2. But we won’t object if majority companies would like to support scenario 2. |
| MediaTek | No | We do not see the point in introducing such a case for handover. This approach results in an unnecessarily complicated handover procedure with no real gain: - For the first listed case under Scenario 2, the RedCap specific BWP already has an NCD-SSB. There’s no need to complicate the handover procedure with an intermediate BWP switch, when the procedure can take place without needing such a BWP switch (by using the NCD-SSB as the sync reference) - For the second listed case, RAN4 will not define connected mode measurement requirements for operation in a BWP without an SSB (see R4-2206950). So we end up in a situation where there are no requirements for the UE operating in the first active BWP following handover! It’s a waste of our time and effort to optimize for this case that will only result in a paper design and will not be adopted in real deployments. |
| Qualcomm | No | We share the same view as MediaTek |
| Apple | No |  |
| NEC | No | We do not see the benefit of supporting this considering the additional complexity. |
| Samsung | No | Share the view with many others. |
| Interdigital | No | We don’t see any point to support the scenario. RedCap specific BWP is introduced for the case that RedCap UE doesn’t support the BW of the initial BWP. If RedCap UE can access the initial BWP, then no need to have the RedCap specific BWP from the beginning. |
| Huawei, HiSilicon | Yes | This question is not about the whether it is beneficial. RAN4 is just asking whether it is valid. RAN2 should not preclude the feasibility for NW configuration. This should be up to NW implementation. We don’t see any technical blocking issue from R2 to not support those case. |
| vivo | No | There is no need to support scenario2, which brings more complexity. |
| DENSO | No | Agree with MediaTek |
| Intel | ? | If the UE can sync/access to RedCap specific BWP, it is not clear to us why/when the scenario would happen |
| OPPO | No | Agree with MediaTek. |

## SIBs reception in BWP without CD-SSB

For SIB reception, the following proposals are provided in AI summary:

Proposal 7: During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell does not contain CD-SSB, UE acquires SI either from CSS for SIBs or via dedicated signaling only within the first active BWP. Proposal 8: RAN2 confirm that RedCap UEs in RRC Connected only need to support the following three options for acquiring SI update or ETWS/CMAS message in a dedicated DL BWP that does not contain CD-SSB: - From CSS for SIBs configured within this DL BWP; - Via dedicated signaling; - Switched by network (either DCI or RRC) to an initial DL BWP where SIBs are sent.

Companies are invited to show your views to above proposals:

Q10: Do companies agree with above Proposal 7?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | We understand there are only three ways for the UE to obtain SIB, as indicated in P8. But during handover procedure, it doesn’t make sense to switch the UE back to initial BWP to obtain SIB, if that is the case, the network should switch the UE directly to then initial BWP(or a BWP that contains CD-SSB and CORESET#0), then switch to other BWP for data transmission. |
| MediaTek | Yes, with clarification | With the clarification that ‘only within the first active BWP’ applies to both CSS for SIBs and to dedicated signalling. |
| Qualcomm | Yes | We agree with both the comments from MediaTek and ZTE. |
| Apple | Yes |  |
| NEC | Yes |  |
| Samsung | Yes | It should be the case. |
| Interdigital | Yes |  |
| Huawei, HiSilicon | Yes, but | We support the intention. As to the comment from MediaTek, we believe there is still the case “UE HO directly to CD-SSB and switch to NCD-SSB as its first BWP” (see scenario 2.), where UE may still have the chance to obtain SIB on CD-SSB. So, the originally wording is better. Also, we believe another intention is that NW should ensure UE have the SIB1. So, we suggest minor update: “UE is expected to acquire SI either from CSS for SIBs or via dedicated signaling only within the first active BWP.” |
| vivo | Yes |  |
| DENSO | Yes |  |
| Intel | Yes with clarification (see comment) | When UE acquires SI from CSS for SIBs, we assume it is the CSS for SIBs configured within this DL BWP, we wonder whether this is the correct/same understanding as the proponent? If yes, then we agree P7. |
| OPPO | Yes |  |

Q11: Do companies agree with above Proposal 8?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | We think the proposal is aligned with R15/R16 operations. |
| MediaTek | Yes | Proposal 8 is the generic case |
| Qualcomm | Yes | We think these three possible methods are the most likely options network can apply. Additional options would not offer more benefits but on the other hand would increase uncertainties for UE implementations. They therefore should be excluded. |
| Apple | Yes | Same as legacy operation. |
| NEC | Yes |  |
| Samsung | Yes | - |
| Interdigital | Yes |  |
| vivo | Yes |  |
| DENSO | Yes |  |
| Intel | Yes |  |
| OPPO | Yes |  |

## Paging reception in BWP without CD-SSB

For paging reception, the following proposals are provided in AI summary:

Proposal 9: Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP includes the CD-SSB. Proposal 10: RAN2 confirms for separate initial DL BWP does not contain CD-SSB and CORESET#0, the BWP will not be configured for paging no matter the RedCap UE is in RRC_IDLE/INACTIVE or RRC_CONNECTED state Proposal 11: Reply to RAN1 and explain there is no need to support paging connected RedCap UEs in case the separate initial DL BWP does not contain CD-SSB and CORESET#0.

Among the proposals, Proposal 10 and 11 are related to the below RAN1 agreements mentioned in RAN1 LS R2-2204422.

RAN1 agreements extracted from LS R2-2204422. - For BWP#0 configuration option 1, - For FR1, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - For FR2, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB. - Note: For BWP#0 configuration option 2, - For FR1, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB - For FR2, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB

In short, proponent suggests to clarify there is no need to define different paging principles for RRC_IDLE/INACTIVE and RRC_CONNECTED RedCap Ues, and the agreement made in RAN1 may cause problem when the RedCap UE in RRC_CONNECTED state wants to obtain SIB after receiving system modification indication or ETWS/CMAS notification in Paging message.

Companies are invited to show your views to above proposals:

Q12: Do companies agree with above Proposal 9?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes, but | P9 and P10 are related, for RRC_CONNECTED Ues, although RAN1 agreed the separate initial BWP that does not include CD-SSB can be configured for paging. We see no benefit to support this scenario, so we are aligned with P9. However, it seems not necessary to update the field description, it can be handled by network implementation, if needed, maybe it is better to capture the principle in stage2 spec. |
| MediaTek | Yes | It is important to capture this as we have already agreed to this but it’s missing in the specification (which therefore results in a specification that is the opposite of what we’ve agreed!) |
| Qualcomm | Yes |  |
| Apple | Yes | We think it’s better to have this captured in spec for clarity. There are explicit RAN1 agreements. |
| NEC | Yes | This clarification is useful. |
| Samsung | Yes | - |
| Interdigital | Yes |  |
| vivo | Yes | We prefer to add this field description for paging search space to make things clearer although this field could only present when RedCap initial BWP includes the CD-SSB. |
| DENSO | Yes |  |
| Intel | No | We do not see the problem to support RAN1 agreements. In contrast to Idle mode, in CONNECTED mode UE, the UE doesn’t need to perform any serving cell measurements before paging monitoring. In fact, the UE would not need CD-SSB for monitoring and reception of Paging PDCCH in connected mode. The behavior is same as that of regular PDCCH monitoring in connected mode, i.e. paging monitoring in CONNECTED mode is just to receive PDCCH and thus NCD-SSB can be used. Therefore, RAN1 agreed that there is no need to restrict configuration of paging monitoring in an active DL BWP that may not contain CD-SSB. |
| OPPO | Yes |  |

Q13: Do companies agree with above Proposal 10?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | As we discussed in R2-2205771, even for BWP operation 2, the paging search space is configured in BWP common configuration, which is applicable to RRC_IDLE/INACTIVE and RRC_CONNECTED UEs. So to only ask RRC_CONNECTED Ues to monitor paging requires RRC spec change (e.g. introduce additional paging search space configuration for connected UE). In addition, RRC_CONNECTED UE monitors paging to obtain SI change indication and ETWS/CMAS notification, but RAN1 indicates there is no SIB transmitted in that BWP. So only reading Paging is useless. So from RAN2 perspective, we do not see the benefit of supporting different paging configurations for Ues in different RRC states. |
| MediaTek | Yes |  |
| Qualcomm | Yes | From UE’s perspective, we support Proposal 10 for both RRC Idle/Inactive and RRC Connected. It simplifies UE implementation. |
| Apple | Yes |  |
| NEC | Yes |  |
| Samsung | Yes |  |
| Interdigital | Yes |  |
| vivo | Yes | From RAN2 current signaling configuration perspective, there is no other way to configure CSS for paging for the separate initial DL BWP, hence it can’t be configured for paging. |
| DENSO | Yes |  |
| OPPO | Yes |  |

Q14: Do companies agree with above Proposal 11?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | Same our comments to Q13. As we known, RAN1 didn’t discuss the role of paging for connected UEs, so if P10 can be agreed, we should inform RAN1 and ask them to revisit their agreement. |
| MediaTek | Yes |  |
| Qualcomm | Yes |  |
| Apple | Yes | Maybe we can add that paging in this case can be done via dedicated signaling (if there are questions in RAN1) |
| NEC | Yes |  |
| Samsung | Yes | - |
| Interdigital | Yes |  |
| Vivo | Yes |  |
| DENSO | Yes |  |
| OPPO | Yes |  |

## Number of SSBs

Regarding the number of SSBs, the following proposal is provided in AI summary:

Proposal 12: In Rel-17, one BWP can only contain up to one SSB (CD-SSB or NCD-SSB).

The intention is to align with RAN4 because RAN4 will only define RRM requirements for scenarios where the active BWP includes either a CD-SSB or an NCD-SSB, not both.

In RAN2#117e, RAN2 discussed the number of NCD-SSB and made the following agreement.

Agreements online:

- A RedCap UE may be configured with multiple NCD-SSBs provided that each BWP is configured with at most one SSB

However, rapporteur thinks some company may interpret “at most one SSB” as “at most one NCD-SSB”, that is why contribution is submitted to clarify one BWP can only contain one SSB (either CD-SSB or NCD-SSB). So companies are invited to show your opinion about P12.

Q15: Do companies agree with above Proposal 12?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| MediaTek | Yes |  |
| Qualcomm | Yes |  |
| Apple | Pls see comments | We think it’s simpler to limit to 1 NCD-SSB for the UE, and the same can be present in multiple BWPs (overlapped), while the current question does not bring this up, we think for R17 it would be simpler to go with this. This also helps with ref SSB switching topic (at the time of BWP switching), and we understand that this restricts the NW, but our view is practically it might not. |
| NEC | Yes |  |
| Samsung | Yes | - |
| Interdigital | Yes |  |
| Huawei, HiSilicon | Yes |  |
| vivo | Yes | If the BWP contains CD-SSB, there is no strong motivation to configure NCD-SSB for this BWP. |
| DENSO | Yes and agree with Apple | NW may transmit multiple NCD-SSB within a carrier bandwidth. On the other hand, from UE point of view, it would be enough to use one of NCD-SSB. One may consider that NW wishes to configure different NCD-SSB per BWP for a given RedCap UE, e.g. for dynamic load balancing by BWP switching. Nevertheless, it is not clear if such a dynamic mechanism is needed for the first release of RedCap. |
| Intel | Yes |  |

Note: The following proposal from AI summary will be discussed under Offline-109:

Proposal 13: When the type of measured SSB (e.g. CD-SSB, NCD-SSB) is changed, the UE shall set the value of SrxlevRefStationaryConnected to the current Srxlev value of the serving cell.

## Other

Besides the questions listed in previous section, if companies think there are other NCD-SSB related issues that need further discussion, please elaborate it in below table.

Q16: Any other NCD-SSB related issues?

| Company | Comments |
| --- | --- |

# Conclusion

To be provided.

# References

AI summary of AI 6.12.2

[1] R2-2206143 [Pre118-e][105][RedCap] Summary of AI 6.12.2.1 on NCD-SSB aspects	ZTE Corporation

Relevant incoming LS:

[2] R2-2204486 LS on NCD-SSB issues for RedCap UE (R4-2207104; contact: Ericsson)	RAN4 LS in	Rel-17	To:RAN2

[3] R2-2204422 LS on operation with and without SSB for RedCap UE (R1-2202886; contact: Ericsson)	RAN1	LS in	Rel-17	To:RAN2, RAN4

NCD-SSB related company contributions submitted to RAN2#118e:

[4] R2-2204544 Handover to BWP without CD-SSB   ZTE Corporation, Sanechips

[5] R2-2204547 Discussion on serving cell measurements on NCD-SSB   ZTE Corporation, Sanechips

[6] R2-2204812 Discussion on NCD-SSB for RedCap UEs	vivo, Guangdong Genius

[7] R2-2205038 Discussion on NCD-SSB aspects for RedCap UE   Huawei, HiSilicon

[8] R2-2205285 [J002] Clarification on reference value in connected RRM relaxation critrion	Sharp

[9] R2-2205522 Aspects related to the use of NCD-SSB   MediaTek Inc.

[10] R2-2205636 Discussion on NCD-SSB handling at handover	Apple discussion

[11] R2-2205771 About paging monitoring in BWP#0 without CD-SSB   ZTE Corporation, Sanechips Late

[12] R2-2206019 [DRAFT] Reply LS on NCD-SSB issues for RedCap UE   Ericsson   LS out   To: RAN4

[13] R2-2206032 Further discussion on SI acquisition in RedCap-specific BWP    Qualcomm Incorporated

[14] R2-2206033 Measurement object configuration with NCD-SSB   Qualcomm Incorporated	discussion