3GPP TSG-RAN WG2 Meeting #118-e	draft R2-2206259

Electronic meeting, May 9th – May 20th, 2022

Agenda item:		6.11.2.6

Source:				CATT

Title:					[AT118-e][638][POS] Tx TEG and LOS/NLOS aspects (CATT)

Document for:		Discussion and Agreement

# 1	Introduction

This is the report of following offline discussion:

[AT118-e][638][POS] Tx TEG and LOS/NLOS aspects (CATT)

Scope: Discuss P1a-P1e and P3a/P3b of R2-2206333.

Intended outcome: Report to CB session in R2-2206259

Deadline:  Tuesday 2022-05-17 1800 UTC

The discussion will continue to discuss the remaining proposals P1a-P1e and P3a/P3b of R2-2206333:

R2-2206333	[Pre118-e][607][POS] Summary of AI 6.11.2.6 on accuracy (CATT)	CATT	discussion	Rel-17

- TxTEG report mechanism in RRC aspect: P1a/ P1b;

- TxTEG report of asn.1 issues in RRC and LPP: P1c/P1d;

- Failure report mechanism of Tx/Rx TEG in RRC and LPP: P1e;

- LOS/NLOS related enhancement: P3a/P3b.

# 2	Contact Information

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Contact: Name (E-mail) |
| --- | --- |
| Huawei, HiSilicon | yinghaoguo@huawei.com |
| Apple | Sasha Sirotkin <ssirotkin@apple.com> |
| InterDigital | jaya.rao@interdigital.com, fumihiro.hasegawa@interdigital.com |

# 3	Discussion

## 3.1 TxTEG report mechanism in RRC aspect

Event-triggered report or periodic report were discussed in[Pre117-e][611][POS] Open issues on positioning accuracy enhancements (CATT) but the periodic report gets more support. LMF actually may update the periodicRreporting Interval if there is no TxTEG change during the reporting, so there is no big signalling issue observed in the existing periodic reporting.

However two companies still suggested supporting event-triggered report in R2-2205654 and R2-2205730. Apple proposed to remove the periodic UE Tx TEG association reporting and to introduce change-triggered reporting instead. The proposal 1a in R2-2206333 says:

Proposal 1a: RAN2 to agree configuring event triggered reporting for UL-TDOA to enable reporting of the association between UE TxTEG ID and SRSp resources when a change in the association is identified.

| Apple R2-2205654 | Observation 1: periodic UE Tx TEG association reporting signalling design is extremely inefficient. Observation 1: in their LS [1], RAN1 have confirmed that there is no need for periodic UE Tx TEG association reporting. Proposal 1: to remove the periodic UE Tx TEG association reporting and to introduce change-triggered reporting instead. Proposal 2: to remove timestamp from the UE Tx TEG association report. |
| --- | --- |
| InterDigital R2-2205730 | Observation 1: By allowing the UE to report the association between UE Tx TEG and SRSp resources only when an event associated with the change of Tx TEG association is identified, signalling overhead can be reduced significantly Proposal 1: Support configuring event triggered reporting for UL-TDOA to enable reporting of the association between UE Tx TEG ID and SRSp resources when a change in the association is identified Proposal 2: Support configuring of reportAmount of 1 and infinity for event triggered reporting of UE Tx TEG association Observation 2: For event-triggered reporting, it is possible that the UE may report the Tx TEG association too frequently (e.g. due to frequent movement/changes at UE), which may result in difficulty at network for controlling the resources for reporting Proposal 3: Support configuring reportInterval for event-triggered reporting of UE Tx TEG association. Proposal 4: The configurable reportInterval values for event-triggered reporting are reused from periodic reporting (e.g. ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240) |

#### Q1: Do companies agree that configuring event triggered reporting for UL-TDOA to enable reporting of the association between UE Tx TEG ID and SRSp resources when a change in the association is identified, and remove periodic reporting in the existing RRC protocol? Please provide also a brief justification for your answer.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | periodic reporting with timestamp is enough or oneshot reporting. if event-triggered is indeed needed, need R1 to decide |
| Qualcomm | See comment | This was discussed at previous meetings, and in general we agree that event triggered reporting would have been the correct approach. However, given that we made the periodic agreements at previous meetings, it may be too late to change now. |
| Apple | Yes | We would like to stress that it is not just “a company proposal”, but it is based on the new information we received from RAN1 in R2-2204420, which clarified that “RAN1’s decision to support periodicity reporting of UE Tx TEG association for the SRS resources for positioning was made mainly based on the consideration of the signalling simplicity. In RAN1’s view, further signalling optimization is up to RAN2.” In other words, the decision to support periodic signalling was made based on the wrong assumption that RAN1 had technical motivation to ask for it, which as we can now clearly see was not the case. Therefore, the decision to support periodic (and not event triggered) signalling was a mistake which should be corrected. |
| InterDigital | Yes | In R2-2204420, it states that further signalling optimization is up to RAN2. From our perspective, at least reportAmount of 1 should be event triggered reporting of UE Tx TEG association. Our proposal is also motivated by the inefficiency associated with periodic reporting, as we do not see the need for the UE to report the association between UE Tx TEG ID and SRSp resources periodically when a change is not identified in the association. In this regard, we believe a correction for event triggered reporting is justified. |

Summary

Furthermore, InterDigital observed it is possible that the UE may report the Tx TEG association too frequently (e.g. due to frequent movement/changes at UE), which may result in difficulty at network for controlling the resources for reporting. They proposed the configurable reportInterval values for event-triggered reporting are reused from periodic reporting (e.g. ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240).

#### Q2: Do companies agree to update the asn.1 of UE-TxTEG-RequestUL-TDOA-Config-r17 as event triggered reporting in RRC below? Please provide also a brief justification for your answer.

EventTriggerConfig-r17::=   SEQUENCE {

reportInterval-r7       ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240}

reportAmount-r17        ENUMERATED {1, infinity},

...

}

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | In terms of future extensibility it is better. but needs to have restriction in reportInterval that it is only present when the value of report Amount is set to infinity |
| Qualcomm |  | See comment in our reply to Question 1. |
| Apple | See comments | We acknowledge the issue raised by InterDigital, but the proper way to solve it is to define event triggered signalling, not “play” with periodicities (which are arbitrary anyway). |
| InterDigital | Yes | The reportAmount value set to infinity is intended to handle the issue where the gNB is unable to predict the changes in UE Tx TEG and SRSp resource association at UE. In this case, the gNB can configure the reporting interval to control the number of reporting occasions while ensuring accurate association info available at gNB. Since the reportInterval values were previously discussed for periodic reporting with the similar reasoning (e.g. to control number of reporting occasions), we think they are applicable for event triggered reporting. |

Summary

## 3.2 TxTEG report of asn.1 issues in RRC and LPP

There are few issues of asn.1 in RRC and LPP are observed in R2-2204706, R2-2204707 and R2-2204708. Most of these issues are captured in the RRC update in R2-2205859 and R2-2205829:

- R2-2205859	Correction based upon Positioning RILs	Ericsson	CR	Rel-17	38.331	17.0.0	3121	-	F	NR_pos_enh-Core	Late

- R2-2205829	LPP Updates	Qualcomm Incorporated	draftCR	Rel-17	37.355	17.0.0	F	NR_pos_enh-Core

Issue #1:

The maximum number reported UE TxTEG ID which is defined in UE-TxTEG-AssociationList-r17 in RRC means that how many changes of TxTEG-ID will be reported in one RRC message.

- The maxi number reported UE TxTEG ID in the existing LPP is 64, but it is 8 in the existing RRC.

- The maximum numbers of TxTEG-IDs in one change is 8 according to RAN1 LS.

It seems that the volume of reported TxTEG-IDs in one RRC message is not proper if it is only 8.

Existing RRC in R2-2205859 is 8:

maxNrOfTEG-ID-r17             INTEGER ::= 8 -- Maximum number of UE Tx Timing Error Group ID

Existing LPP in R2-2205829 is 64:

maxTxTEG-Sets-r17						INTEGER ::= 64	-- FFS 8 TxTEGs and max 8 time stamps

So CATT propose to update the volume of UE TxTEG IDs report in RRC as 64 which is aligned with LPP, because 64 is well considered based on the possible times of the change and the number of TxTEG IDs in one change.

UE-TxTEG-AssociationList-r17 ::= SEQUENCE (SIZE (1.. maxNrOfTEG-ID-r17)) OF UE-TxTEG-Association-r17

maxNrOfTEG-ID-r17 						INTEGER ::= 64    	-- Maximum reported number of UE Tx Timing Error Group ID

Considering the maxNrOfTEG-ID-r17 in LPP is still FFS, companies may discuss the maximum numbers of reported UE TxTEG IDs in both LPP and RRC here. Since RAN4 doesn't conclude the changes of TxTEG mentioned in R2-2202165, it is necessary to send RAN2 agreement on the volume of changes of TxTEG-IDs in one report to RAN1 and RAN4 for confirming.

- R2-2202165	Reply LS on reporting of the Tx TEG association information (R4-2202685; contact: Huawei)	RAN4	LS in	Rel-17	To:RAN1, RAN2	Cc:RAN3

The UE Tx TEG association between UE Tx TEG IDs and SRS resources for positioning is up to UE implementation, so it is not necessary nor practical to define the condition when the TEG association is changed.

#### Q3: Do companies agree that the maximum numbers of reported TxTEG-IDs in one RRC message and maxTxTEG-Sets-r17 in LPP message is 64? If No, please provide your preferred value of the max numbers. Please provide also a brief justification for your answer.

| Company | Yes/No for RRC | Yes/No for LPP | Comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Yes | Yes | OK with 8 timestamps. but the name is a bit ambiguous. prefer to change the name to something like maxTEG-ReportsPerPeriod |
| Qualcomm | Not sure | Yes | For RRC, my understanding from previous discussions is that the UE reports the last SRS/TEG association when the (periodic) report is triggered. SRS/TEG changes between periodic reports seem not supported with the current RRC signalling structure. |
| Apple | No | Yes | If event triggered reporting is agreed, the number of “changes” reported in one message would be small and therefore 64 is not needed. |
| InterDigital | No | Yes | Same understanding with Apple. Changing the max number of SRSp-TxTEG association reports per reporting occasion to 64 in RRC is not needed. |

#### Q4: Do companies agree to send the agreement to RAN1 and RAN4 for conforming if there is? Please provide also a brief justification for your answer.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | depends | If we can not resolve the issues in RAN2, asking RAN1/4 for guidance would be required. TEG turned out to be rather confusing, and the RAN1 input is often not that clear as it could be. However, we need then consider that a response from RAN1/4 may not be received before the next meeting. |

Summary

Issue #2: the usage of nr-SRS-TxTEG-Set-r17

CATT believes that the TxTEG-IDs from UE to LMF are not only for UE Rx-Tx measurement in LMF, but also for gNB Rx-Tx measurement in LMF. So this IE nr-SRS-TxTEG-Set-r17 should not be reported only within Cond Case2-3. The IE nr-SRS-TxTEG-Set-r17 is something like the UE TxTEG report via RRC in UL-TDOA.

So CATT proposed to delete the condition of TxTEG as below. Similarly the time stamp in NR-SRS-TxTEG-Element-r17 is needed here to help LMF figure out which UE-TxTEG-ID is selected for the gNB Rx-Tx measurement in LMF. However the LPP rapporteur has difference understanding, i.e. for case-1, no Tx TEG is needed. In case of doubt, RAN2 may need to ask RAN1 on the usage of nr-SRS-TxTEG-Set-r17 in Multi-RTT report.

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

]]

#### Q5: Do companies agree to delete the condition of TxTEG report in Multi-RTT? Do company agree to send the doubt to RAN1 that if TxTEG IDs are still required when TxTEG is not required in case 2-3 in NR-UE-RxTx-TEG-Info-r17? Please provide also a brief justification for your answer.

| Company | Yes/No (to delete the condition) | Yes/No (LS to RAN1) | Comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | No | No | This condition is just for restricting when the field can be configured and when it cannot be configured, i.e., it is only needed when case2-3 report is sent to the LMF. it is not a restriction on how it should be used. |
| Qualcomm | No | See Q4 | Tx TEG and RxTx TEG are different concepts. There is no Tx TEG involved in RxTx TEG (in my understanding, this is the advantage of the definition of a RxTx TEG) and the RAN4 LS cited before Question 3 mentions only Tx TEG association change. |
| Apple | No | No | Agree with Huawei |
| InterDigital | No | No |  |

Summary

The updated LPP in R2-2205829 is FFS on CHOICE in NR-UE-RxTx-TEG-Info-r17 as below:

NR-UE-RxTx-TEG-Info-r17 ::= CHOICE { -- FFS if the CHOICE structure is needed

case1-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17)

},

case2-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

case3-r17				SEQUENCE {

nr-UE-Rx-TEG-ID-r17		INTEGER (0..maxNumOfRxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

...

} -- FFS the nr-UE-Tx-TEG-ID-r17 in case2 and case3 (pending RAN1)

CATT proposed to change the structure of NR-UE-RxTx-TEG-Info-r17 from choice to sequence which is more flexible in the LPP as below:

NR-UE-RxTx-TEG-Info-r17 ::= SEQUENCE {

nr-UE-RxTx-TEG-ID-r17		INTEGER (0..maxNumOfRxTxTEGs-1-r17)				OPTIONAL,

nr-UE-Tx-TEG-Index-r17		INTEGER (0..maxNumOfTxTEGs-1-r17) 				OPTIONAL,

nr-UE-Rx-TEG-ID-r17			INTEGER (0..maxNumOfRxTEGs-1-r17) 				OPTIONAL,				...

}

#### Q6: Do companies agree to modify the IE NR-UE-RxTx-TEG-Info-r17 from CHOICE to SEQUENCE? Please provide also a brief justification for your answer.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | CHOICE is clear from the readability point of view. from signalling overhead point of view, it also seems to be more or less the same. |
| Qualcomm | No | The SEQUENCE definition would allow all possible combinations between the 3 fields, which however, seems not the RAN1 intention. I.e., only the current 3 CHOICEs are supported by RAN1. The proposal would also make the corresponding request and capability definition more difficult. I.e., the nr-UE-RxTxTEG-Request and nr-UE-RxTx-TEG-ID-ReportingSupport can simply refer to the 3 choices. With the removal of the CHOICE, more field description would be needed on which combinations are allowed/supported/requested (without any functional difference). However, the additional SEQUENCE for case-1 above is not needed. |
| Apple | No | CHOICE is aligned with the agreements |
| InterDigital | No |  |

Summary

## 3.3 Failure report mechanism Tx/Rx TEG in RRC and LPP

Ericsson introduces the failure report mechanism on the corresponding Rx/Tx TEG association in LPP and RRC.

Currently there is no provision for failure handling for the above reports, if UE is unable to report the TEG association how should that be handled. UE should be able to handle the failure; i.e provide failure report on the TEG association report and continue transmitting UL-SRS.

| Ericsson R2-2205806 | Proposal 1 Failure to provide (periodic) Rx/Tx TEG association does not result in termination of UL SRS Tx or DL-PRS Measurements Proposal 2 UE provides the failure report on the corresponding Rx/Tx TEG association and continue with the positioning procedure Proposal 3 Below TP on ASN.1 for RRC and LPP is agreed for TEG failure Reporting failureIndication-r17 ENUMERATED {state-transition, lowpowerstate, unknown, spare1} OPTIONAL, |
| --- | --- |

So Ericsson proposed to support the failure report in both LPP and RRC as below.

- LPP

NR-SRS-TxTEG-Element-r17 ::= SEQUENCE {

nr-TimeStamp-r17				NR-TimeStamp-r16						OPTIONAL,	-- Need OP

nr-UE-Tx-TEG-ID-r17				INTEGER (0..maxNumOfTxTEGs-1-r17),

srs-PosResourceList-r17			SEQUENCE (SIZE (1..maxNumOfSRS-PosResourceSets-r17)) OF

SRS-PosResources-r17,

...,

[[

tegTxReportingfailureIndication-r17	ENUMERATED {state-transition, lowpowerstate,                 											unknown, spare1}				OPTIONAL

]]

NR-UE-RxTx-TEG-Info-r17 ::= CHOICE {

case1-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17)

},

case2-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

case3-r17				SEQUENCE {

nr-UE-Rx-TEG-ID-r17		INTEGER (0..maxNumOfRxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

...,

failureCase-r17					ENUMERATED {state-transition, lowpowerstate,                 											unknown, spare1}		OPTIONAL,

}

- RRC

UEPositioningAssistanceInfo-IEs-r17 ::= SEQUENCE {

ue-TxTEG-AssociationList-r17            UE-TxTEG-AssociationList-r17        OPTIONAL,

failureIndication-r17						ENUMERATED {state-transition, lowpowerstate,                 											unknown, spare1}					OPTIONAL,

lateNonCriticalExtension                OCTET STRING                        OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                         OPTIONAL

}

#### Q7: Do companies agree the TP of Failure report mechanism of Tx/Rx TEG in RRC and LPP? Please provide also a brief justification for your answer.

| Company | Yes/No (TP on RRC) | Yes/No (TP on LPP) | Comments |
| --- | --- | --- | --- |
| Huawei, HiSIlicon | No | No | Justification is not clear from R2’s perspective. Why the UE may fail to provide TEG association at certain times? This needs to be discussed in R1 if needed |
| Qualcomm | No | No | If TEG is requested, but absent in the report, it obviously means that the TEG is not available. This is not different compared to any other "failure handling" in e.g. LPP (e.g., RSRP etc.). In addition, the proposed failure cases seem all unrelated to TEG. |
| Apple | No | No | Agree with QC |
| InterDigital | No | No | Same understanding with QC |

Summary

## 3.4 LOS/NLOS related enhancement

Two companies believe that the current field cannot differentiate whether the report is per TRP or per resource for the main measurement. There are two candidate options on the update:

- #Option 1: Huawei suggest updating to indicate whether the LOS-NLOS report is per TRP or per resource in R2-2205004.

[[

nr-LOS-NLOS-Indicator-r17		CHOICE {

perTRP						LOS-NLOS-Indicator-r17,

perResource					LOS-NLOS-Indicator-r17

}																				OPTIONAL,

- #Option 2: ZTE propose to modify the indicator as below:

#### –	LOS-NLOS-Indicator

The IE LOS-NLOS-Indicator provides information on the likelihood of a Line-of-Sight (LOS) propagation path from the source to the receiver.

LOS-NLOS-Indicator-r17 ::= CHOICE{

per-trp-r17			Indicator-r17,

per-resource-r17	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

LOS-NLOS-Indicator-PerResource-r17,

...

},

LOS-NLOS-Indicator-PerResource-r17 ::=

SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF

Indicator-r17

Indicator-r17			CHOICE {

soft-r17				INTEGER (0..10),

hard-r17				BOOLEAN,

...

},

-- ASN1STOP

LOS-NLOS-Indicator field descriptions

LOS-NLOS-Indicator This field indicates whether the LOS or NLOS indicator is provided per TRP or per PRS resource.

#### Q8-1: Do companies agree to modify the nr-LOS-NLOS-Indicator-r17 to choice of per TRP or per resource? Please provide also a brief justification for your answer.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | the current LPP CR already handles this. |
| Qualcomm | Yes | Seems according to RAN1 parameter. The CHOICE is simpler, otherwise significant additional field description would be required. |
| Apple | Yes |  |
| InterDigital | Yes |  |

#### Q8-2: If yes, which TP do you prefer? Please provide also a brief justification for your answer.

| Company | Option 1/2 | Comments |
| --- | --- | --- |
| Qualcomm | Option 1 | Option 2 is unclear. Seems only applicable to the NR-DL-PRS-ExpectedLOS-NLOS-Assistance, where the CHOICE is already supported anyhow. |
| Apple | Option 1 |  |
| InterDigital | Option 1 |  |

Summary

Furthermore, Huawei proposed several corrections to LOS-NLOS indication in R2-2205004 according the RIL:

[H027] UE should be allowed to choose its own reporting mode of per TRP or pre resource report and does not have to follow the LMF’s request. This should be also clarified in the field descritpion or IE description.

[H029]Change the name to nr-LOS-NLOS-IndicatorPerResource to differentiate it with the per TRP/perResource Indication

[H030] Is it possible that the UE chooses a NLOS reference TRP? If that is the case, all the LOS-NLOS indicator will be NLOS and signaling can be optimized. If not, this indication is only for the neighbour TRP indicated by the PRS ID.

Correction[R2-2205004] #2 / Add in the field description that in spite of the request from the network in RLI, the UE can choose its LOS-NLOS reporting by TRP or by resource.

nr-los-nlos-Indicator

This field specifies the target device's best estimate of the LOS or NLOS of the TOA measurement for the TRP or resource. Note, the TOA measurement refers to the TOA of this neighbour TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. In spite of the request from the network in requestLocationInformation, the UE can choose its resourcetype and ganularity for LOS-NLOS reporting.

Correction [R2-2205004] #3/ Change the name to nr-LOS-NLOS-IndicatorPerResource to differentiate it with the per TRP/perResource Indication

nr-los-nlos-IndicatorPerResource-r17

LOS-NLOS-Indicator-r17							OPTIONAL,

nr-LOS-NLOS-IndicatorPerResource

This field specifies the target device's best estimate of the LOS or NLOS of the TOA measurement for the resource. The field is only present when the field nr-LOS-NLOS-Indicator adopts the field perResource.

Correction[R2-2205004] #4/ Remove the reference TRP in the field description. If clarification is needed from R1, send an LS.

nr-los-nlos-Indicator

This field specifies the target device's best estimate of the LOS or NLOS of the TOA measurement for the TRP or resource. Note, the TOA measurement refers to the TOA of this neighbour TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff.

However it seems that correction #4 is not essential because the agreement in RAN1 parameter [R1-2202759] says:

•	For DL-TDOA one LoS/NLoS indicator can be associated with each RSTD measurement performed with a target TRP and one LoS/NLoS indicator is associated with the RSTD measurement performed with a reference TRP

•	For DL-TDOA one LoS/NLoS indicator can be associated with each target TRP and one LoS/NLoS indicator can be associated with the reference TRP in the measurement report

So companies will review these corrections #2, #3, #4 in R2-2205004 one by one.

#### Q9: Which correction #2/#3/#4/None do you agree? Please provide also a brief justification for your answer.

| Company | Modification #2/ #3/ #4/ None | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | OK, but these issues seem to have already been captured by the LPP CR. |
| Qualcomm | #2 / #3 | #2 depends on the current UE situation/location and the request may not always be possible to fulfil. #3 needs to go together with Question 8/Option1, otherwise it will be inconsistent. #4 seems not correct, since the indicator is per TOA measurement, not per RSTD. |
| Apple | 2 and 3 |  |
| InterDigital | #2, #3 |  |

Summary

# 4	Conclusion

TBD

# 5	Reference

- R2-2206333	[Pre118-e][607][POS] Summary of AI 6.11.2.6 on accuracy (CATT)	CATT	discussion	Rel-17

- R2-2205654	On periodic UE Tx TEG reporting	Apple	discussion	Rel-17	NR_pos_enh-Core

- R2-2205730	Discussion on UE TX TEG association reporting	InterDigital, Inc.	discussion	Rel-17

- R2-2205859	Correction based upon Positioning RILs	Ericsson	CR	Rel-17	38.331	17.0.0	3121	-	F	NR_pos_enh-Core	Late

- R2-2205829	LPP Updates	Qualcomm Incorporated	draftCR	Rel-17	37.355	17.0.0	F	NR_pos_enh-Core

- R2-2204706	Discussion on the left issues on UE TxTEG report in RRC and LPP protocols	CATT	discussion

- R2-2204707	[C243] Correction on the UE TxTEG report in TS 38.331	CATT	CR	Rel-17	38.331	17.0.0	2994	-	F	NR_pos_enh-Core

- R2-2204708	[C013][C014][C015][C016][C017]Corrections on the UE TxTEG report in TS 37.355	CATT	CR	Rel-17	37.355	17.0.0	0335	-	F	NR_pos_enh-Core

- R2-2202165	Reply LS on reporting of the Tx TEG association information (R4-2202685; contact: Huawei)	RAN4	LS in	Rel-17	To:RAN1, RAN2	Cc:RAN3

- R2-2205004	[H026][H027][H029][H030] Correction to LOS-NLOS indication	Huawei, HiSilicon	CR	Rel-17	37.355	17.0.0	0339	-	F	NR_pos_enh-Core

- R2-2205806	Remaining Issues on TEG reporting; failure Handling	Ericsson	discussion	Rel-17