3GPP TSG RAN WG2#118-e	R2-2204496

e-Meeting, 9th - 20th May, 2022

3GPP TSG RAN WG2#115 -e	R3-222861

Online meeting, February 21st – March 3rd, 2022

# Title:	Reply LS on UE location during initial access in NTN

# Response to:	R3-221357 / R2-2201881

# Release:	Release 17

Source:	RAN3

To:	RAN2

Cc:	CT1, SA3, SA2

Contact Person:

Name:	Nicolas Chuberre

E-mail Address:	nicolas.chuberre@thalesaleniaspace.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 thanks RAN2 for its Liaison in which the following questions was submitted:

Is it acceptable that no UE location information is reported at the NG-RAN in a NTN network during initial access ?

RAN3’s response can be found below:

Without knowledge of the UE location during the initial access, the gNB may not be able to select the correct AMF.

If this happens, the incorrect AMF de-registers the UE, asks the UE to re-register and may inform NG-RAN with an appropriate NGAP cause value in the NGAP UE CONTEXT RELEASE COMMAND message. On subsequent network access attempt by the UE, the NG-RAN may be able to select the right AMF based on the information from the UE. This translates into a risk of extended UE registration (or connection set-up), but only in extreme cases of large radio cell covering more than 1 country, and only at the transition to RRC CONNECTED state or after significant UE movement.

In addition, the initial mapped cell ID reported over NGAP may not be able to provide the level of granularity that has been requested by SA groups, but it has previously been clarified that this is acceptable at initial access.

There are no significant impacts in RAN3 specifications resulting from this change.

Overall RAN3 confirms that the above is acceptable from RAN3 point of view.

2. Actions:

To RAN2

ACTION:	RAN3 kindly asks RAN2 to take into account the response above.

3. Date of Next RAN2 Meetings:

TSG-RAN WG3#116- e	May 16th – 27th, 2022	Online meeting