3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2205254

# Online Meeting, 9th – 20th May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.340 | CR | 0025 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the handling of unknown, unforeseen, and erroneous protocol data for header rewriting case in TS 38.340 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in the current specification TS 38.340, clause 5.5., the BAP Data PDU will be handled as unknown, unforeseen, and erroneous protocol data if it “contains a BAP address which is not included in the configured BH Routing Configuration and is not the BAP address of this node is received”. In R17, there may be some BAP data with the BAP address in header not included in the BH Routing Configuration, due to the header rewriting: Case 1: At the boundary node, the non-F1-terminating topology data just received may contains BAP address not included in the routing configuration, but will contains the BAP address included in the routing configuration after header rewriting. So, we need to clarify that as long as the data after header rewriting contains the BAP address included in the routing configuration, it should not be discarded. Case 2: At the IAB-node configured with inter-donor-DU re-routing, after the routing configuration is updated by F1AP after RLF recovery, there may be still some old data received from the descendant node, which contains the old BAP address not included in the new routing configuration. But, those data can be re-routed with header rewriting, which should not be discarded. So, we need to clarify that as long as the data after header rewriting contains the BAP address included in the routing configuration, it should not be discarded. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add ”(after the BAP header rewriting operation in accordance with clause 5.2.1.5 or 5.2.1.3, if applied)” on the configured BH Routing Configuration in handling of unknown, unforeseen, and erroneous protocol data in clause 5.5. Impact analysis Impacted functionality: BAP handling of unknown, unforeseen, and erroneous protocol data, BAP header rewriting for re-routing Inter-operability: - If the IAB-MT is implemented according to this CR but the network is not, there is no inter-operability issue forseen. - If the network is implemented according to this CR but the IAB-MT is not, there is no inter-operability issue forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some BAP Data PDU may be mishandled as unknown, unforeseen, and erroneous protocol data and then discarded, which was supposed to be re-routed based on header rewriting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

## 5.5	Handling of unknown, unforeseen, and erroneous protocol data

When a BAP Data PDU (after the BAP header rewriting operation in accordance with clause 5.2.1.5 or 5.2.1.3, if applied) that contains a BAP address which is not included in the configured BH Routing Configuration and is not the BAP address of this node is received; or when a BAP Control PDU that contains reserved or invalid values is received the BAP entity shall:

-	discard the received BAP PDU.

End of Change