3GPP TSG-RAN2 Meeting #119	R2-2208815

# E-meeting, August 17-29, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.985 | CR |  | rev |  | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to introduce sidelink relay and discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SL relay and discovery operation should be added to TR 37.985. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add SL relay and discovery operation to TR 37.985 with a new section |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SL relay and discovery operation in TR 37.985 is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2,3, New 6.x, New 6.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 36.885: "Study on LTE-based V2X Services".

[3]	ETSI EN 302 637-2: "Specification of Cooperative Awareness Basic Service".

[4]	SAE J2735: "Dedicated Short Range Communications (DSRC) Message Set Dictionary".

[5]	ETSI EN 302 637-3 "Specifications of Decentralized Environmental Notification Basic Service".

[6]	3GPP TS 22.185: "Service requirements for V2X services".

[7]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios".

[8]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[9]	3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[10]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC)".

[11]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[12]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Measurements".

[13]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[14]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC)".

[15]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP)".

[16]	3GPP TS 38.211: "NR; Physical channels and modulation".

[17]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[18]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[19]	3GPP TS  37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[20]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[21]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[22]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[23]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Where the same abbreviation is used for LTE V2X and NR V2X, which is meant can be derived from the clause within which it appears, unless otherwise stated.

5GC	Fifth Generation core network

AGC	Automatic gain control

AMBR	Aggregate maximum bit rate

BSM	Basic safety message

BWP	Bandwidth part

CA	Carrier aggregation

CAM	Cooperative awareness message

CBR	Channel busy ratio

CPS	Contiguous partial sensing

CR	Channel occupancy ratio

DENM	Decentralized environmental notification message

DMRS	Demodulation reference signal

DRX	Discontinuous reception

EPC	Evolved packet core

L2					Layer-2

L3		Layer-3

MBSFN	Multicast-broadcast single-frequency network

MNO	Mobile network operator

PBPS	Periodic-based partial sensing

PPPP	ProSe per-packet priority

PPPR	ProSe per-packet reliability

PSBCH	Physical sidelink broadcast channel

PSCCH	Physical sidelink control channel

PSSCH	Physical sidelink shared channel

PSSS, S-PSS	Primary sidelink synchronization signal (LTE), sidelink primary synchronization signal (NR)

PT-RS	Phase-tracking reference signal

P-UE	Pedestrian UE

RSU	Roadside unit

SA	Scheduling assignment

SCI	Sidelink control information

SC-PTM	Single-cell point-to-multipoint

SL-BCH	Sidelink broadcast channel

SLSS	Sidelink synchronization signal

SRAP	Sidelink Relay Adaptation Protocol

S-RSSI	Sidelink received signal strength indicator

S-SSB	Sidelink synchronization signal block

SSSS, S-SSS	Secondary sidelink synchronization signal (LTE), sidelink secondary synchronization signal (NR)

U2N 	UE-to-Network

V2I	Vehicle-to-infrastructure

V2P	Vehicle-to-pedestrian

V2V	Vehicle-to-vehicle

V2X	Vehicle-to-everything

Next Change

## 6.x Sidelink Relay

Sidelink-based UE-to-Network relaying functionality is specified in order to support network coverage extension and power efficiency. Both L2 and L3 U2N Relay architectures are supported.

For L3 U2N Relay, the coresponding architecture is transparent to the serving RAN of the L3 U2N Relay UE, except for sidelink resources control. The detailed architecture and procedures for L3 U2N Relay can be found in TS 23.304 [23].

For L2 U2N Relay, the L2 U2N Remote UE establishes the RRC connection with gNB via a L2 U2N Relay UE. A single unicast link is established between one L2 U2N Relay UE and one L2 U2N Remote UE. The protocol stacks for user plane and control plane of L2 U2N Relay architecture are shown in Figure 6.x-1 and Figure 6.x-2, which are in section 16.12.2.1in TS 38.300 [20]. A new sublayer (i.e., SRAP sublayer) is placed above the RLC sublayer for both CP and UP at both PC5 interface and Uu interface for bearer mapping purpose. The SRAP header includes the Uu RB ID of L2 U2N Remote UE and a local Remote UE ID. The SRAP sublayer of L2 U2N Relay UE performs bearer mapping according to the local Remote UE ID and Uu RB ID included in both PC5 SRAP header and Uu SRAP header.

Figure 6.x-1: User plane protocol stack for L2 UE-to-Network Relay

Figure 6.x-2: Control plane protocol stack for L2 UE-to-Network Relay

For both L3 U2N relay and L2 U2N relay, the U2N Remote UE needs to perform the U2N relay discovery and relay (re)selection. The relay discovery can refer to clause 6.y. For the relay (re)selection, a U2N Relay UE is considered suitable by a U2N Remote UE in terms of radio criteria if the PC5 link quality measured by U2N Remote UE towards the U2N Relay UE exceeds (pre)configured threshold. The U2N Remote UE searches for suitable U2N Relay UE candidates that meet all AS layer and higher layer criteria. If there are multiple such suitable U2N Relay UEs, it is up to U2N Remote UE implementation to choose one U2N Relay UE among them.

Once U2N Remote UE selects a suitable U2N Relay UE, it establishes the PC5 connection with the  U2N Relay UE. For L2 U2N Remote UE, it needs to further establish its own PDU sessions and DRBs with the network before user plane data transmission. The RRC connection management for L2 U2N Remote UE is specified v.

The in-coverage L2 U2N Remote UE is allowed to acquire any necessary SIB(s) over Uu interface irrespective of its PC5 connection to L2 U2N Relay UE. The L2 U2N Remote UE can also receive the system information from the L2 U2N Relay UE after PC5 connection establishment with L2 U2N Relay UE.

When both L2 U2N Relay UE and L2 U2N Remote UE are in RRC IDLE or RRC INACTIVE, the L2 U2N Relay UE monitors paging occasions of its connected U2N Remote UE(s). When L2 U2N Relay UE is in RRC CONNECTED and L2 U2N Remote UE(s) is in RRC_IDLE or RRC_INACTIVE, the L2 U2N Relay UE may monitor the POs of its connected L2 U2N Remote UE(s) or receive the L2 U2N Remote UE’s paging via dedicated RRC message from the gNB. Upon receving the paging message, the L2 U2N Relay UE sends relevant paging record to the L2 U2N Remote UE(s) accordingly.

For purpose of service continuity of L2 U2N relay, the L2 U2N Remote UE is switched between indirect and direct path.  The detailed procedures for path switch are specified in section 16.12.6 of TS 38.300[20].

## 6.y Sidelink Discovery

The UE may perform NR sidelink discovery while in-coverage or out-of-coverage for L2 relay, L3 relay and non-relay operation. There are two discovery models, Model A and Model B, which are defined in TS 23.304 [23]. The protocol stack used for discovery is presented in Figure 6.y-1 as specified in section 16.12.3 of TS 38.300 [20].

Figure 6.y-1: Protocol Stack of Sidelink Discovery

For L2 and L3 relay discovery, the network may broadcast or configure via dedicated signalling, a threshold, which is used by the U2N Remote UE to determine if it can transmit Relay discovery messages to U2N Relay UE(s). The network may also broadcast, or configure via dedicated signalling,  a maximum Uu RSRP threshold and/or a minimum Uu RSRP threshold, which are used by the U2N Relay UE to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

For L2 and L3 relay discovery, the resource pool(s) used for NR sidelink communication can be used for Relay discovery or the network may configure a resource pool(s) dedicated for Relay discovery. Whether a dedicated resource pool(s) for Relay discovery is configured is based on network implementation.

End of Change