3GPP TSG-RAN WG2 #119                                                                                         R2-220xxxx

E-Conference, 17th – 29th August 2022

Agenda Item:	    6.24.1

Source: 	    Apple

Title:		    Discussion on FR2 UL Gap MAC CR (Apple)

Document for:	    Discussion and decision

# 1 Introduction

The is the summary of the following email discussion.

[AT119-e][030][NR17] FR2 UL Gap MAC CR (Apple)

Scope: Treat R2-2206959, R2-2208931

Intended outcome: Brief Report, Agreed CR (if possible).

Deadline: EOM

R2-2206959	LS to RAN2 on UL gap in FR2 RF enhancement (R4-2211222; contact: Apple)	RAN4	LS in	Rel-17	NR_RF_FR2_req_enh2	To:RAN2	Cc:RAN1

R2-2208931	Correction on FR2 UL gap		Apple 	CR	Rel-17	38.321	17.1.0	1399	-	F	NR_RF_FR2_req_enh2		LATE

Chair: This CR was provided at the meeting.

# 2 Discussion

## 2.1 Discussion on the LS

In the LS [1], RAN4 indicates two updates. First one is about UE capability copied below.

UL gap for Tx power management: On UE capability, for inter-band FR2-FR2 CA/DC to indicate if UL transmission is feasible during UL gap when it is configured/activated - UE capability type: per band per band combination and optional - RAN4 confirm that this UE capability applies to inter-band FR2-FR2 CA/DC with CBM or IBM. - When UE indicates it supports this UE capability for a band in a band combination, it means it supports UL transmission during the configured UL gap in the band of the band combination. When UE indicates it does not support this UE capability for a band in a band combination, it means UL transmission is not feasible during the configured UL gap in the band of the band combination, except the transmission of UL signals defined in clause 9.1.X1 of 38.133.

Rapporteur’s understanding is this has already been captured in the current TS38.306 spec excerpted below.

| tx-Support-UL-GapFR2-r17 Indicates whether the UE supports UL transmission in FR2 bands within an FR2 UL gap when the FR2 UL gap is activated in inter-band UL CA. The UE which indicates support for tx-Support-UL-GapFR2-r17 shall also indicate support for ul-GapFR2-r17 in an FR2 band. | FS | No | No | FR2 only |
| --- | --- | --- | --- | --- |

One possible glitch is that FR2-FR2 DC was not captured in TS38.306. However, RAN4 has agreed in [3] that there is no FR2-FR2 band combination specified for NR-DC in RAN4 and it is up to RAN2 if FR2-FR2 NR-DC should be supported from signalling perspective. And RAN2 then agreed to not support NR-DC with FR2-FR2 for NR-DC.

Excerpted from RAN2#117 meeting: [058] FR2 UL gap design does not support NR-DC with FR2-FR2 band combination. FFS how to capture the restriction in spec.

So, rapporteur thinks there is no necessity to update current UE capability tx-Support-UL-GapFR2-r17 captured in TS38.306.

Question 1: Do companies agree that the field description of tx-Support-UL-GapFR2-r17 captured in TS38.306 do not need update?

| Company | Yes / No | Comments |
| --- | --- | --- |

The second point from the LS is on UE behavior regarding the allowed UL transmission during the UL gap.

Procedure to be prioritized over UL gap: When an UL gap overlaps with an uplink transmission in NR serving cells in FR2 single CC or FR2 intra-band CA, then the UE is not required to conduct any transmission during the UL gap on the NR serving cells other than those listed below: - UL signals belonging to random access procedure according to TS 38.321. - the PUSCH transmissions due to configured grant (CG-PUSCH (type 1 and 2)). - the valid CSI report and/or valid L1-RSRP report during SCell activation procedure, where the valid CSI report is valid CQI with non-zero CQI index defined in clause 5.2.2.1, TS 38.214 and the valid L1-RSRP report is non lowest L1-RSRP defined in clause 10.1.6. - The UE need not apply UL gap prioritization rules specified above for SCell activation procedure if the time period between UL gap colliding with CSI report of non-zero CQI or L1-RSRP and the slot where the SCell activation MAC CE or CSI report activation command is received is less than [X, and X is >=10 ms]. the PUCCH allocations for scheduling request (SR) and link recovery request (LRR) defined in clause 8.5.

The CR in [2] proposed to remove the detailed description in TS38.321 and to only refer to TS38.133.

## 5.30 Handling of FR2 UL gap During the FR2 UL gap configured by ul-GapFR2-Config as specified in TS 38.331 [5], the MAC entity shall not conduct uplink transmission other than the exceptions listed in TS 38.133 [11], clause 9.1.11, on the Serving Cell(s) of FR2 single CC and intra-band CA, or on the Serving Cell(s) in FR2 band(s) where UE does not support UL transmission within FR2 UL gap.

Question 2: Please companies indicate if they have comments on the change proposed in [2] as above.

| Company | CR to TS38.321 is needed? | Comments |
| --- | --- | --- |

# 3 Conclusions

[TBA]

# 4 References

[1] R2-2206959	LS to RAN2 on UL gap in FR2 RF enhancement (R4-2211222; contact: Apple)	RAN4	LS in	Rel-17	NR_RF_FR2_req_enh2	To:RAN2	Cc:RAN1

[2] R2-2208931	Correction on FR2 UL gap		Apple 	CR	Rel-17	38.321	17.1.0	1399	-	F	NR_RF_FR2_req_enh2		LATE

[3] R4-2202419 LS from RAN4 on FR2 UL gap