3GPP TSG-RAN2#119-e	   	 					 			       R2-2207983

Electronic meeting, August 17-26, 2022

Source: 			ZTE Corporation, Sanechips

Title:       CSI-RS related issues for NR operation above 71 GHz

Agenda item:	6.20.2

Document for: Discussion and Decision

# Introduction

In this contribution, we discuss the CP issues for NR operation above 71 GHz.

# Discussion

For CSI-RS measurement for mobility, according to TS 38.331[1], the periodicity and the corresponding offset are configured as below:

CSI-RS-Resource-Mobility ::=        SEQUENCE {

csi-RS-Index                        CSI-RS-Index,

slotConfig                          CHOICE {

ms4                                 INTEGER (0..31),

ms5                                 INTEGER (0..39),

ms10                                INTEGER (0..79),

ms20                                INTEGER (0..159),

ms40                                INTEGER (0..319)

},

...

}

For each SCS, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 is summarized as Table 1 below based on the field description for slotConfig.

Table 1 the maximum offset values for each periodicity and SCS

| SCS | the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 |
| --- | --- |
| 15kHz | 3/4/9/19/39 slots |
| 30kHz | 7/9/19/39/79 slots |
| 60kHz | 15/19/39/79/159 slots |
| 120kHz | 31/39/79/159/319 slots |

Since SCS of 480kHz and 960kHz for CSI-RS has been supported, the corresponding maximum offset values for each periodicity should be extended. Otherwise, only 1/8 periodicity offset may be used for 960kHz SCS. We think the restriction has a big impact on CSI-RS measurement. So it is beneficial to extend periodicity offset for 480 and 960kHz SCS. The extension may be based on the scaling principle, i.e. the maximum periodicity offset = periodicity * new SCS / 15 as Table 2.

Table 2 the maximum offset values for 480 and 960kHz SCS

| SCS | the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 |
| --- | --- |
| 480kHz | 127/159/319/639/1279 slots |
| 960kHz | 255/319/639/1279/2559 slots |

Proposal 1: For CSI-RS measurement for mobility, the maximum offset values for periodicity of 480 and 960kHz should be defined (as proposed in the attached draft CR).

Once proposal 1 is accepted, RAN2 can discuss whether this extended values are to be supported by the UEs or if a new UE capability is needed to indicate support for these new values since ASN.1 is frozen. If a new capability is needed, this can be added per UE as shown in the annex.

Proposal 2: RAN2 to discuss whether a new UE capability is needed to support the new periodicity values and if deemed necessary, add a UE capability to indicate whether the UE supports periodicity offset value extension.

For CSI-RS SCS used for mobility measurement, there is a typo. In FR2-1, the applicable SCS should be 60 kHz or 120 kHz, not 120kHz or 240kHz.

Proposal 3：CSI-RS SCS in FR2-1 should be modified as 60kHz or 120kHz as proposed in the attached draft CR.

# Conclusion and proposals

Based on the discussion in sections above we propose the following:

Proposal 1: For CSI-RS measurement for mobility, the maximum offset values for periodicity of 480 and 960kHz should be defined (as proposed in the attached draft CR – see Annex).

Proposal 2: RAN2 to discuss whether a new UE capability is needed to support the new periodicity values and if deemed necessary, add a UE capability to indicate whether the UE supports periodicity offset value extension.

Proposal 3：CSI-RS SCS in FR2-1 should be modified as 60kHz or 120kHz as proposed in the attached draft CR.

# Reference

- TS 38331-h10.

- TS 38306-h10.

- TS 36331-h10.

# Annex (Draft CR)

3GPP TSG-RAN2#119-e	Draft

# e-meeting, 17-29 August

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | Draft | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CSI-RS corrections for above 71 GHz operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 10/08/2022 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Issue 1): For CSI-RS measurement for mobility, the periodicity and the corresponding offset are configured as below: CSI-RS-Resource-Mobility ::= SEQUENCE { csi-RS-Index CSI-RS-Index, slotConfig CHOICE { ms4 INTEGER (0..31), ms5 INTEGER (0..39), ms10 INTEGER (0..79), ms20 INTEGER (0..159), ms40 INTEGER (0..319) }, ... } For each SCS, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 is summarized as Table 1 below based on the field description for slotConfig. Table 1 the maximum offset values for each periodicity and SCS SCS; the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40. 15kHz; 3/4/9/19/39 slots. 30kHz; 7/9/19/39/79 slots. 60kHz; 15/19/39/79/159 slots. 120kHz; 31/39/79/159/319 slots. Since SCS of 480kHz and 960kHz for CSI-RS are added, the corresponding maximum offset values for each periodicity should be extended. Otherwise, only 1/8 periodicity offset may be used for 960kHz SCS. Such a restriction has an impact on CSI-RS measurement and seems unintended. So, it is necessary to extend periodicity offset for 480 and 960kHz SCS. The extension may be based on the scaling principle, i.e. the maximum periodicity offset = periodicity * new SCS / 15 as shown below in Table 2. Table 2 the maximum offset values for 480 and 960kHz SCS SCS; the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40. 480kHz; 127/159/319/639/1279 slots. 960kHz; 255/319/639/1279/2559 slots. Issue 2) For CSI-RS SCS used for mobility measurement, there is a typo. In FR2-1, the applicable SCS should be 60 kHz or 120 kHz, not 120kHz or 240kHz. |
| Summary of change: |  | For Issue 1) Impact analysis Impacted 5G architecture options: SA, MR-DC Impacted functionality: SDT Inter-operability: For 1) - If the UE is implemented according to this CR while the network is not, there is no interoperability issue foreseen since the change is a clarification of behaviour that is captured in the field description. - If the network is implemented according to this CR while the UE is not, there is no interoperability issue since the change is a clarification of the behaviour that is captured in the field description. For 2) - If the UE is implemented according to this CR while the network is not, the networ may configure lower values (than 5ms) for CG-SDT periodicity which may not be supported at the UE and hence the configuration may be treated as invalid configuration on the UE side. As a result, the UE may not use the CG resource. - If the network is implemented according to this CR while the UE is not, there is no interoperability issue since the network anyway will not configure the invalid values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in the specs remain for the handling of ROHC continuity and for the periodicity of CG resources supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.3, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 6	Protocol data units, formats and parameters (ASN.1)

### 6.3.2	Radio resource control information elements

#### –	CSI-RS-ResourceConfigMobility

The IE CSI-RS-ResourceConfigMobility is used to configure CSI-RS based RRM measurements.

CSI-RS-ResourceConfigMobility information element

-- ASN1START

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-START

CSI-RS-ResourceConfigMobility ::=   SEQUENCE {

subcarrierSpacing                   SubcarrierSpacing,

csi-RS-CellList-Mobility            SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility,

...,

[[

refServCellIndex                    ServCellIndex                                                           OPTIONAL    -- Need S

]]

}

CSI-RS-CellMobility ::=             SEQUENCE {

cellId                              PhysCellId,

csi-rs-MeasurementBW                SEQUENCE {

nrofPRBs                            ENUMERATED { size24, size48, size96, size192, size264},

startPRB                            INTEGER(0..2169)

},

density                             ENUMERATED {d1,d3}                                                      OPTIONAL,   -- Need R

csi-rs-ResourceList-Mobility        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility

}

CSI-RS-Resource-Mobility ::=        SEQUENCE {

csi-RS-Index                        CSI-RS-Index,

slotConfig                          CHOICE {

ms4                                 INTEGER (0..31),

ms5                                 INTEGER (0..39),

ms10                                INTEGER (0..79),

ms20                                INTEGER (0..159),

ms40                                INTEGER (0..319)

},

associatedSSB                       SEQUENCE {

ssb-Index                           SSB-Index,

isQuasiColocated                    BOOLEAN

}                                                                                                           OPTIONAL, -- Need R

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12))

},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

sequenceGenerationConfig            INTEGER (0..1023),

...,

[[

slotConfig-v1720                    CHOICE {

ms4                                 INTEGER (0..255),

ms5                                 INTEGER (0..319),

ms10                                INTEGER (0..639),

ms20                                INTEGER (0..1279),

ms40                                INTEGER (0..2559)

}

]]

}

CSI-RS-Index ::=                    INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1)

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-STOP

-- ASN1STOP

CSI-RS-CellMobility field descriptions

csi-rs-ResourceList-Mobility List of CSI-RS resources for mobility. The maximum number of CSI-RS resources that can be configured per measObjectNR depends on the configuration of associatedSSB and the support of increasedNumberofCSIRSPerMO capability (see TS 38.214 [19], clause 5.1.6.1.3).

density Frequency domain density for the 1-port CSI-RS for L3 mobility. See TS 38.211 [16], clause 7.4.1.

nrofPRBs Allowed size of the measurement BW in PRBs. See TS 38.211 [16], clause 7.4.1.

startPRB Starting PRB index of the measurement bandwidth. See TS 38.211 [16], clause 7.4.1.

CSI-RS-ResourceConfigMobility field descriptions

csi-RS-CellList-Mobility List of cells for CSI-RS based RRM measurements.

refServCellIndex Indicates the serving cell providing the timing reference for CSI-RS resources without associatedSSB. The field may be present only if there is at least one CSI-RS resource configured without associatedSSB. If this field is absent, the UE shall use the timing of the PCell for measurements on the CSI-RS resources without associatedSSB. The CSI-RS resources and the serving cell indicated by refServCellIndex for timing reference should be located in the same band.

subcarrierSpacing Subcarrier spacing of CSI-RS. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

CSI-RS-Resource-Mobility field descriptions

associatedSSB If this field is present, the UE may base the timing of the CSI-RS resource indicated in CSI-RS-Resource-Mobility on the timing of the cell indicated by the cellId in the CSI-RS-CellMobility. In this case, the UE is not required to monitor that CSI-RS resource if the UE cannot detect the SS/PBCH block indicated by this associatedSSB and cellId. If this field is absent, the UE shall base the timing of the CSI-RS resource indicated in CSI-RS-Resource-Mobility on the timing of the serving cell indicated by refServCellIndex. In this case, the UE is required to measure the CSI-RS resource even if SS/PBCH block(s) with cellId in the CSI-RS-CellMobility are not detected. CSI-RS resources with and without associatedSSB may be configured in accordance with the rules in TS 38.214 [19], clause 5.1.6.1.3.

csi-RS-Index CSI-RS resource index associated to the CSI-RS resource to be measured (and used for reporting).

firstOFDMSymbolInTimeDomain Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS, see TS 38.211 [16], clause 7.4.1.5.3. Value 2 is supported only when dmrs-TypeA-Position equals pos3.

frequencyDomainAllocation Frequency domain allocation within a physical resource block in accordance with TS 38.211 [16], clause 7.4.1.5.3 including table 7.4.1.5.2-1. The number of bits that may be set to one depend on the chosen row in that table.

isQuasiColocated Indicates that the CSI-RS resource is quasi co-located with the associated SS/PBCH block, see TS 38.214 [19], clause 5.1.6.1.3.

sequenceGenerationConfig Scrambling ID for CSI-RS (see TS 38.211 [16], clause 7.4.1.5.2).

slotConfig Indicates the CSI-RS periodicity (in milliseconds) and for each periodicity the offset (in number of slots). When subcarrierSpacing is set to kHz15, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 3/4/9/19/39 slots. When subcarrierSpacing is set to kHz30, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 7/9/19/39/79 slots. When subcarrierSpacing is set to kHz60, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 15/19/39/79/159 slots. When subcarrierSpacing is set kHz120, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 31/39/79/159/319 slots. When subcarrierSpacing is set to kHz480, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 127/159/319/639/1279 slots. When subcarrierSpacing is set to kHz960, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 255/319/639/1279/2559 slots. If slotConfig-v1720 is present, UE shall ignore the slotConfig (without suffix).