3GPP TSG RAN WG2 Meeting #119bis-e                                                        	   R2-220xxxx

E-Meeting, 10th – 19th October 2022

Agenda Item:	6.0.2

Source: 	Intel Corporation

Title:	Report of [AT119b-e][004][NR17] UE caps Main (Intel)

Document for: 	Discussion/Decision

Introduction

This document aims to initiate the following offline discussion:

[AT119bis-e][004][NR17] UE caps Main (Intel)

Scope: Treat R2-2210660, R2-2210661, R2-2210565, R2-2210585 (if / when updated R1 feature list is available). Take into account updates to R1 and R4 feature lists, if they become available during the meeting. Determine agreeable parts, for agreeable parts capture in CRs,

Intended outcome: Report, Agreed-in-principle CRs (rapporteur can choose if to merge into mega CRs at current or next meeting).

Deadline: Schedule 1, or modifications by Rapporteur

Companies’ point of contact

| Company | Point of contact | Email address |
| --- | --- | --- |
| Intel Corporation | Seau Sian Lim | seau.s.lim@intel.com |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |
| Qualcomm Incorporated | Masato KITAZOE | mkitazoe@qti.qualcomm.com |
| OPPO | Qianxi Lu | qianxi.lu@oppo.com |
| MediaTek | Felix Tsai | Chun-fan.tsai@mediatek.com |
| ZTE | Wenting Li | Li.wenting@zte.com.cn |

NTN support by RedCap UE

The CR [1] has the following reason for change:

In 4.2.21.1,feature group not supported by RedCap UE  are explicitly listed, and other features not listed there are assumed to be supproted by RedCap UEs, as specified below.

CA, MR-DC, DAPS, CPAC and IAB (i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as non-RedCap UEs, unless indicated otherwise.

It would be difficult for RedCap UE to support NTN becaue link budget of NTN link would be insufficient for RedCap UEs and NTN requires extra capabilities to enable NTN on top of TN capabilities, which is not acceptable for RedCap UE.

Q1 Do companies agree with the proposed changes in the CR? If not, please provide your reasons in the comment column.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| Xiaomi | Yes |  |
| Qualcomm Incorporated | No | From RAN2 perspective, we have not identified any blocking issue for supporting RedCap UEs supporting NTN access. Link budget is essentially deployment issue, and no quantitative analysis was provided by the CR proponent. |
| MediaTek | No | Similar view as QC, we cannot find blocking issue that prevents Redcap UE from using NTN functionality. Link budget could be challenging, but Redcap UE choosing to implement NTN should already take this into consideration. |
| ZTE(Liujing) | Yes |  |

Clarification on the MBS feature 33-1-2 and 33-3-2

The CR [2] has the following reason for change:

In the RAN1#109-e meeting, RAN1 made the following agreements:

For FDM between one unicast PDSCH and one group-common PDSCH in a slot, only case 1 in the following cases is supported.

- Case 1: the unicast PDSCH and the group-common PDSCH in a slot are partially or fully overlapping in time domain and non-overlapping in frequency domain

- Case 2: the unicast PDSCH and the group-common PDSCH in a slot are non-overlapping in time domain and non-overlapping in frequency domain

- Case 3: the unicast PDSCH and the group-common PDSCH in a slot are non-overlapping in time domain and overlapping in frequency domain

The corresponding RAN1 features are as follows:

According to the RAN#97-e meeting discussion, two CRs (i.e. RP-222552 and RP-222553) related to MBS feature 33-1-2 and 33-3-2 are reserved. According to the 38.306 CR  in RP-222552, it is still unclear whether the FDM capability covers the case that two PDSCHs can be partially or fully overlapping in time domain.

Q2 Do companies agree with the proposed changes in the CR? If not, please provide your reasons in the comment column.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | This should be discussed in RAN1 first and if this is the correct understanding, the R1 feature list should be updated to reflect on this before RAN2 update 38.306. Proponent of the CR can bring this to RAN1 directly to discuss it as part of the feature list update. |
| Xiaomi | Yes | Since this FDM feature has been agreed by RAN1 several meetings ago, we think that companies can double check with their RAN1 colleagues on the correct understanding on the MBS feature 33-1-2 and 33-3-2. I would also agree that the RAN1 feature list does not provide the detailed description on the agreed Case 1. If companies think that the feature list needs to be updated to capture the RAN1 agreements correctly, we can send an LS to RAN1. |
| Qualcomm Incorporated | Yes | In general, it is preferred to act only based on RAN1 input. But our internal check with RAN1 colleagues suggests that the proposed change is agreeable. |
| MediaTek | Yes | We also think the clarificaiton is correct and aligned with RAN1’s intention. |

Clarification on the ue-PowerClassPerBandPerBC-r17

The discussion paper [3] and the corresponding CR [4] attempt to provide the dependencies related to the new UE capability ue-PowerClassPerBandPerBC-r17 with the existing power class UE capabilities in Rel-15 and 16. The following proposals are provided in the discussion paper:

Proposal 1: For MR-DC BCs containing only single CC or intra-band CA in NR side, the ue-PowerClassPerBandPerBC-r17shall be aligned to the corresponding powerClassNRPart-r16.

Proposal 2: If the ue-PowerClassPerBandPerBC-r17 was reported, the minimum value of ue-PowerClassPerBandPerBC-r17 and powerClass(powerClass-v1610) determines maximum TX power available in the corresponding band.

Proposal 2a: If the ue-PowerClassPerBandPerBC-r17 was not reported, the minimum value of ue-PowerClass(-v1610/1700)  and powerClass(powerClass-v1610) determines maximum TX power available in the corresponding band.

Rapporteur noticed that the note in the latest R4 feature list R4-2215143 (R4 16-8) is removed but the note for ue-PowerClassPerBandPerBC-r17 seems to have been left or included mistakenly as follow:

| ue-PowerClassPerBandPerBC-r17 Indicates the UE power class per band per band combination. NOTE: It is not applicable to the case when UL-MIMO and intra-band UL CA are in operation at the same time. | FS | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Q3 Do companies agree with the proposals above in [4]?

| Company | Proposal 1 (yes/no) | Proposal 2 (yes/no) | Proposal 2a (yes/no) | Comments |
| --- | --- | --- | --- | --- |
| Intel | See comment | See comment | See comment | We think that whether to add these kind of dependencies or restriction should be first discussed in RAN4. Our understanding is that the proposals are related to the the applicability of the feature to intra-band and inter-band UL CA. As the rapporteur indicated, in the last RAN4 meeting the note in the R4 feature list related to intra-band UL CA is removed. Therefore, whether to add additional dependencies should be first discussed in RAN 4. If the proposals are supported by majority, we could check with RAN4 whether the proposals on the dependencies are correct and asked them to include them to the feature list table. |
| Qualcomm Incorporated | See comment | See comment | See comment | Interaction among different power class parameters has been difficult topic which RAN2 could not resolve themselves in the past. Better to check with RAN4. |
| OPPO | No | Check with R4 | Check with R4 | For P1, according to our R4 colleagues, the new P17 IE is mainly for CA case, and the deletion of Is just to avoid the limitation to either intra- or inter-band CA, but no intention to extend it to EN/NE-DC where the powerClassNRPart-r16 is applied. And please note that the two IEs are of different value range, so infeasible to align the value setting. For P2 and P3, safer to check with R4, for the relationship between the 3 classes, per-band, per-BC and per-FS. |
| MediaTek | No | See comment | See comment | FG R4 16-8 is only applicable for inter-band UL CA therefore we do not agree to link it to FG R2 2-23. For P2 and P2a, we share the same view as Intel and think the RAN2/RAN4 mutual check is needed before adding such dependencies. |
| ZTE(Wenting) | Yes(proponent) | Yes(proponent) | Yes(proponent) | We are also OK to ask RAN4 for clarification. Based on companies comments, it seems that at least the below issues need to be confirmed by RAN4: - Whether the R4 16-8 is only applicable for inter-band UL CA - The Interaction among different power class parameters |

Q4 If the proposals are agreeable from RAN2 perspective, do companies agree to send a LS to RAN4 to check the proposals?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| Xiaomi | Yes |  |
| Qualcomm Incorporated | See comment | Support sending LS to RAN4. We do not need to make any RAN2 agreement and should just focus on asking questions for clarification. |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| ZTE(Wenting) | Yes |  |

Q5 Do companies agree to remove the note as below in ue-PowerClassPerBandPerBC-r17 to align with R4 feature list?

NOTE:	It is not applicable to the case when UL-MIMO and intra-band UL CA are in operation at the same time.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes | To align with the latest R4 list until the next one is received. |
| Qualcomm Incorporated | Yes |  |
| OPPO | Yes |  |
| MediaTek | Yes | On the other hand, RAN2 could consider to add applicability description for FG 16-8 since it had been clarified in the title of corresponding RAN4 CR (38.101-1 CR1113r1): “Increasing the maximum power limit for inter-band UL CA” |
| ZTE(Wenting) | Yes |  |

Conclusion

To be added latter

References

[1] R2-2210565	Corrections to NTN capabilities	LG Electronics	CR	Rel-17	38.306	17.2.0	0817	-	F	NR_NTN_solutions-Core, NR_redcap-Core

[2] R2-2210585	Clarification on the MBS feature 33-1-2 and 33-3-2	Xiaomi	draftCR	Rel-17	38.306	17.2.0	F	NR_MBS-Core

[3] R2-2210660	Clairificaiton on the ue-PowerClassPerBandPerBC-r17	ZTE Corporation, Sanechips	discussion	Rel-17	NR_RF_FR1_enh

[4] R2-2210661	CR on the ue-PowerClassPerBandPerBC-r17	ZTE Corporation, Sanechips	CR	Rel-17	38.306	17.2.0	0820	-	F	NR_RF_FR1_enh