3GPP TSG-RAN WG2 Meeting # 119bis-e 	R2-2210080

# e-meeting,  October 10th  - 19th , 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1422 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Enhanced TCI States Indication for UE-specific PDCCH MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to RAN1 LS (R2-2209151), RAN1 indicated that they have agreed that UE does not expect CORESET#0 to be activated with two TCI states when it is associated with SS#0 for Type 0/0A/2 CSS. RAN1 would like to request RAN2 to update the correspnoding specifiction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a Note. ‘The CORESET #0 will be not activated with two TCI states when it is associated with searchspace #0 for Type 0/0A/2 CSS.’ Impact analysis. Impacted 5G architecture options: NR SA, NR-DC, NE-DC Impacted functionality: Enhanced TCI States Indication for UE-specific PDCCH MAC CE Inter-operability: - If the network is implemented according to the CR and the UE is not, there are no interoperability issues. - If the UE is implemented according to the CR and the network is not, there are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of Enhanced TCI States Indication for UE-specific PDCCH MAC CE is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.44 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

first of Changes

#### 6.1.3.44	Enhanced TCI States Indication for UE-specific PDCCH MAC CE

The Enhanced TCI States Indication for UE-specific PDCCH MAC CE is identified by a MAC PDU subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size of 24 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits. If the indicated Serving Cell is configured as part of a simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 as specified in TS 38.331 [5], this MAC CE applies to all theServing Cells in the set simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2, respectively;

-	CORESET ID: This field indicates a Control Resource Set identified with ControlResourceSetId as specified in TS 38.331 [5], for which the TCI State is being indicated. In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5]. The length of the field is 4 bits;

-	TCI state IDi: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5] applicable to the Control Resource Set identified by CORESET ID field. If the field of CORESET ID is set to the other value than 0, this field indicates a TCI-StateId configured by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList in the controlResourceSet identified by the indicated CORESET ID. The length of the field is 7 bits.

NOTE 1:	The Enhanced TCI State Indication for UE specific PDCCH MAC CE is not applicable to any of the configured CORESETs in a BWP if the CORESETs are configured with different CORESETPoolindex values in the BWP.

NOTE 2:	The Enhanced TCI State Indication for UE specific PDCCH MAC CE is applied only if sfnSchemePdcch is configured.

NOTE 3:  The CORESET #0 will be not activated with two TCI states when it is associated with searchspace #0 for Type 0/0A/2 CSS.

Figure 6.1.3.44-1: Enhanced TCI States Indication for UE-specific PDCCH MAC CE

End of changes