3GPP TSG-RAN WG2 Meeting #119bis-e	R2-2210170

# Electronic Meeting, Oct, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3527 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for receiving notification message during path switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-9-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When UE receives RRCReconfiguration message for path switching from indirect to direct path, UE performs random access and starts T304. For the source inirect path, the timing to execute source link release is up to UE implementation. In addition, the RRCReconfiguration message to the L2 U2N Relay UE can be sent any time after transmitting RRCReconfiguration message to the remote UE based on gNB implementation (see TS38.300, 16.12.6.1). That means the remote UE still can receive NotificationMessageSidelink message due to e.g RLF from the source relay UE. According to 5.8.9.10.4, upon receiving the NotificationMessageSidelink, the U2N Remote UE in RRC_CONNECTED shall initiate the RRC connection re-establishment procedure if T301 is not running and indicationType is included. It is obious that the remote UE does not initiate the RRC connection re-establishment procedure during path switching upon receiving the NotificationMessageSidelink from the source relay UE. Therefore, the above case should be excluded in 5.8.9.10.4. Namely, ‘T304 is not running’ should be added to exclude the case that UE is performing path switching from indirect to direct path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.8.9.10.4, ‘T304 is not running’ is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE is triggered to initiate re-establishment procedure during path switching from indirect to direct path upon receiving the NotificationMessageSidelink from the source relay UE. Impact Analysis Impacted 5G architecture options: NG-RAN Architecture supporting the PC5 interface Impacted functionality: Relay selection for remote UE Inter-operability: There is no inter-operatbility issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.8.9.10.4	Actions related to reception of NotificationMessageSidelink message

Upon receiving the NotificationMessageSidelink, the U2N Remote UE shall:

1>	if the indicationType is included:

2>	if the UE is L2 U2N Remote UE in RRC_CONNECTED:

3>	if T301 and T304 are not running, initiate the RRC connection re-establishment procedure as specified in 5.3.7;

2>	else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE):

3>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

4>	indicate upper layers to trigger PC5 unicast link release.

3>	else (i.e., maintain the PC5 RRC connection):

4>	if the UE is L2 U2N Remote UE and the indicationType is relayUE-HO or relayUE-CellReselection:

5>	consider cell re-selection occurs;

NOTE:	For L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, it is up to Remote UE implementation whether to release or keep the PC5 unicast link.

End of changes