3GPP TSG-RAN WG2 Meeting #119-bis-e	 R2-2210368

Online, 10th Oct – 19th Oct 2022

Agenda Item:	8.6.1

Souce:	MediaTek Inc.

Title:	List of RAN2 Agreements in IoT-NTN

Document for:	    Discussion and decision

# 1 Introduction

Rel-18 enhancements for IoT-NTN Work Item was approved at RAN Plenary #96e [1]. During RAN2#119-e, some important agreements are made. As RAN2#119-e was the first meeting, and we have not decided to start making any Stage-2 running CR yet, this Tdoc is prepared to maintain the list of major RAN2 agreements.

# 2 RAN2 Progress and Agreement-List

## 2.1	Agreements

Agreements on “8.6.2 Performance Enhancements”

Agreements:

- Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability

- For UL HARQ operation, introduce two HARQ modes, i.e., HARQ mode A and HARQ mode B in IoT NTN (both NB-IoT and eMTC NTN), similarly to NR NTN

- From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT (and especially for CP solution for NB-IOT).

Agreements on “8.6.3 Mobility Enhancements”

Agreements:

- IoT NTN can use the mechanism for neighbor cell measurements in connected mode (specified in Rel-17 for NB-IoT). FFS if any enhancements are needed (e.g., triggers) for both NB-IoT and eMTC.

- RAN2 to continue working on a new time-based trigger for triggering intra and inter frequency measurements in connected mode, e.g., the serving cell is going to stop covering the current area, for both earth-moving and earth-fixed cell (FFS on distance-based trigger)

- CHO enhancements for eMTC NTN (i.e., time/timer based solution) are introduced based on the R17 NR NTN solution. FFS on location-based solution

- Measurement results reporting is not supported in Rel-18 NB-IoT NTN.

## 2.2	Remaining Open Issues

Objective 1 (IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17):

- RAN2 aspects for Disabling of HARQ feedback in NB-IoT NTN to mitigate impact of HARQ stalling.

- Further details on disabling HARQ feedback in DL for both NB-IoT and eMTC based NTN.

- Further details on HARQ mode A and HARQ mode B for UL HARQ operation in IoT NTN.

Objective 2 (mobility enhancements):

- Possible enhancements (e.g., triggers) for neighbour cell measurements in connected mode (for both NB-IoT and eMTC).

- Possible CHO enhancements for eMTC-based IoT-NTN.

Objective 3 (further enhancement to discontinuous coverage): No discussion in RAN2 119bis-e (Oct) meeting, waiting for some progress in SA2 Study.

# 3 References

- RP-221554, MediaTek, WID on IoT NTN Enhancements, RAN#96e, June 2022.