3GPP TSG-RAN WG2 Meeting #120 electronic	R2-2212866

Toulouse, 14th November – 18th November, 2022

Agenda item:		8.9.4

Source:		Lenovo

Title:		Discussion on Multi-path relaying

Document for:		Discussion and Decision

# Introduction

The following agreements for multi-path relaying are achieved in RAN2#119bis-e.

Agreements:

Proposal 1-1A (modified): The following cases are to be supported for Scenario 1.

A.	The remote UE operating only on the direct path adds the indirect path under the same gNB;

B.	The remote UE operating only on the indirect path adds the direct path under the same gNB;

C.	The remote UE operating in multi-path releases the indirect path;

D.	The remote UE operating in multi-path releases the direct path;

G.	The remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.  FFS if this case would be supported via separate release-and-add (A+C in separate reconfigurations) or a single switch procedure (e.g. similar to i2i service continuity).

Proposal 1-1B (modified): The following case is to be not supported for Scenario 1 as a group mobility scenario.

F.	The remote UE configured with multi-path keeps the serving relay UE for the indirect path and the serving cell of the remote UE for the direct path while the serving relay UE changes the serving cell of the relay UE under the same gNB;

Agreement:

The following case can be supported via separate release-and-add for scenario 1 (B+D in separate reconfigurations):

E.	The remote UE operating in multi-path changes the direct path to a different cell of the same gNB while using the serving relay UE for the indirect path under the same gNB.

FFS if a single procedure for this case would be supported.

Agreements:

Proposal 1	[21/21] Multi-path Relay is applicable to RRC_CONNECTED [18/18] remote-UE, for scenario-1 and scenario-2.

Proposal 3	[21/21] Multi-path Relay is NOT applicable to RRC_IDLE [18/18] remote-UE, for scenario-1 and scenario-2.

Proposal 10	[21/21] For multi-path Relay, support RRC_IDLE/RRC_INACTIVE target relay UE, for the path switching scenario where there is an addition of indirect path or a change of indirect path.

Proposal 12	[21/21] (modified) When UE operating in multi-path Relay, it performs RLM for Uu interface, for Scenario-1 and Scenario-2. For PC5 interface in Scenario-1, it performs sidelink RLF detection based on Rel-16 V2X specification [20/21]. For UE-UE link in Scenario-2, whether/how to have failure detection is out of 3GPP scope.

FFS whether there is impact to layers under our control from a failure of the UE-UE link in scenario 2.

Agreements:

Alternative proposal 7-1 (modified): FFS CPDU submission; if legacy CPDU submission behaviour is supported, the primary RLC entity of the MP split bearer for DRB can be configured on any of the paths for Scenario 1.

Proposal 8-1 (modified): PDCP DRB duplication is supported for the MP split bearer in Scenario 1 based on the existing framework.

Proposal 8-2 (modified): PDCP DRB duplication is supported for the MP split bearer in Scenario 2 based on the existing framework.

Note: Alternative proposal 7-1 was edited after the session to clarify the wording.

Agreements:

Alternative proposal 7-1 (modified): FFS CPDU submission; if legacy CPDU submission behaviour is supported, the primary RLC entity of the MP split bearer for DRB can be configured on any of the paths for Scenario 1.

Proposal 8-1 (modified): PDCP DRB duplication is supported for the MP split bearer in Scenario 1 based on the existing framework.

Proposal 8-2 (modified): PDCP DRB duplication is supported for the MP split bearer in Scenario 2 based on the existing framework.

Note: Alternative proposal 7-1 was edited after the session to clarify the wording.

In this contribution, we further discuss the related issues in multi-path relaying case.

# Discussion

Mobility scenarios

It has been agreed that the relay change for the remote UE can be supported in multi-path relaying case. Considering that it has been included in the scope of WI to enhance service continuity for the scenarios e.g., intra-gNB and inter gNB indirect-to-indirect path switching, it is natural to support the intra-gNB Relay change via a single switch procedure. Correspondingly, the related legacy timer, e.g., T420 can also be reused in multi-path relaying case. Similarly, the intra-gNB cell change can also be via a single procedure.

Proposal 1: Intra-gNB Relay change via a single switch procedure while the remote UE keeps the direct path under the same gNB should be supported.

Proposal 2: Legacy timer T420 can be reused in the case of relay change.

Proposal 3: Intra-gNB cell change via a single procedure while the remote UE keeps the indirect path under the same gNB should be supported.

To support the transmission via multiple paths, no consensus has been reached although the architecture has been discussed. Referring to the existing architecture, there is a high degree of similarity between multi-path and dual connectivity, for example there are more than one paths for both cases. Then, it is straightforward to configure the paths to primary path and secondary paths and the concept of cell group is ignored. On the other side, there also is difference between multi-path and dual connectivity, for example only one gNB involved in the Rel-18 multi-path relaying case. Then, it may work in a new architecture, for example no concept of primary path/secondary path or MCG/SCG. However, how to determine whether the path switching is successful in the case that multi-path is configured in target needs to be further discussed if there is no concept of primary path/secondary path or MCG/SCG.

Proposal 4: The concept of primary path for CP can be supported.

Similar to the discussion of mobility with DC, there are several potential cases when considering the inter-gNB handover in the multi-path relaying case. It could be one path or multi-path in the source side, and it could also be one path or multi-path in the target side. And for the indirect path, it can be further distinguished as same or different relays in the source side and target side. Therefore, we list the three potential use cases to be discussed in multi-path relaying case.

Proposal 5: The following Inter-gNB handover associated with Multi-path should be discussed.

- Use Case 1: Multi-path in source gNB-> Only one path in target gNB;

- Use Case 2: One path in source->Multi-path in target gNB;

- Use Case 3: Multi-path in source gNB-> Multi-path in target gNB (different relays in the source gNB and target gNB);

For scenario 2, the remote UE and relay UE is a pair, which could be pre-configured. That means the remote UE is allowed to select a relay UE based on the pre-configuration. One case is that the relay UE is in proximity of the remote UE. And they may move together. Therefore, Inter-gNB handover for remote UE with the same relay UE should be supported for scenario 2.

Proposal 6: Inter-gNB handover of a remote UE together with the same relay UE in Scenario 2 should be discussed.

Enhancement of throughput and reliability

Considering the existing mechanisms, when configuring duplication for a DRB, RRC also sets the state of PDCP duplication (either activated or deactivated) at the time of (re-)configuration. After the configuration, the PDCP duplication state can then be dynamically controlled by means of a MAC control element and in DC, the UE applies the MAC CE commands regardless of their origin (MCG or SCG). When duplication is configured for a SRB, the state is always active and cannot be dynamically controlled.

Since the purpose is similar to the legacy case, the existing activation/deactivation procedure can be used as the baseline for multi-path relaying case. Correspondingly, the Duplication Activation/Deactivation MAC CE can be reused to activation/deactivation PDCP DRB.

Proposal 7: The activation/deactivation of PDCP duplication for a DRB in multi-path relaying case can be dynamically controlled by the gNB.

Proposal 8: The Duplication Activation/Deactivation MAC CE can be reused.

With the similar consideration, the mechanism of data split can be used to allow the transmission to be switched among the direct path and indirect path in Rel-18 multi-path relaying case. The legacy configuration and transmission rule can be applied to Rel-18 multi-path relaying case as the baseline. Then the existing data volume threshold for split bearer can be considered as a baseline, and some enhancements may be needed to support MP split bearer in multi-path relaying case.

Proposal 9: Both legacy data volume threshold for split bearer and some enhancements can be considered to support MP split bearer in multi-path relaying case.

Sidelink Resource scheduling

Based on the above agreement, the following Figure 1 depicts the required scenario:

Figure 1: Inter cell - Intra gNB multipaths for a SL remote UE

The Figure 1 shows a scenario where the two “serving” cells of the Remote UE end up in the same gNB i.e., cell-1 and cell-2 Distributed Units (DU) are terminating in the same Centralized Unit (CU):

In legacy a sidelink transmitter is only provided sidelink resources from one serving cell. In multipath scenario however there are two serving cells available to schedule the UE which could come in handy in special circumstances like RLF on one link, or due to Consistent LBT failures or simply due to radio geometry.

Further, following sidelink data need to be transmitted by the remote UE:

- data that is destined to be sent to gNB and is configured to be carried in an indirect bearer i.e., a bearer configured only on the indirect path (data type-a)

- data that is destined to be sent to gNB and is configured to be carried in a split bearer i.e., data from this application will be carried in direct Uu link as well as in the indirect link/ path (data type-b)

- sidelink data that is destined for the U2N sidelink relay i.e., relay does not forward this data on the Uu interface to its serving cell (data type-c)

- sidelink data that is destined for another sidelink UE i.e., relay acts also as a UE to UE relay and forward this data on the sidelink interface to a destination sidelink remote UE (data type-d)

Figure 2

As shown in Figure 2, Bearer-1 is using only Uu resources of Cell-1. Bearer-2 is split bearer and remote UE may use both Uu and SL resources. For Bearer-3 the remote UE shall only use sidelink resources. Bearer-4 terminates in another SL device (U2N Relay or another SL UE). It is possible that different SL data are scheduled differently e.g., using Mode 1 or Mode 2 resources from Cell 1 or Cell 2. As one simple possibility, a UE would use sidelink resources (i.e., perform resource selection for transmission) provided in broadcast signalling (e.g., SIB12) by the cell with which UE had first link established. If a sl-ConfigDedicatedNR is received the UE uses the dedicated SL configuration for Mode 2. If Mode 1 scheduling is used, when the first link is the direct link, UE uses Mode 1 SL resources.

When the second path is added, following options are possible for SL scheduling:

- The first cell continues to schedule all SL resources for any SL data transmission of the remote UE.

- UE may use Mode 2 transmission resources from the indirect link in addition to Mode1/ Mode2 transmission resources from the direct link. Being able to use resources from both cells can be helpful in mobility situations.

- UE uses SL resources provided by the Primary path for all SL data type. If the primary path changes, the UE’s SL resources are scheduled by the new primary path.

As a first discussion, it can be beneficial to discuss if the remote UE can use mode 2 SL transmission resources from both the cells and if use of Mode 1 from direct link and Mode 2 from indirect must be restricted, we see no reason for this, and this does not mean a violation of simultaneous Mode 1 Mode 2 principle.

Proposal 10: Remote UE can use mode 2 SL transmission resources from both the serving cells in a multipath scenario.

Proposal 11: Remote UE can use mode 1 SL transmission resources from direct path and mode 2 SL transmission resources from the serving cell on the indirect path, in a multipath scenario.

# Conclusion

In this contribution, the mobility scenarios and enhancement of throughput and reliability for multi-path relaying are discussed. We have the following proposals:

Proposal 1: Intra-gNB Relay change via a single switch procedure while the remote UE keeps the direct path under the same gNB should be supported.

Proposal 2: Legacy timer T420 can be reused in the case of relay change.

Proposal 3: Intra-gNB cell change via a single procedure while the remote UE keeps the indirect path under the same gNB should be supported.

Proposal 4: The concept of primary path for CP can be supported.

Proposal 5: The following Inter-gNB handover associated with Multi-path should be discussed.

- Use Case 1: Multi-path in source gNB-> Only one path in target gNB;

- Use Case 2: One path in source->Multi-path in target gNB;

- Use Case 3: Multi-path in source gNB-> Multi-path in target gNB (different relays in the source gNB and target gNB);

Proposal 6: Inter-gNB handover of a remote UE together with the same relay UE in Scenario 2 should be discussed.

Proposal 7: The activation/deactivation of PDCP duplication for a DRB in multi-path relaying case can be dynamically controlled by the gNB.

Proposal 8: The Duplication Activation/Deactivation MAC CE can be reused.

Proposal 9: Both legacy data volume threshold for split bearer and some enhancements can be considered to support MP split bearer in multi-path relaying case.

Proposal 10: Remote UE can use mode 2 SL transmission resources from both the serving cells in a multipath scenario.

Proposal 11: Remote UE can use mode 1 SL transmission resources from direct path and mode 2 SL transmission resources from the serving cell on the indirect path, in a multipath scenario.

# References

RAN2#119bis-e chairman notes.