3GPP TSG-RAN WG2 #120	R2-2211215

Toulouse, France, November 14-18, 2022

Agenda Item:	6.15.1

Source:	OPPO

Title:	Discussion on left issues on Tx profile

Document for:	Discussion, Decision

Introduction

This paper will discuss some left issues on C1-226055.

Discussion

Based on the A1

A1: CT1 confirms the above understanding. In “V2X service identifier to PC5 RAT(s) and Tx profiles mapping rules” and “ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rules”, CT1 specification includes a value of TX profile (by referring to SL-TxProfile-r17 in clause 9.3 of 3GPP TS 38.331) instead of a list of Tx profiles.

Clearly, C1 spec refers to the single Tx profile value IE, not the list.

Currently, in 331, the list is defined in pre-configuration, with field description as follows

sl-TxProfileList List of one or multiple Tx profiles, which are indicated by upper layer in order of increasing Tx profile pointer identities, indicate the compatibility of supporting SL DRX as specified in TS 38.321 [3].

While ‘upper layer’ (C1 spec), does NOT refer to this list IE, what C1 spec does is: the C1 spec would provide a profile value, in the form of the IE ‘SL-TxProfile-r17’ as specified in 331 (again, a single value instead of a list).

In fact, the C1 spec status is: C1 spec provides the list in the shape of,

| service-1 | profile-1 |
| --- | --- |
| service-2 | profile-2 |
| service-3 | profile-3 |

Where for profile-1/2/3, C1 spec defined the field with length already, i.e., ‘Broadcast and groupcast mode NR-PC5 Tx profile’.

The only place where R2 spec is used, is when the profile field is provided to the UE, the coding is based on the IE defined in 331, i.e., ‘The broadcast and groupcast mode NR-PC5 Tx profile field is coded as SL-TxProfile-r17 in clause 9.3 of 3GPP TS 38.331 [12].’

<Start of TS 24.588>

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to PC5 RAT(s) and Tx profiles mapping rules contents |  |  |  |  |  |  |  | octet o2+1 octet o2+2 |
| V2X service identifier to PC5 RAT(s) and Tx profiles mapping rule 1 |  |  |  |  |  |  |  | octet (o2+3)* octet o10* |
| V2X service identifier to PC5 RAT(s) and Tx profiles mapping rule 2 |  |  |  |  |  |  |  | octet (o10+1)* octet o11* |
| ... |  |  |  |  |  |  |  | octet (o11+1)* octet o12* |
| V2X service identifier to PC5 RAT(s) and Tx profiles mapping rule n |  |  |  |  |  |  |  | octet (o12+1)* octet o3* |

Figure 5.3.1.12: V2X service identifier to PC5 RAT(s) and Tx profiles mapping rules

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to PC5 RAT(s) and Tx profiles mapping rule contents |  |  |  |  |  |  |  | octet o10+1 octet o10+2 |
| V2X service identifiers |  |  |  |  |  |  |  | octet o10+3 octet o79 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | BGNTI | PC5 RAT(s) |  | octet o79+1 |
| Length of E-UTRA-PC5 Tx profiles |  |  |  |  |  |  |  | octet (o79+2)* |
| E-UTRA-PC5 Tx profiles |  |  |  |  |  |  |  | octet (o79+3)* octet o82* |
| Broadcast and groupcast mode NR-PC5 Tx profile |  |  |  |  |  |  |  | octet (o82+1)* octet o83* = o11* |

Figure 5.3.1.13: V2X service identifier to PC5 RAT(s) and Tx profiles mapping rule

Table 5.3.1.13: V2X service identifier to PC5 RAT(s) and Tx profiles mapping rule

[…]

Broadcast and groupcast mode NR-PC5 Tx profile field:

The broadcast and groupcast mode NR-PC5 Tx profile field indicates NR Tx profile corresponding to the NR-PC5 for broadcast mode V2X communication over PC5 and groupcast mode V2X communication over PC5. The broadcast and groupcast mode NR-PC5 Tx profile field is coded as SL-TxProfile-r17 in clause 9.3 of 3GPP TS 38.331 [12].

If the length of V2X service identifier to PC5 RAT(s) and Tx profiles mapping rule contents field indicates a length bigger than indicated in figure 5.3.1.13, receiving entity shall ignore any superfluous octets located at the end of the V2X service identifier to PC5 RAT(s) and Tx profiles mapping rule contents.

<End of TS 24.588>

Even if there is some ambiguity before sending the LS to C1, it should be clarified after seeing the C1 reply.

On the other hand, there seems still some view in R2 that this ‘sl-TxProfileList’ can be used.

So as the first step, R2 needs to understand whether this ‘sl-TxProfileList’ is still useful

- If it is, the proponent needs to clarify how the UE and network make use of it. And this way it is used should be compatible with C1 spec status as clarified in LS C1-226055.

- Or if it is not used, it should be dummified.

Dummify ‘sl-TxProfileList’.

If P1 is not agreeable (i.e., this IE is to be kept), R2 clarify how it is used considering C1 spec status as clarified in C1-226055.

Conclusion

We have the following proposals:

Proposal 1	Dummify ‘sl-TxProfileList’.

Proposal 2	If P1 is not agreeable (i.e., this IE is to be kept), R2 clarify how it is used considering C1 spec status as clarified in C1-226055.

Reference

[1] C1-226055, Reply LS on Tx profile