3GPP TSG-RAN WG2 Meeting #120	                                                                                   R2-2211243

Toulouse, November 14-18, 2022

Source:	CATT, CBN

Title:	Further discussions on multicast reception in RRC_INACTIVE

Agenda Item:	8.11.2

Document for:	Discussion and Decision

- 1. Introduction

This contribution further discusses the open issues related to multicast reception in RRC_INACTIVE, including aspects such as PTM configuration, mobility, RRC state transition and notifications for session stage changes. The proposals are summarized in section 6.

- 2. PTM configuration(s) delivery

The following have been agreed previously

- For PTM configuration delivery, RAN2 further investigates the following solutions: - Option 1: Dedicated signalling - Option 2: Solution based on SIB+MCCH - We do not preclude some “mix” of the options

RAN2 also agreed on the general descriptions for Option 1 and 2. The potential issues have also been discussed for different options. In the following table, we provide detailed analysis on these two options.

Table 1 Further analysis of the Option 1 and 2

|  | Option 1 | Option 2 |
| --- | --- | --- |
| Resource/signaling load | High Dedicated resource/signaling used for configuration delivery to each UE | Low Shared resource/signaling used for configuration delivery to the UEs |
| PTM configuration update | Issue in the cases with high number of UEs If a large number of UEs need PTM configuration update, maybe need to further handle the RACH congestion/system load | No issue UEs can obtain the updated PTM configuration from MCCH, no need to trigger RRC Resume and no other efforts needed to handle the RACH congestion/system load |
| PTM configuration for neighbor cell | Optimization required It is essential to support PTM configuration applicable area, otherwise UE triggers RRC connection resume upon each cell change | Optimization is not required PTM configuration applicable area is not essential, UE can read MCCH from target cell directly, no need to trigger RRC Resume |
| Availability of PTM configuration without joining the session | Yes If a UE once joined but has already left the session, it can still keep the PTM configuration that was acquired previously | Yes UEs can obtain the PTM configuration from MCCH |

Among the aspects as shown in Table 1, the first two aspects clearly show that Option 2 is a better choice from performance point of view.

Regarding the third aspect, it has been pointed by some companies in RAN2 that whether to support PTM configuration applicable area needs RAN3 involvement. The current status with this issue in RAN3 seems to be that RAN3 is evaluating the impacts to interface signaling as well as to network implementation complexity, and there is so far no consensus on these aspects.

Regarding the last aspect, we note that for both options the configuration may be available to the UE regardless whether UE has joined the session or not previously. Also, in our view from RAN perspective there is no big issue in terms of security since application layer enables the security protection.

Taking the above into account, we have the following observations and proposal.

Observation 1: For Option 1, Resource/signaling load is high.

Observation 2: For Option 1, there may be a need to further handle RACH congestion/system load when a large number of UEs in the cell needs PTM configuration update.

Observation 3: For Option 1, it is essential to support PTM configuration applicable area, which is not very easy to realize at the network side and it requires more involvements with RAN3 to decide on this issue.

Observation 4: There is no critical issue that a UE can obtain all the PTM configurations for a multicast service via Option 2 without/before joining the multicast session on the condition that security is enabled by application layer. And, even for Option 1 UE can also have all the PTM configuration stored if UE joined the session ever but has leaved group now.

Proposal 1: Support Option 2 as baseline for PTM configuration of multicast reception in RRC_INACTIVE.

One more question is whether some mixed solution is needed. Technically since we haven’t identified any strong issue for Option 2 we see no need for the mixed solution. On the other hand, one may understand that for Option 2 (SIB+MCCH), it is possible to support that upon state transition from CONNECTED to INACTIVE the UE may keep the PTM configurations that it obtained when in RRC connected, or UE may be provided the PTM configurations by the network via the RRCRelease message and it can be used temporarily before get the PTM configuration from MCCH. Although these may not be so essential, there may be benefit since it saves the time that UE obtains the configurations via SIB+MCCH, which gives better UE experience in service reception. So in our view this kind of mixed solution can be further considered.

Proposal 2: RAN2 can further consider enhancements to Option 2 (i.e., the mixed solution), e.g., combined use of  PTM configuration from Dedicated signaling and MCCH.

- 3. Mobility

Previously the following were agreed

Multicast service continuity after cell reselection in RRC_INACTIVE state (i.e. without resuming RRC connection) will be supported (if the configuration of the new cell is available for the UE). FFS whether there are cases where the UE needs to resume the connection. FFS RAN3 impacts due to inter-gNB mobility. Upon cell reselection to neighbour cells during active multicast session, if the configuration of the session is not available for the new cell for UEs in INACTIVE, then the UE is required to resume RRC connection to get the Multicast MRB configuration.

One issue is whether it is beneficial to introduce some prioritization to the cells and/or frequencies where UE can receive multicast in INACTIVE in the cell reselection procedure.

In Rel-17, there are the following mechanism for service continuity: the UE which is receiving or interested to receive MBS broadcast service(s) via PTM and can only receive these MBS broadcast service(s) via PTM while camping on the frequency providing these MBS broadcast service(s) is allowed to make this frequency highest priority when the conditions described in TS 38.304 are met; when the MBS broadcast service(s) which the UE is interested in are no longer available (after the end of the session) or the UE is no longer interested in receiving the service(s), the UE no longer prioritizes the frequency providing these MBS broadcast service(s).

And in Rel-18 we think the following options can be further discussed.

Proposal 3:	The following options can be further discussed for cell reselection when UE is receiving multicast in RRC_INACTIVE

- Option 1: neighbor cell(s) on which the same multicast session can be received in INACTIVE may be prioritized for cell reselection

- Option 2: a frequency on which multicast reception in INACTIVE is supported may  be prioritized for cell reselection

- Option 3: no enhancement to the cell reselection

- 4. RRC state transitions

- 4.1 From CONNECTED state to INACTIVE state

Previously the following has been agreed

Dedicated RRC signalling (i.e. RRC release message with suspendConfig) is used for switching a multicast receiving UE from RRC_CONNECTED to RRC_INACTIVE and continue multicast reception (details FFS).

Here the difference compared with Rel-17 is that when a Rel-17 UEs that has been receiving the multicast session is released to INACTIVE, it assumes that either the session has been deactivated or there is currently no data to receive, and if the situation changes it will receive group paging. For Rel-18, the case that is being discussed here is the UE has been receiving the multicast session, but network may want to move it to INACTIVE and continue with the multicast reception. So this is different from Rel-17, and we need to discuss how network indicates so to the UE. In our view, there are two possible options,

Option 1: explicit indication (i.e., 1-bit flag)

In option 1, based on the indication in RRC release message, UE determines whether multicast reception should be continued when UE is in RRC_INACTIVE.

Option 2: implicit indication via the presence of the PTM configuration

In option 2, if PTM configuration is included in RRC release message, UE knows that the multicast reception can be continued when UE is in RRC_INACTIVE. With this option, it basically restrict the gNB to always include PTM configuration in RRC release message, if it wants to indicate the above mentioned state change to the UE. In our view such restriction is not needed and it should be up to network implementation. It can be further discussed whether such indication is per UE or per multicast session.

Proposal 4: When switching a multicast receiving UE from RRC_CONNECTED to RRC_INACTIVE, UE is indicated in the RRC Release message whether multicast reception is continued after UE transits to RRC_INACTIVE. FFS indication is per UE or per multicast session.

- 4.2 From INACTIVE state to CONNECTED state

Previously the following has been agreed

It is assumed the network can choose which UEs receive in RRC INACTIVE and which in RRC Connected and can move UEs between the states for Multicast service reception.

And then it was also agreed that group paging can be used to switch UEs receiving multicast from RRC_INACTIVE to RRC_CONNECTED, and UEs continue the multicast reception in CONNECTED,

For both option 1 and option 2, as a baseline, group paging can be used to switch UEs receiving multicast from RRC_INACTIVE to RRC_CONNECTED, and UEs continue the multicast reception in CONNECTED. FFS if there is any potential issue if Rel-17 group paging is reused. FFS if there are other cases when UE triggers resume. FFS if MCCH can also be used in case of option 2.

In order to further progress on the FFS part above, we look into different cases in terms of whether a subset or all the UEs receiving multicast in INACTIVE are switched from RRC_INACTIVE to RRC_CONNECTED by the gNB.

Firstly, the Rel-17 group paging may not be efficient or flexible when gNB only wants to address a subset of all the UEs. In many cases the changes in the system load is gradual, and gNB may want to switch only a portion of all the UEs receiving multicast from RRC_INACTIVE to RRC_CONNECTED. There are different possible solutions, e.g., the UEs receiving multicast in INACTIVE can be divided in different groups, and the paging is enhanced so that it can address one or multiple such UE groups. Other possible ways such as predefined rules could also be discussed to address only part of all the UEs.

Then, even if gNB only wants to use the group paging to switch all the UEs receiving multicast in INACTIVE are switched from RRC_INACTIVE to RRC_CONNECTED, it seems problematic to just reuse Rel-17 group paging. For example, let us assume Rel-17 group paging is reused, then the Rel-18 UE behaviors in the different cases are:

- If UE is in INACTIVE and the session is not activated, UE will start multicast reception in INACTIVE if it has valid PTM configuration when receiving the group paging.

- If UE is in INACTIVE and the session is activated, UE will trigger RRC Connection Resume when receiving the group paging.

If gNB sends Rel-17 group paging for the sake of multicast session activation multiple times (which is typical case in practice), a Rel-18 UE in INACTIVE may receive the same group paging message more than once. If so it is possible a Rel-18 UE do behavior b) above upon receiving the same Rel-17 group paging message which is intended for session activation, which is an error case.

With the above analysis, no matter all or only a subset of UEs are addressed by this group paging message, it seems some enhancements are needed. Therefore the following is proposed.

Proposal 5: To move all or a subset of multicast receiving UEs from RRC_INACTIVE to RRC_CONNECTED, enhancements are needed to Rel-17 group paging. Details FFS.

- 5. Notification of session state change

- 5.1 Notification of the session activation and whether the session can be received in INACTIVE

The following has been agreed session activation to R18 UEs

Rel-18 UE in INACTIVE can be informed when the session is activated (Details FFS). As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification).

Then regarding how UE determines whether it can receive the multicast session in RRC_INACTIVE or not when the session is activated, it is kept as FFS and several options are on the table.

- FFS how UE determines whether it can receive the multicast session in RRC_INACTIVE or not when the session is activated, taking into account the following solutions (can further update the descriptions if needed, and several solutions may be needed, some solutions may apply only for certain configuration options) - 1. When the multicast session is activated, UE can receive the multicast session in RRC_INACTIVE if the PTM configuration used in RRC_INACTIVE for the session is available to the UE and the UE has joined the session already (e.g., configuration provided to UE via dedicated RRC signaling or via MCCH), otherwise it goes back to RRC_CONNECTED to receive the multicast session. - 2. When the multicast session is activated, UE is indicated by group paging whether it can receive the multicast session in RRC_INACTIVE or not (detailed signaling FFS). - 3. UE is configured "whether it can receive the multicast session in RRC_INACTIVE" by dedicated signaling before UE is released. When the multicast session is activated, UE stays in RRC_INACTIVE or resumes RRC connection accordingly (detailed signaling FFS).

As discussed in section 4.2, we might anyway need to enhance Rel-17 group paging for RRC state transitions. Taking this into account, we think it is quite straightforward to take Option 2 above as the baseline. One benefit is it gives the network a tool and thus more flexibility to control the UE behavior via the group paging message.

Proposal 6: When the multicast session is activated, UE is indicated via group paging whether it can receive the multicast session in RRC_INACTIVE or not.

- 5.2 Session deactivation

For session deactivation, the following has been agreed

- If a UE is in RRC_INACTIVE and is configured to receive a multicast session in RRC_INACTIVE, the UE may be notified when the multicast session is deactivated. FFS how (e.g., informed via group paging, MCCH, or other ways).

This may relate to how PTM configurations are delivered. If Option 2 (i.e., SIB+MCCH) is supported, we see there are following alternatives to inform session deactivation.

| Alt. | Initial analysis |
| --- | --- |
| Alt 1: explicit indication in group paging | when a session is deactivated, gNB may sent group paging and indicate the deactivation state for the concerned multicast session ID (i.e. via TMGI). |
| Alt 2: explicit indication of the session state (e.g. active or inactive) on MCCH | UEs need to read the MCCH if the session state changes for any ongoing multicast session |
| Alt 3: implicitly indicated by the presence of PTM configuration on MCCH (i.e. the PTM configuration is removed from MCCH when the session is deactivated) | UEs need to read the MCCH to get the PTM configuration again when session is reactivated after deactivation. |

In our view these options can be further discussed.

Proposal 7: Down select the solution for session deactivation to UE in INACTIVE,

-	Alt 1: explicit indication in group paging

-	Alt 2: explicit indication of the session state (e.g. active or inactive) on MCCH

-	Alt 3: implicitly indicated by the presence of PTM configuration on MCCH (i.e. the PTM configuration is removed from MCCH when the session is deactivated)

- 6. Conclusion

In this contribution we discuss the open issues related to multicast reception in RRC_INACTIVE. The following observations and proposals are made based on the discussions.

PTM configuration delivery

Observation 1: For Option 1, Resource/signaling load is high.

Observation 2: For Option 1, there may be a need to further handle RACH congestion/system load when a large number of UEs in the cell needs PTM configuration update.

Observation 3: For Option 1, it is essential to support PTM configuration applicable area, which is not very easy to realize at the network side and it requires more involvements with RAN3 to decide on this issue.

Observation 4: There is no critical issue that a UE can obtain all the PTM configurations for a multicast service via Option 2 without/before joining the multicast session on the condition that security is enabled by application layer. And, even for Option 1 UE can also have all the PTM configuration stored if UE joined the session ever but has leaved group now.

Proposal 1 	Support Option 2 as baseline for PTM configuration of multicast reception in RRC_INACTIVE.

Proposal 2	RAN2 can further consider enhancements to Option 2 (i.e., the mixed solution), e.g., combined use of  PTM configuration from Dedicated signaling and MCCH.

Mobility

Proposal 3	The following options can be further discussed for cell reselection when UE is receiving multicast in RRC_INACTIVE

- Option 1: neighbor cell(s) on which the same multicast session can be received in INACTIVE may be prioritized for cell reselection

- Option 2: a frequency on which multicast reception in INACTIVE is supported may  be prioritized for cell reselection

- Option 3: no enhancement to the cell reselection

RRC state transitions

Proposal 4	When switching a multicast receiving UE from RRC_CONNECTED to RRC_INACTIVE, UE is indicated in the RRC Release message whether multicast reception is continued after UE transits to RRC_INACTIVE. FFS indication is per UE or per multicast session.

Proposal 5	To move all or a subset of multicast receiving UEs from RRC_INACTIVE to RRC_CONNECTED, enhancements are needed to Rel-17 group paging. Details FFS.

Notification of session state change

Proposal 6	When the multicast session is activated, UE is indicated via group paging whether it can receive the multicast session in RRC_INACTIVE or not.

Proposal 7	Down select the solution for session deactivation to UE in INACTIVE,

- Alt 1: explicit indication in group paging

- Alt 2: explicit indication of the session state (e.g. active or inactive) on MCCH

- Alt 3: implicitly indicated by the presence of PTM configuration on MCCH (i.e. the PTM configuration is removed from MCCH when the session is deactivated)

- 7. Reference

[1] Draft meeting report for RAN2#119bis-e, in https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_119bis-e/Report