3GPP TSG RAN WG2 Meeting #120          	      	                	                R2-2211374

Toulouse, FR, 14th – 18th Nov, 2022

Agenda Item:	8.12.2

Source:	Intel Corporation

Title:	Mobility Enhancement of mobile IAB-node and served UEs

Document for:	Discussion and Decision

# Introduction

During RAN2 #119bis-e meeting, RAN2 and RAN3 agreed to send mobile IAB-node indication to donor CU via UE capability and mobile IAB-node’s broadcast information to assist UE’s cell (re)selection: In this contribution, we further discuss the need of sending other information to donor CU and broadcast information to optimize cell (re)selection.

Moreover, we will also further discuss group mobility aspects, e.g. RACH-less handover and pre-configuration to served UEs.

# Discussion

## Mobile IAB-node Indication to Donor CU

Following agreements are agreed during RAN2 #119bis-e and RAN3 #117bis-e meeting:

RAN2 #119bis-e - UE capability signalling is the baseline to let CU know that the MT is a “mobile-IAB” type. FFS early mobile-IAB indication, e.g. in Msg5. - Regarding moving status/mode indication, R2 observes that legacy reporting of mobility state (e.g. mobilityState-r16) could be reused, and maybe also current location reporting from the UE. FFS whether any of this need to be enhanced or complemented, e.g. for the potential purpose of predictive mobility. RAN3 #117bis-e Source donor CU of mobile IAB-MT informs the target donor CU of mobile IAB-MT that the migrating node is a mobile IAB-node, via explicit indication in XnAP HO Request message. RAN3 to discuss whether source donor should know whether the target cell belongs to a mIAB-Node.

It is still open on whether to adopt some early mobile-IAB indication over Msg5 or not. During Rel-16, early indication “iab-NodeIndication-r16” was used to allow RAN to select an AMF/MME supporting IAB for authorization. For mobile IAB-node, since it’s still one type of IAB-node, this “iab-NodeIndication-r16” can be reused for its authorization. There’s no need to use early indication over Msg5 to indicate the network it’s a mobile IAB-node. It should be sufficient to identify a mIAB UE based on UE capability reporting.

Based on UE capability information, the mIAB UE information is passed from source donor CU to the target donor CU of mIAB-MT transparently over Xn interface without any additional specification change needed on Xn.

Proposal 1: Early mobile-IAB indication in Msg5 is not supported.

Different from stationary IAB-node, when an IAB-donor CU configures mobile IAB-node and its served UEs, it’s better to consider the real-time mobility status of a mobile IAB-node. With such information, the IAB-donor CU can configure suitable measurement configuration towards the served UEs and avoid unnecessary handover or selecting unsuitable type of target cell for the UEs based on the current status of the moving mobile IAB-node.

For example, when mobile IAB-node is moving, based on such information, IAB-donor CU may configure the served UEs to only perform measurement towards other mIAB nodes with the same velocity (a mobile IAB-node with different velocity is assumed to be equipped on another vehicle). This can help to reduce unnecessary measurement at UE side and further reduce power consumption for the UE. Or, the IAB-donor CU will not select those unsuitable stationary cells as targets for the served UEs. On the other hand, when the vehicle is going to stop, the served UE may move out of the vehicle and switch to a normal gNB/stationary node. When observing mobile IAB-node’s mobility status becomes zero/lower than certain threshold/becomes in the low level, the IAB-donor CU may send reconfiguration messages towards the served UEs, which includes measurement configuration towards neighbouring normal gNBs/stationary nodes. Hence, when the UE gets out of the vehicle, a normal gNB/stationary node can be selected as target.

Alternatively, the IAB-donor CU can also ignore the measurement reports towards fixed cells from the mIAB served UEs, when their connected mobile IAB-node’s velocity is not zero/in the high level.

Observation 1: It is useful for the mobile IAB-node reports its real-time mobility status to IAB-donor CU to allow IAB-donor CU to decide a suitable type of target cell (mIAB or stationary normal cell) for the served UEs based on different UE mobility scenarios.  Network can either configure suitable measurement configuration(s) for the served UE(s) or ignore served UEs’ measurement reports based on mobile IAB-node’s velocity information, avoiding handover to unsuitable target cells.

The question is how to report this real-time mobility status to the IAB-donor CU. During last meeting, legacy reporting of mobility state (e.g. mobilityState-r16) and current location reporting from the UE were proposed. However, based on our understanding, both of them cannot satisfy the IAB-donor CU to configure the served UEs adaptively based on mobile IAB-node’s mobility status.

First of all, mobilityState-r16 was reported over RRCResumeComplete or RRCSetupComplete message, indicating the mobility state (as specified in TS 38.304 [3]) of the UE just prior to entering RRC_CONNECTED state. However, this information is just a one-time report and cannot be continuously reported to IAB-donor CU after mIAB-MT enters RRC_CONNECTED state. Moreover, mobility state (as specified in TS 38.304 [3]) is determined based on the number of cell reselection during a certain time, which is defined as below:

Normal-mobility state criteria: - If number of cell reselections during time period TCRmax is less than NCR_M. Medium-mobility state criteria: - If number of cell reselections during time period TCRmax is greater than or equal to NCR_M but less than or equal to NCR_H. High-mobility state criteria: - If number of cell reselections during time period TCRmax is greater than NCR_H.

As defined above, mobility state (as specified in TS 38.304) can only 1) be collected during cell reselection and 2) represent a total number of cell change over a certain period. The IAB-donor CU cannot use this information to understand the real-time moving status of a mobile IAB-node after it enters RRC_CONNECTED state.

Although it is possible to extend mobility state to RRC_CONNECTED state, it still cannot be used as real-time moving status. This is because the simple reflection of cell change cannot be used to measure the moving status if a mobile IAB-node’s movement is changed within a cell (e.g. stop – move – stop).

Observation 2: mobilityState-r16 cannot be used to reflect the real-time moving status of a mobile IAB-node, because 1) it is not calculated after RRC_CONNECTED 2) it cannot represent a moving status change within a cell.

To help IAB-donor CU make a real-time configuration based on the moving status of a mobile IAB-node, it should report its real-time mobility status to IAB-donor CU. Compared with current location reporting, velocity information is easier to be reported. The velocity information can either be an accurate velocity or a speed state (to differentiate with mobility state). The speed state can be defined as a higher granularity of velocity change, for example:

Normal-mobility status criteria: - If the speed during time period T is less than N. Medium-mobility status criteria: - If speed during time period T is greater than or equal to N but less than or equal to N. High-mobility status criteria: - If speed during time period T is greater than N.

Proposal 2: The mobile IAB-node reports its real-time mobility status to IAB-donor CU.  Granularity of the reported velocity, frequency of reporting can be FFS.

## Broadcast System Information to Enhance Mobility

In RAN2 #119bis-e meeting, it was agreed that the mobile IAB-cell will broadcast 1 bit mobile-IAB cell type indication to assist mobility in IDLE/INACTIVE mode for Rel-18 UEs.

- RAN2 assumption: For the mobile IAB cell broadcasting info: - 1 bit mobile-IAB cell type indication is introduced, to assist mobility in Idle/Inactive mode for Rel-18 UEs (FFS if to assist UE to know it is onboard, if this need to be known) - FFS how this is used (might be implementation specific).

However, by only knowing the connected/camped cell is a mobile IAB-cell is not enough, as it cannot prioritize cell (re)selection towards a suitable cell for different scenarios.

For the mIAB served UEs, different types of target cell (i.e. gNB/stationary network or another mobile IAB-node) can be selected depends on the behaviour of the served UE(s). Following scenarios are considered need further optimization for cell (re)selection and handover of a Rel-18 UEs:

Scenario 1 (stationary network -> mobile IAB-node): The UE is moving inside the vehicle equipped with a mobile IAB-node

1) before moving inside the vehicle 2) after moving inside the vehicle

In this scenario, before the UE moves inside the vehicle (e.g. waiting in the bus station), the UE is originally connected/camped on a normal gNB/stationary network. When the vehicle is stopped and the UE gets onboard of the vehicle, the UE connects to the network via the mobile IAB-node equipped on the vehicle.

Scenario 2 (mobile IAB-node -> stationary network): The UE is moving out of the vehicle equipped with a mobile IAB-node

1) before moving out of the vehicle 2) after moving out of the vehicle

On the other hand, when the UE moves out of the vehicle (when the vehicle is stopped) that is equipped with a mobile IAB-node, the UE cannot connect to the mobile IAB-node anymore. The UE will reselect another cell (i.e. a normal gNB/stationary network) to remain its connection to the network.

Both scenarios 1 and 2 normally happens when the vehicle has stopped.

Observation 3: The UE moves inside/out of the vehicle and switches between mobile IAB-node and normal gNB when the vehicle (mIAB node) is stationary.

Scenario 3 (stay with one mobile IAB-node): The onboard UE is moving together with the vehicle equipped with a mobile IAB-node

In this scenario, the onboard UEs are moving together with the mobile IAB-node. During this period, there might be multiple normal gNBs/stationary network along the trajectory of this moving vehicle. Moreover, it is also possible that other moving vehicles are moving in the same direction with the one that the UE is onboard. The onboard UEs may be subject to handed-over among the normal gNBs/stationary network and/or other mobile IAB-nodes equipped on another moving vehicle based on the measurement report. However, such behavior is not optimal for the onboard UEs, as the connection towards other network (expect the onboarded mobile IAB-node) can only exist temporarily and can fade away in a short time when the vehicle moves away.

To reduce service interruption caused by frequent handover and avoid unnecessary handover/cell reselection for the on-board UEs, it is desirable that the onboard UEs always connect to the mobile IAB-node that it initially selected when the vehicle is moving. Therefore, the onboard UEs can maintain its connection to the network without being handover to another gNB/stationary network or mobile IAB-node equipped on another vehicle.

Observation 4: When the vehicle is moving and the UE is onboard, the UE should remain its connection to the mobile IAB-node and not perform handover/cell reselection towards other gNBs/stationary network or mobile IAB-nodes equipped on another vehicle.

Scenario 4 (mobile IAB-node -> mobile IAB-node): The onboard UE is moving inside the vehicle equipped with multiple mobile IAB-nodes

1) before UE’s mobility 2) after UE’s mobility

As captured in [1], some large vehicle, e.g. trains or trams, can be equipped with multiple mobile IAB-nodes to provide good coverage to onboard UEs in different parts of the vehicle. When the onboard UE moves from one carriage/wagon to another, the connection needs to be switched as well. Therefore, the onboard UE can also perform handover from one mIAB-node to another.

To allow the served UEs select the most suitable cell based on different mobility scenarios, above four scenarios should be considered when designing mobility enhancement for mobile IAB-node’s served UEs.

For RRC_CONNECTED UEs, mobility enhancement for above four scenarios can help to reduce service interruption and ping-pong handover of the served UEs, especially when they are moving together with the vehicle.

Mobility enhancement for RRC_IDLE and RRC_INCATIVE UEs for above four scenarios should also be considered to reduce power consumption of those UEs by avoiding performing additional measurements and cell reselection and potentially tracking area update signalling.

Proposal 3: Mobility enhancement of mobile IAB-node’s served RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE UEs for handover/cell (re-)selection should be considered for following four scenarios:

Scenario 1: The UE is moving inside the vehicle equipped with a mobile IAB-node

Scenario 2: The UE is moving out of the vehicle equipped with a mobile IAB-node

Scenario 3: The onboard UE is moving together with the vehicle equipped with a mobile IAB-node

Scenario 4: The onboard UE is moving inside the vehicle equipped with multiple mobile IAB-nodes

It is observed that the UE will switch between stationary network and mobile IAB-node in Scenario 1/2 when the vehicle is stopped. Moreover, when the vehicle is moving, the served UEs should remain connected to the original mobile IAB-node or switch to another mobile IAB-node based on measurements if it’s moving inside a vehicle. For Scenario 4, since the other mobile IAB-node are equipped on the same moving vehicle as the original mobile IAB-node the UE is connected to, the moving speed/direction of both mobile IAB-nodes are the same.

Therefore, based on the above analysis, the key information to differentiate UE behaviours for the 4 scenarios discussed above (e.g. on/off the vehicle, etc) and select suitable target cell for UE’s handover is based on whether the mobile IAB-node is moving or not (i.e. real-time mobility status of the mobile IAB-node). Broadcasting a mobile IAB-cell type is not enough for Rel-18 UEs for mobility enhancement.

For RRC_INACTIVE/IDLE UEs, if the velocity of a mobile IAB-cell is not zero, an RRC_IDLE/INACTIVE UE camped on a fixed gNB should not reselect a cell with mobile IAB-cell indication to camp on, as it is not going to be onboard when the vehicle is moving.  Similarly, an RRC_IDLE/INACTIVE UE camped on a mIAB node should not reselect a fixed gNB when the vehicle is moving.

For RRC_CONNECTED UEs, velocity information of one mobile IAB-node can also be used to enhance its measurement procedure, e.g. as new measurement event. By knowing mobile IAB-node’s velocity, if a new measurement event (e.g., mIAB velocity-based measurement report) is configured to the served UEs, the UE can then decide whether to send measurement reports by itself based on such information. For example, the served UE(s) only sends its measurement report of a fixed gNB to the network when its connected mobile IAB-node’s velocity is zero/lower than certain threshold/becomes in the low level.

Observation 5: Broadcasting mobile IAB-cell type in system information is not enough for Rel-18 UEs to enhance mobility and prioritize selection towards a mIAB-cell for the suitable scenarios such to prioritize cell (re)selection/handover towards a mIAB or fixed gNB based on when mIAB is temporarily stopped or not.

The velocity of mobile IAB-node can either be an accurate value (e.g. as defined in TS37.355) or a granular level (e.g. high/low).

Sending an accurate value of mobile IAB-node’s velocity may lead to an increase of SIB update frequency. Several mechanisms are possible to reduce the SIB update frequency.   For example, only update its SIB information if the velocity change is larger than certain threshold, use this mechanism and broadcast velocity only by vehicles that do not go through rapid start/stop etc. With that, mIAB served UEs can know the velocity of mIAB-node in real-time and understand the trend of its moving status.

Alternatively, to avoid SIB frequently update, paging can also be used to send those information to the UEs.

Observation 6: Solutions are possible to reduce SIB update frequency.

Proposal 4: Mobile IAB-node real-time mobility status (details FFS) is provided to the UE to enhance handover, cell (re-)selection decision.

For RRC_CONNECTED UEs, when they are moving together with the mobile IAB-node, TA of the served UEs are not changed. However, if a UE is getting on/off the vehicle, the TA of its own will become smaller/larger than before. Therefore, TA of the UE can also be used as reference to evaluate the UE’s moving behavior. Moreover, if the UE can estimate its relative speed to the mIAB based on doppler shift estimation, such information may also be useful for UE’s movement estimation.

With such information, the suitable cell can be selected for the served UEs for cell (re-)selection and/or handover.

Proposal 5: For Rel-18 UEs, consider new information/measurement event based on mIAB node’s velocity, TA, UE’s relative speed estimated by doppler shift, etc, as mobility enhancement for served UEs during their cell (re)selection, handover and conditional handover.

## Mobile IAB-node Topology Integration

In RAN2 #119bis-e meeting, it is FFS whether the network needs to indicate “supporting mIAB” in its system information:

- FFS if to Introduce that stationary network broadcasts indication of “supporting mobile-IAB” (into intended for the Mobile IAB MT)

Different from IAB-donor CU in Rel-16 and Rel-17, the IAB-donor CU is required to support full migration functionalities and mobility enhancements (e.g. group mobility, etc) for mobile IAB-node and served UEs. Hence, during mobile IAB-node integration to an IAB topology, it is necessary for mobile IAB-node to know which IAB-DU or IAB-donor DU can be selected as its parent IAB-node.

The IAB-DU or IAB-donor DU which can support direct integration of a mobile IAB-node (i.e. become the parent IAB-node of a mobile IAB-node) needs to provide the “mobile IAB-node support” indication (e.g. miab-Support) in its broadcast information, e.g. SIB1, so that the mobile IAB-node can select the suitable IAB-node and IAB-donor CU.

Proposal 6: The IAB-DU which can be the parent IAB-node of a mobile IAB-node broadcasts a “mobile IAB-node support” indication in its SIB1, indicating it supports mobile IAB-node integration.

## RACH-less Handover of served UEs

As agreed in RAN2 #119e meeting:

- R2 assumes RACH-less procedure may be considered for on-board RRC_CONNECTED UEs, which are to be handed over together with the mobile IAB-node (would depend also on the assumptions for UL synch).

When group handover is performed for all served UEs (e.g., due to inter-donor-CU full migration), the RRCReconfiguration messages are almost received at the same time after mobile IAB-node’s full migration. Considering the number of served UEs under a mobile IAB-node can be large, even with gradual HO of UEs with partial migration based full migration (i.e. “dual-DU-way” of doing full migration), it is possible that congestion may happen during served UEs’ handover.

Considering the served UEs are still connecting to the same mobile IAB-DU cell during full migration procedure, apart from the security key, most of the existing configuration such as Timing Advance, PHY configuration of the served UEs is not changed during mobile IAB-node’s migration. Therefore, the served UEs are UL synchronized with the mobile IAB-DU before and after its migration. A simplification of the HO procedure for the served UEs can be considered to reduce RACH and lower layer configuration.

Due to the change of CU, it is proposed to simply perform a change of security key for the served UEs. It can be done with only impacting user plane.

Normally, upon receiving RRCReconfiguration message during handover, the UE shall execute a reconfiguration with sync, reset MAC entity and re-establish L2. The UE shall also update the security key and use it after successful RACH towards the new cell. However, if RACH-less handover is adopted, there’s no way for the gNB to know when the UE starts to use the new security key.

In LTE, RACH-less is also supported. As captured in TS 36.300 [2], if RACH-less HO is configured, UE performs synchronisation to target eNB and derives target eNB specific keys, then configures the selected security algorithms to be used in the target cell. We think the same mechanism can be used for mIAB served UEs as a baseline, where the security key is updated upon receiving RRCReconfiguration message.

Proposal 7: During mIAB-MT handover, RACH- less HO with a change of security key of the served UEs should be supported. UE starts to use new security key for target cell after receiving RRCReconfiguration message.

## Pre-configuration/CHO for served UEs

Three options were discussed in RAN2 #119bis-e meeting as candidate solutions for pre-configuration/CHO for group of served UEs:

1) message withholding by the logical source IAB-DU with conditional delivery, e.g., upon on MT migration, 2) conditional execution by the UE based on, e.g., a broadcast indication such as SIB indication of service time or DCI indication of MT-migration, (includes CHO with new trigger). 3) legacy CHO (with implementation specific behaviour, e.g. using source-cell power down or target cell power up triggering the actual HO) - RAN2 assumes that O1 and O3 above could work, and FFS if O2 above (new trigger etc) is needed.

The main motivation of having RRC pre-configuration and/or CHO towards the mIAB served UEs is to reduce signaling congestion during group UEs’ handover. However, it is also agreed in RAN2 that two logical DUs in full migration use different physical cells. During online discussion, it was further clarified that the pre-configuration and CHO is configured to mIAB served UEs after the successful F1AP setup of second logical DU. At this time, there are two logical DUs exist simultaneously at the mobile IAB-node, while the served UEs are still connecting to the first logical DU. The IAB-donor CU can gradually handover the group of served UEs from the first logical DU to the second one. It will not lead to signaling congestion, as the signaling of group UEs can be spread over a certain period. Hence, there’s no need to consider pre-configuration or CHO for the mIAB served UEs.

Observation 7: Signalling congestion peak can be avoided by handover the group of UEs gradually from one logical DU to another. RRC pre-configuration or CHO also requires the same level of signalling is not helpful during group mobility of mIAB served UEs.

Proposal 8: RRC pre-configuration is not supported and CHO is not enhanced in Rel-18 mIAB.

# Conclusion

In this contribution, we discussed the mobility scenarios of the served UEs for mIAB and possible enhancements to mIAB to provide better HO and cell reselection performance for the legacy served UEs moving between mIAB and fixed gNB and also within the vehicle.  Possible enhancements for Rel-18 UEs were also discussed.

We have following observations and proposals:

Mobile IAB-node indication to Donor CU:

Proposal 1: Early mobile-IAB indication in Msg5 is not supported.

Observation 1: It is useful for the mobile IAB-node reports its real-time mobility status to IAB-donor CU to allow IAB-donor CU to decide a suitable type of target cell (mIAB or stationary normal cell) for the served UEs based on different UE mobility scenarios.  Network can either configure suitable measurement configuration(s) for the served UE(s) or ignore served UEs’ measurement reports based on mobile IAB-node’s velocity information, avoiding handover to unsuitable target cells.

Observation 2: mobilityState-r16 cannot be used to reflect the real-time moving status of a mobile IAB-node, because 1) it is not calculated after RRC_CONNECTED 2) it cannot represent a moving status change within a cell.

Proposal 2: The mobile IAB-node reports its real-time mobility status to IAB-donor CU.  Granularity of the reported velocity, frequency of reporting can be FFS.

Broadcast system information for mobility enhancement

Observation 3: The UE moves inside/out of the vehicle and switches between mobile IAB-node and normal gNB when the vehicle (mIAB node) is stationary.

Observation 4: When the vehicle is moving and the UE is onboard, the UE should remain its connection to the mobile IAB-node and not perform handover/cell reselection towards other gNBs/stationary network or mobile IAB-nodes equipped on another vehicle.

Proposal 3: Mobility enhancement of mobile IAB-node’s served RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE UEs for handover/cell (re-)selection should be considered for following four scenarios:

Scenario 1: The UE is moving inside the vehicle equipped with a mobile IAB-node

Scenario 2: The UE is moving out of the vehicle equipped with a mobile IAB-node

Scenario 3: The onboard UE is moving together with the vehicle equipped with a mobile IAB-node

Scenario 4: The onboard UE is moving inside the vehicle equipped with multiple mobile IAB-nodes

Observation 5: Broadcasting mobile IAB-cell type in system information is not enough for Rel-18 UEs to enhance mobility and prioritize selection towards a mIAB-cell for the suitable scenarios such to prioritize cell (re)selection/handover towards a mIAB or fixed gNB based on when mIAB is temporarily stopped or not.

Observation 6: Solutions are possible to reduce SIB update frequency.

Proposal 4: Mobile IAB-node real-time mobility status (details FFS) is provided to the UE to enhance handover, cell (re-)selection decision.

Proposal 5: For Rel-18 UEs, consider new information/measurement event based on mIAB node’s velocity, TA, UE’s relative speed estimated by doppler shift, etc, as mobility enhancement for served UEs during their cell (re)selection, handover and conditional handover.

Topology Integration

Proposal 6: The IAB-DU which can be the parent IAB-node of a mobile IAB-node broadcasts a “mobile IAB-node support” indication in its SIB1, indicating it supports mobile IAB-node integration.

RACH-less

Proposal 7: During mIAB-MT handover, RACH- less HO with a change of security key of the served UEs should be supported. UE starts to use new security key for target cell after receiving RRCReconfiguration message.

Pre-configuration/CHO

Observation 7: Signalling congestion peak can be avoided by handover the group of UEs gradually from one logical DU to another. RRC pre-configuration or CHO also requires the same level of signalling is not helpful during group mobility of mIAB served UEs.

Proposal 8: RRC pre-configuration is not supported and CHO is not enhanced in Rel-18 mIAB.

# References

- TR22.839, Study on Vehicle-Mounted Relays, Stage 1

- TS36.300, E-UTRA and E-UTRAN overall description, Stage 2

- TS38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state