3GPP TSG-RAN WG2 Meeting #120                                           　    　　　　　　 	　　　R2-2211466

Online, Nov 14th – Nov 18th 2022

Source: 	KDDI Corporation

Title:	Discussion on MAC partial reset

Tdoc Type:	Discussion

Agenda Item:	8.4.2.3　Cell Switch

Document for:	Discussion

Release:	Rel-18

Introduction

The following assumption was made in the last RAN2#119-bis-e meeting,

R2 assumes that at L1L2 cell switch: Whether the UE performs partial or full MAC reset (FFS what partial reset is, e.g. to avoid data loss), re-establish RLC, perform data recovery with PDCP is explicitly controlled by the network. R2 assumes that this can be configured by RRC. FFS if MAC CE indication(s) is/are needed.

In this contribution, we would like to discuss MAC reset in LTM to avoid data loss and achieve low latency mobility.

Discussion

MAC reset

When the MAC state cannot be maintained during cell switch, UE would need to perform MAC reset as described in TS38.321[1]. However, if we can obtain benefits such as low latency by maintaining MAC reset, partial MAC reset, e.g. maintain some timers, and procedures, can be performed intra-CU intra-DU and intra-CU inter-DU are two scenarios that is under discussion in RAN2. For both inter DU case and intra DU case, tight coordination among DUs is required to maintain MAC status after cell switch. With regards to inter-DU case, there are many aspects to consider because it is difficult and complicated to link and be implemented between DUs located in different sites. On the other hand, in intra-DU case, DUs reside in the same location and those DUs are more likely to be from same network vendors so it seems to be much easier to be implemented, thus we suggest to discuss the intra-DU scenario as the first step.

Proposal 1: RAN2 agree that MAC partial reset is supported for intra-DU scenarios.

In the last RAN2#119-bis-e meeting, MAC partial reset was discussed but we haven’t discussed which timers and procedures are maintained and which are canceled through MAC partial reset yet. Therefore, we need to discuss details.

Proposal 2: RAN2 discuss details of MAC partial reset, which timers and procedures are maintained and which are canceled through MAC partial reset

We describe MAC partial reset which timers and procedures are maintained. Among various timers and procedures, we think that maintaining Timing Advance related procedures work for latency reduction because maintaining these procedures omit uplink transmission timing synchronization and enables Rach less LTM.

Proposal 3: RAN2 agree that Timing Advance related procedures and timers and are maintained thorough MAC partial reset.

Further, we think that maintaining HARQ process during cell switch also bring some gain. In the case of HARQ process, in current MAC specification, the soft buffer is flushed and the received packets are discarded after cell switch indication. However, it is possible to combine the received packets before and after cell switch by maintaining the received packets without clearing soft buffer. It enables more efficient process. But it’s not RAN2 scope, so we propose to ask RAN1 its benefit and feasibility.

Proposal 4: RAN2 agree to ask RAN1 to evaluate its benefit and feasibility of avoid initializing HARQ process through MAC partial reset.

We describe procedures which can be canceled through MAC partial reset. It may lead to power saving of the user equipment by maintaining Power Head Room reporting procedure and Discontinuous Reception (DRX) related timers through MAC partial reset, but there is no significant advantage in terms of low latency. Therefore, in Rel-18, we think it is not necessary to maintain these procedures and timers.

Proposal 5: RAN2 agree not to maintain triggered Power Headroom Reporting procedure through MAC partial reset.

Proposal 6: RAN2 agree not to maintain Discontinuous Reception (DRX) related timers through MAC partial reset.

Conclusion

We introduce the following proposals.

Proposal 1: RAN2 agree that MAC partial reset is supported for intra-DU scenarios.

Proposal 2: RAN2 discuss details of MAC partial reset, which timers and procedures are maintained and which are canceled thorough MAC partial reset

Proposal 3: RAN2 agree that Timing Advance related procedures and timers and are maintained thorough MAC partial reset.

Proposal 4: RAN2 agree to ask RAN1 to evaluate its benefit and feasibility of avoid initializing HARQ process thorough MAC partial reset.

Proposal 5: RAN2 agree not to maintain triggered Timing Advance Reporting procedure thorough MAC partial reset.

Proposal 6: RAN2 agree not to maintain Discontinuous Reception (DRX) related timers thorough MAC partial reset.

Reference

[1] TS38.321 Medium Access Control (MAC) protocol specification