3GPP TSG-RAN WG2 Meeting #120		R2-2211479

# Toulouse, France, 14th Nov. – 18th Nov. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0576 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TS 38.300 for RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR includes some stage-2 corrections for RedCap: - In 9.2.7, the following wording is captured for RLF: For other DL BWPs, RLM can only be performed based on CSI-RS. It is not clear what “other DL BWPs” here means. According to the previous description, it seems that the other DL BWPs include the BWPs except “initial DL BWP and for DL BWPs containing the SSB associated to the initial DL BWP”. However, as we have agreed the SSB-based RLM could be also performed based on the NCD-SSB. Thus, the DL BWPs which are configured with the non-cell defining SSB should also be excluded. Hence we should clarify what the other BWPs include. The similar issue exists in 9.2.8. - In 9.2.8, as we agreed to remove the ”only” in the description for RLM: “SSB-based RLM is based on the SSB associated to the initial DL BWP and can only be configured for the initial DL BWP and for DL BWPs containing the SSB associated to the initial DL BWP”. Similarly, the wording “only” for BFD and BFR in 9.2.8 should also be removed. - In 9.2.10, it is not clear to describe as “UE starts monitoring paging DRX”. In our understanding, the actual meaning is that UE monitors paging or paging occasion according to the paging DRX cycle. Thus, the wording should be refined accordingly. - In 16.13.5, the wording” A RedCap UE monitors paging only in an initial BWP (default or RedCap specific) associated with CD-SSB” includes the case that UE in RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED. However, UEs in RRC_CONNECTED could monitor paging in the initial BWP or active BWP configured with paging common search space, not only in the initial BWP associated with CD-SSB. Thus, we think connected mode should be excluded here. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 9.2.7 and 9.2.8, clarify the other DL BWP for RLM/BFD - In 9.2.8, remove the ”only” for BFD in the description of SSB-based Beam Failure Detection is based on the SSB associated to the initial DL BWP and can only be configured for the initial DL BWPs and for DL BWPs containing the SSB associated to the initial DL BWP. - In 9.2.10, refine the wording that UE monitors paging according to paging DRX in PTW. - In 16.13.5, add the RRC states of idle and inactive for a RedCap UE on paging detection in intial BWP. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality RLM, BFD, Extended DRX, BWP operation Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no impact. - If the UE is implemented according to the CR and the network is not, there is no impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current description on RLM/BFD/eDRX/BWP operartion is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.7, 9.2.8, 9.2.10, 16.13.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 9.2.7	Radio Link Failure

In RRC_CONNECTED, the UE performs Radio Link Monitoring (RLM) in the active BWP based on reference signals (SSB/CSI-RS) and signal quality thresholds configured by the network. SSB-based RLM is based on the SSB associated to the initial DL BWP and can be configured for the initial DL BWP and for DL BWPs containing the SSB associated to the initial DL BWP. Besides, SSB-based RLM can be also performed based on the non-cell defining SSB , if configured for RedCap UEs. For other DL BWPs except the initial DL BWP, DL BWPs containing the SSB associated to the initial DL BWP and DL BWPs configured with the non-cell defining SSB, RLM can only be performed based on CSI-RS. In case of DAPS handover, the UE continues the detection of radio link failure at the source cell until the successful completion of the random access procedure to the target cell.

(text omitted)

Next change

### 9.2.8	Beam failure detection and recovery

For beam failure detection, the gNB configures the UE with beam failure detection reference signals (SSB or CSI-RS) and the UE declares beam failure when the number of beam failure instance indications from the physical layer reaches a configured threshold before a configured timer expires. For beam failure detection in multi-TRP operation, the gNB configures the UE with two sets of beam failure detection reference signals each associated with a TRP, and the UE declares beam failure for a TRP when the number of beam failure instance indications associated with the corresponding set of beam failure detection reference signals from the physical layer reaches a configured threshold before a configured timer expires.

SSB-based Beam Failure Detection is based on the SSB associated to the initial DL BWP and can be configured for the initial DL BWPs and for DL BWPs containing the SSB associated to the initial DL BWP. Besides, SSB-based Beam Failure Detection can be also performed based on the non-cell defining SSB, if configured for RedCap UEs. For other DL BWPs except the initial DL BWP, DL BWPs containing the SSB associated to the initial DL BWP and DL BWPs configured with the non-cell defining SSB, Beam Failure Detection can only be performed based on CSI-RS.

(text omitted)

Next change

### 9.2.10	Extended DRX for RRC_IDLE and RRC_INACTIVE

When extended DRX (eDRX) is used, the following applies:

-	For RRC_INACTIVE, eDRX configuration for RAN paging is decided and configured by NG-RAN. In RRC_INACTIVE the UE monitors both RAN and CN paging;

-	For RRC_IDLE, eDRX for CN paging is configured by upper layers. In RRC_IDLE the UE monitors only CN paging;

-	Information on whether eDRX for CN paging and RAN paging is allowed on the cell is provided separately in system information;

-	The maximum value of the eDRX cycle is 10485.76 seconds (2.91 hours) for RRC_IDLE and 10.24 seconds for RRC_INACTIVE, while the minimum value of the eDRX cycle is 2.56 seconds for both RRC_IDLE and RRC_INACTIVE;

-	The hyper SFN (H-SFN) is broadcast by the cell and increments by one when the SFN wraps around;

-	Paging Hyperframe (PH) refers to the H-SFN in which the UE starts monitoring paging according to DRX during a Paging Time Window (PTW) used in RRC_IDLE. The PH and PTW are determined based on a formula (see TS 38.304 [10]) that is known by the AMF, UE and NG-RAN;

-	H-SFN, PH and PTW are used if the eDRX cycle is greater than 10.24 seconds;

-	When the eDRX cycle is longer than the system information modification period, the UE verifies that stored system information remains valid before establishing an RRC connection.

Next change

### 16.13.5	BWP operation

A RedCap UE in RRC_IDLE or RRC_INACTIVE monitors paging only in an initial BWP (default or RedCap specific) associated with CD-SSB and performs cell (re-)selection and related measurements on the CD-SSB. If a RedCap-specific initial UL BWP is configured and NUL is selected, RedCap UEs in RRC_IDLE and RRC_INACTIVE shall use only the RedCap-specific initial UL BWP to perform RACH.

A RedCap UE may be configured with multiple NCD-SSBs provided that each BWP is configured with at most one SSB. NCD-SSB may be configured for a RedCap UE in RRC_CONNECTED to perform RLM, BFD, and RRM measurements when the active BWP does not contain CD-SSB.

End of change