3GPP TSG-RAN WG2 Meeting #120	 R2-2211658

Toulouse FR, 14th – 18th Nov 2022

Agenda Item:	8.6.1

Souce:	MediaTek Inc.

Title:	List of RAN2 Agreements in IoT-NTN

Document for:	    Discussion and decision

# 1 Introduction

Rel-18 enhancements for IoT-NTN Work Item was approved at RAN Plenary #96e [1]. During RAN2#119-e [2], some important agreements are made. Further agreements are made in RAN2#119bis-e [3]. As we have not decided to start making any Stage-2 running CR yet, this Tdoc is prepared to maintain the list of major RAN2 agreements.

# 2 RAN2 Progress and Agreement-List

## 2.1	Agreements made in RAN2#119bis-e

Agreements on “8.6.2 Performance Enhancements”

Agreements:

- For NB-IoT, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signaling (e.g. RRCConnectionSetup). This does not preclude other options (e.g. DCI-based). We can also revert this decision if requested by RAN1.

- Disabling HARQ feedback is supported for NB-IoT with single HARQ process, and it is up to eNB implementation whether to disable the HARQ feedback

- Working Assumption: Blind retransmission can be used in IoT NTN when HARQ feedback is disabled and when HARQ mode B is used (RAN2 assumes there is no spec change for this)

- HARQ mode A/B for uplink transmission may be configured per UL HARQ process at least via UE specific RRC signalling for eMTC and NB-IOT NTN. We can also revert this decision if requested by RAN1

- HARQ mode A/B for uplink transmission may be configured per UL HARQ process at least via UE specific RRC signalling for eMTC and NB-IOT NTN. We can also revert this decision if requested by RAN1

- RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process with DL HARQ feedback disabled, the UE will not start the corresponding DL HARQ RTT timer.

- For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception (can still check whether the alternative to set the HARQ RTT timer to 0 also works)

- RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process in HARQ mode B, the UE will not start the corresponding UL HARQ RTT timer.

- For NB-IoT NTN with single HARQ process in HARQ mode B, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission (can still check whether other alternatives also work)

- The solutions of LCP restriction on allowed HARQ mode in NR NTN can be reused for eMTC NTN.

Agreements on “8.6.3 Mobility Enhancements”

Agreements:

- For eMTC over NTN, for both earth-moving and earth-fixed cell scenarios, we introduce location based CHO triggering events.

## 2.2	Agreements made in RAN2#119-e

Agreements on “8.6.2 Performance Enhancements”

Agreements:

- Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability

- For UL HARQ operation, introduce two HARQ modes, i.e., HARQ mode A and HARQ mode B in IoT NTN (both NB-IoT and eMTC NTN), similarly to NR NTN

- From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT (and especially for CP solution for NB-IOT).

Agreements on “8.6.3 Mobility Enhancements”

Agreements:

- IoT NTN can use the mechanism for neighbor cell measurements in connected mode (specified in Rel-17 for NB-IoT). FFS if any enhancements are needed (e.g., triggers) for both NB-IoT and eMTC.

- RAN2 to continue working on a new time-based trigger for triggering intra and inter frequency measurements in connected mode, e.g., the serving cell is going to stop covering the current area, for both earth-moving and earth-fixed cell (FFS on distance-based trigger)

- CHO enhancements for eMTC NTN (i.e., time/timer based solution) are introduced based on the R17 NR NTN solution. FFS on location-based solution

- Measurement results reporting is not supported in Rel-18 NB-IoT NTN.

# 3 References

- RP-221554, MediaTek, WID on IoT NTN Enhancements, RAN#96e, June 2022.

- R2-2208701, Vice Chairman (ZTE Corporation), Report from Break-out session on NR-NTN, IoT-NTN, REDCAP and CE, 3GPP TSG-RAN WG2 Meeting #119 electronic, August 2022.

- R2-2210801, Vice Chairman (ZTE Corporation), Report from Break-out session on NR-NTN and IoT-NTN, 3GPP TSG-RAN WG2 Meeting #119bis electronic, October 2022