3GPP TSG-RAN WG2 Meeting #120	R2-2211722

Toulouse, France, 14 – 18 November 2022

Agenda item:	6.5.3

Source:	Apple

Title:	Discussion on CG timer aspects

WID/SID:	NR_IIOT_URLLC_enh-Core

Document for:	Discussion / Decision

# Introduction

This contribution discusses the start/stop of the configuredGrantTimer in a scenario where an uplink grant is received for MAC entity's C-RNTI and the identified HARQ process is configured for a configured uplink grant.

# Discussion

## The case of a DG overruling a CG

In Licensed, HARQ processes are not shared among different configured grants on the same BWP. However, HARQ process sharing between dynamic grant (DG) and configured grant (CG) may still happen such that a DG may override a CG using the same HARQ process. The HARQ process ID for the DG is allocated by the network via the DCI, the HARQ process ID for the CG is based on a formula in the MAC spec. A CG configuration may comprise multiple CG PUSCHs with the same HARQ process ID.

A configuredGrantTimer should start on the first OFDM symbol of the PUSCH when a PUSCH is transmitted. The timer is associating to the HARQ process of the PUSCH, which may be a CG or a DG.

In a scenario where a DG overrules a CG, the current NR MAC behaviour ensures that the previous uplink grant (i.e., a CG) is considered positively acknowledged. Further, the UE starts or restarts the configuredGrantTimer to protect the HARQ process from being overwritten by a MAC PDU generated for any new data that arrives for the CG while the DG is in progress.

TS 38.321, clause 5.4.1 If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer or a running cg-SDT-TimeAlignmentTimer and for each grant received for this PDCCH occasion: 1> if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or 1> if an uplink grant has been received in a Random Access Response: 2> if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant: 3> consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI. 2> if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant: 3> start or restart the configuredGrantTimer for the corresponding HARQ process, if configured; 3> stop the cg-RetransmissionTimer for the corresponding HARQ process, if running. 2> stop the cg-SDT-RetransmissionTimer for the corresponding HARQ process, if running. 2> deliver the uplink grant and the associated HARQ information to the HARQ entity.

However, the DG may block the HARQ process unnecessarily long, building up additional delay for the logical channels mapped to the CG. This is illustrated in the following two figures below.

Both cases show the same scenario. In figure 1 (case A), the DG associating to HARQ PID 0 is received when a CG configured for the same HARQ PID is due. It can be seen that the DG blocks the HARQ process of the CG. Note that the point in time when the DG was received is not shown in the figure (as the CG timer runs to completion).

Figure 1: DG blocking a HARQ process of a CG - Case A

In figure 2 (case B), the DG associating to HARQ PID 0 is received slightly earlier, even before the CG configured for the same HARQ PID. Thus, the DG blocks the HARQ process of the CG for an even longer time, which further increases the latency of any data available for transmission on the logical channels mapped to the CG.

Figure 2: DG blocking a HARQ process of a CG - Case B

The scenario applies when the configuredGrantTimer was initially started by a dynamic grant received on the PDCCH for the MAC entity's C-RNTI and a HARQ-ACK is received for the dynamic grant. Since the CG timer got started to protect the HARQ process from being overwritten by a different MAC PDU while the DG is in progress, it makes no sense to continue running the CG timer when the DG has received a positive acknowledgement. However, a HARQ-ACK for the dynamic grant occurs if the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process.

## Clarification of the CG Timer for start/stop

To tackle the problem shown in the previous section we think that the UE can stop the CG timer for the HARQ process associated with the DG-PUSCH upon HARQ-ACK. This allows the next CG to use the HARQ process immediately after the DG was transmitted successfully (assuming the next DG is not conditioned to overrule the CG again).

The 38.321 specifications of Rel-16 and Rel-17 already include the following paragraph:

If a HARQ process receives downlink feedback information, the HARQ process shall: 1> stop the cg-RetransmissionTimer, if running; 1> if acknowledgement is indicated: 2> stop the configuredGrantTimer, if running.

Since “Downlink Feedback Information” is received in CG-DFI the above condition can be understood that it applies to shared spectrum only. Indeed the paragraph was introduced for NR-U with CR 694 (from Rel-16 onwards, TS 38.321 version g00).

When the UE has received HARQ feedback on the PDCCH for the earlier DG from the NDI, it seems reasonable to stop the configuredGrantTimer upon a positive HARQ acknowledgement not just for shared spectrum but in licensed spectrum as well.

Proposal 1: Clarify that in licensed spectrum, a CG timer associated with a HARQ process started by a DG-PUSCH is stopped upon HARQ feedback.

We further note that stopping the CG timer upon HARQ feedback alone may not be sufficient in all cases since a HARQ-ACK for a DG comes from a toggled NDI. The NDI is a DCI field within a new DG, so if the NDI has been toggled it means a DG is received for new data. In such cases, the UE should proceed to perform new transmission on this new DG (the PUSCH of which will start the CG timer again).

Therefore, one should also consider the case when the next DG (for the same HARQ process) is not eligible to be used according to the LCP mapping restrictions, or where the MAC entity does not have data available to be multiplexed. For example, the allowedPHY-PriorityIndex could be set to p0 while the DG is for p1 and there are no LCHs allowed to use such DG or there are no LCHs with data available for p1, or another LCP restriction disallows the usage of the next uplink grant.

Proposal 2: Clarify that the UE starts the CG timer, if logical channels that have data available that can be multiplexed in the MAC PDU can be transmitted on the uplink grant according to the LCP mapping restrictions configured by the network.

We think that the two aspects are not clearly captured in the currext MAC specification. Therefore, we would like to propose a clarification to capture this case in the NR MAC specification.

Proposal 3: RAN2 clarifies the CG timer behavior in the MAC specification. R2-2211723 is used as a baseline.

A CR to correct the MAC layer behavior is provided in [2].

# Conclusions

This contribution discusses conditions leading to the stat/stop of the configuredGrantTimer and proposes a clarification. We have the following proposals:

Proposal 1: Clarify that in licensed spectrum, a CG timer associated with a HARQ process started by a DG-PUSCH is stopped upon HARQ feedback.

Proposal 2: Clarify that the UE starts the CG timer, if logical channels that have data available that can be multiplexed in the MAC PDU can be transmitted on the uplink grant according to the LCP mapping restrictions configured by the network.

Proposal 3: RAN2 clarifies the CG timer behavior in the MAC specification. R2-2211723 is used as a baseline.

# References

[1] TS 38.321 version 17.2.0, September 2022

[2] R2-2211723, Clarification for a DG overruling a CG, Apple, RAN2#120