3GPP TSG-RAN WG2 Meeting #120	R2-2211807

Toulouse, France, November 14-18, 2022

Agenda Item:		6.10.3

Source:				ASUSTeK

Title:		Clarification on NTN configuration for handover

Document for: 	Discussion and Decision

- Introduction

In the last meeting, it’s agreed that the UE uses NTN-config in NTN-NeighCellConfig from source cell SIB19 for HO or CHO up to implementation and it is FFS on spec impact. In this contribution, we further discuss the spec impact of this agreement.

Agreements online:

- Whether the UE uses the target cell NTN-config in NTN-NeighCellConfig-r17 IE from source cell SIB19 for HO or CHO is up to UE implementation (FFS on spec impact)

- It’s up to UE implementation what to do with T430 when going to IDLE.

- Discussion

As described in [1] and the offline discussion in the last meeting [2], the NTN-config provided in handover command may be invalid at the time of CHO execution. And it was agreed that “whether the UE uses the target cell NTN-config in NTN-NeighCellConfig-r17 IE from source cell SIB19 for HO or CHO is up to UE implementation.”

However, based on current text as below [3], the UE can only apply NTN-Config from reconfigurationWithSync for HO or CHO and cannot have the implementation to use NTN-config in NTN-NeighCellConfig from source cell SIB19. The UE may not apply available NTN-Config from source cell SIB19 for HO or CHO.

##### 5.3.5.5.2 Reconfiguration with sync The UE shall perform the following actions to execute a reconfiguration with sync. 1> stop timer T430 if running; 1> start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime, if included in the reconfigurationWithSync for target SpCell; 1> if the AS security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

Observation 1: Based on current text in the section 5.3.5.5.2 of TS 38.331, the UE can only apply NTN-Config from reconfigurationWithSync and cannot have the implementation to use NTN-config in NTN-NeighCellConfig from source cell SIB19.

If the UE does not use proper NTN-Config for the target cell, T430 may expire during the HO or CHO procedure. To reflect the agreement to allow the UE implementation of using target cell NTN-config in NTN-NeighCellConfig-r17 IE from source cell SIB19 for HO or CHO, it is proposed to add a note in the RRC reconfiguration procedure to clarify that the UE may use NTN-Config in NTN-NeighCellConfig from source SpCell SIB19.

Proposal 1: Adopt below text proposal for section 5.3.5.5.2 of TS 38.331 with a note to clarify that the UE may use NTN-Config in NTN-NeighCellConfig from source SpCell SIB19.

##### 5.3.5.5.2 Reconfiguration with sync The UE shall perform the following actions to execute a reconfiguration with sync. 1> stop timer T430 if running; 1> start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime, if included in the reconfigurationWithSync for target SpCell; NOTE: The UE may use ntn-UlSyncValidityDuration and epochTime in NTN-NeighCellConfig for target SpCell from source SpCell SIB19. 1> if the AS security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

- Conclusion

In this contribution, we discuss the agreement made in the last meeting and have following observation and proposal:

Observation 1: Based on current text in the section 5.3.5.5.2 of TS 38.331, the UE can only apply NTN-Config from reconfigurationWithSync and cannot have the implementation to use NTN-config in NTN-NeighCellConfig from source cell SIB19.

Proposal 1: Adopt below text proposal for section 5.3.5.5.2 of TS 38.331 with a note to clarify that the UE may use NTN-Config in NTN-NeighCellConfig from source SpCell SIB19.

##### 5.3.5.5.2 Reconfiguration with sync The UE shall perform the following actions to execute a reconfiguration with sync. 1> stop timer T430 if running; 1> start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime, if included in the reconfigurationWithSync for target SpCell; NOTE: The UE may use ntn-UlSyncValidityDuration and epochTime in NTN-NeighCellConfig for target SpCell from source SpCell SIB19. 1> if the AS security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

- Reference

- R2-2210729, “NTN Configuration at Handover and CHO.”

- R2-2210855, “[offline-113] epoch time and validity timer.”

- R2-2211018, “Corrections for Release-17 NR NTN.”