3GPP TSG-RAN WG2 Meeting #120	R2-2211821

Toulouse, France, Nov 14-18, 2022

Agenda Item:	8.9.2

Source:	NEC

Title:	UE to UE relay discovery and (re)selection

Document for:	Discussion, Decision

Introduction

The following proposals were postponed from the last meeting during the discussion of UE to UE relay discovery and (re)selection based on the email discussion summary within R2-2210981.

Proposal 4.1: [14/20]	Whether there is a need for an indication of whether gNB is capable of U2U relay can be discussed following further design of U2U relay.

Proposal 2.1: [15/20] Discovery message transmission at the relay UE is conditioned on at least upper layer indication and channel measured from discovery received from the remote UE (i.e. Model B).

Proposal 2.2: [8/20] RAN2 further discuss whether conditions/content of the neighbour UE list is used to determine whether a relay UE transmits the discovery message.

Proposal 3.1: [13/20] At least a) channel quality between source remote UE and relay and b) channel quality between the relay and destination remote UE are used for selection of the relay.  FFS on other criteria.

We plan to further discuss the discovery message transmission and the criteria for relay selection within this document.

Discussion

Discovery message transmission

The following conditions were discussed in the last meeting to allow transmission of the discovery message at the Relay UE:

a)	Upper layer

b)	Channel quality between remote UE and relay UE

c)	Conditions on the neighbor list at the relay UE

d)	Conditions on the contents of discovery received by another relay UE

Our thinking is to adopt the same principle for U2N Relay when appropriate. For U2N relay case, the upper layer can trigger the NR sidelink discovery message transmission. From AS layer perspective, the channel quality for U2N relay UE should be lower than an upper bound Uu RSRP threshold and above a lower bound Uu RSRP threshold. As a baseline, we did not see a reason to deviate from the principle of U2N Relay for U2U discovery message transmission.

Essentially, the final decision on whether to transmit the discovery message is decided by the upper layer. But the AS layer can inform the radio channel quality to the upper layer based on the result of relay reselection.

Specific to the case of UE to UE relay, for discovery Model B, where the source Remote UE transmits discovery message to find candidate Relay UE and target UE, the Relay UE can forward the discovery message. In this case, the AS condition (e.g., the radio channel quality) may be applied. In addition, the neighbor list at the Relay UE may be also checked as one condition for such discovery message forwarding at Relay UE, otherwise the discovery message transmission may be not useful. Then we make the following proposals:

Proposal 1:	Discovery message transmission at the Relay UE is decided by upper layer based on the input of the radio channel quality between the Remote UE and the Relay UE (i.e. model B).

Proposal 2:	The neighbour UE list is also used to determine whether a relay UE transmits the discovery message.

Criteria for relay selection

The following criteria were discussed in the last meeting to be used to determine which relay(s) can be selected by the Remote UE once (re)selection is triggered.

a)	Channel quality between the remote UE and the relay

b)	Channel quality between the relay and the destination (second hop)

c)	Relay load

d)	Whether PC5 link of the second hop is already established or not

e)	PLMN ID

f)	Cell ID/gNB

g)	Prioritization of the direct link over the relayed link

We do not see a strong motivation deviate the design from U2N Relay (re)selection criteria for U2U, then we think the channel quality between the remote UE and the relay should be the baseline for U2U Relay (re)selection. Specific to U2U case, the remote UE should be able to have access to the channel quality of the second hop in order to make a reasonable decision. Then we make the following proposals:

Proposal 3:	As a baseline, the channel quality between each remote UE and the relay UE are used as criteria for relay (re)selection.

Proposal 4:	The channel quality between the relay and the destination (second hop) are also used as criteria for relay (re)selection for Remote UE.

Conclusion and Proposal

We have the following proposals:

Proposal 1:	Discovery message transmission at the Relay UE is decided by upper layer based on the input of the radio channel quality between the Remote UE and the Relay UE (i.e. model B).

Proposal 2:	The neighbour UE list is also used to determine whether a relay UE transmits the discovery message.

Proposal 3:	As a baseline, the channel quality between each remote UE and the relay UE are used as criteria for relay (re)selection.

Proposal 4:	The channel quality between the relay and the destination (second hop) are also used as criteria for relay (re)selection for Remote UE.

Reference

- RP-213585, WID on NR sidelink relay enhancements

- R2-2210981	Summary of [AT119bis-e][427][Relay] Remaining proposals on UE-to-UE relay