3GPP TSG-RAN WG2 Meeting #120  	                            R2-2211921

Toulouse, France, November 14 – November 18, 2022

Agenda Item:	8.3.5

Source: 	Sony

Title:	Handover enhancement for NES

Document for:	Discussion

# Introduction

In RAN2#119e meeting, the grouping of candidate solutions have been discussed and we have the groupings as followed.

Solution groups:

- Adaption of MIB/SSB/SIB

-  partial/simplified SSB

2	Increase of SSB/SIB periodicity

3	On demand SSB/SIB1 (FFS if there are enhancements for other SIBs)

- FFS for on-demand MIB

4	Receiving SSB/SIB on one carrier/cell and performing access to another carrier/cell

5	Handover/Fast PCell change for NES

- CHO or new configuration

- group HO

6	Resource adaptation (frequency and time domain)

- Including PRACH, SRS, PUSCH, PUCCH resources and periodicities

- cell DTX/DRX

- measurement

- reference signal type and configuration of reference signal pattern for connected mode

- BWP adaptation

7	Any Cell activation/re-activation or UE wake up request signal (connected/idle)

8	Paging enhancements (includes paging-less solutions)

9	Cell selection/reselection (ie. cell prioritization also including legacy UEs)

Things to study

- Study group configuration and signalling for transitions for different solutions

- pre-configuration and L1/L2 signaling to trigger change of configuration

2	Identify/capture RAN2 impact to legacy for the different solutions

3	Awareness of the NES states at the UE side for the different solutions

4	Aim to minimize DL signalling for NES

5	Consider UE complexity and energy consumption

6	UE assistance information for the specific network energy technique, it’s benefits and impact to UE/NW

In RAN2#119bis-e meeting, due to the limit of time, mobility enhancement has not been extensively discussed. In this contribution, we will focus on the solution#5 and address the open issues/concerns on this solution and a TP draft has been provided in the Annex.

# Discussion

2.1 Solution#5

Group-common signalling HO

Our understanding of this solution is:

- UEs will be grouped and pre-configured with target candidate cell(s) configurations.

- Network will trigger the UE to perform the handover via a group-common HO signalling and this could be a multicast signalling or a L1/L2 signalling.

On the grouping of UEs, network can group UEs according to their measurement reports or any other feedback if deemed necessary e.g. mobility states, velocity, trajectory, capability so that UEs with the same characteristics can be grouped together. On the trigger from the network, it could be based on the NES state switching or a load balancing requirements, in order to achieve some energy saving gains.

We think with group-common HO, we can achieve the benefits as follows:

- A group-common HO can save HO overhead signalling.

- Combined with L1/L2 signalling can further inherit the benefits from L1/L2 mobility.

- Grouping of the UEs can provide network with finer granularity on handover control as well as energy saving control

On the concerns expressed by some companies, our understanding is as follows.

- Signalling overhead reduction

We need to discuss what configurations can be included in the group-common HO signalling. If it is just a trigger and no more dedicated signalling is needed, we believe this is beneficial to reduce HO signalling overhead. But if the group-common signalling, as previously discussed during Rel-17 NTN, will include some handover configuration and a further dedicated signalling is still needed, we think the overhead reduction could be marginal. We need to address this issue during further discussions.

2. Acknowledgement of group-common signalling

If the group-common signalling is based on L1/L2 signalling, we think a L2 feedback can address the issue.

Based on the discussion above, we think there are still some open issues to discuss but this is not the reason to not go ahead on further discussion on this solution. So, we proposal

Proposal 1: RAN2 to further discuss the group-common HO signalling in Solution#5.

NES-aware CHO

As proposed by the proponents, NES-aware CHO will define new CHO triggers based on the cell NES state. We are open to further discuss this solution but believe that such triggers will be applicable to group-common HO as well.

In RAN2#119bis meeting, we reached common understanding as follows.

=> Scenario 1: UEs are HO’ed due to switch of SOURCE cell to NES mode is considered for further study. FFS whether any enhancements is needed. => FFS Scenario 2: UEs are HO’ed due to source link degradation, where TARGET cell is selected based on its mode of operation => As a first priority, discussion on RAN2 group handover are confined to the CHO framework

In scenario 1, UE will perform handover due to the NES mode has changed e.g. UE will need to handover due to the NES cell is going to be in sleep mode. In this case, it is natural to introduce a condition for handover execution e.g. NES cell mode change. UEs that are configured with this condition will perform handover when it is triggered. It is still FFS, how the UE will be informed of this cell mode change.

Proposal 2: RAN2 to further discuss the NES-aware CHO in Solution#5.

# Conclusion

We propose RAN2 to discuss the proposals as follows.

Proposal 1: RAN2 to further discuss the group-common HO signalling in Solution#5.

Proposal 2: RAN2 to further discuss the NES-aware CHO in Solution#5.

# Annex

-----------------------------------------------------------------TR 38.864------------------------------------------------------------------

### 6.1.X	Connected mode mobility enhancements

#### 6.1.x.x	Higher layer procedures

For connected mode mobility, the handover enhancement where UEs perform handover due to switching off the source cell to NES mode are studied. Group handover, where a group of UEs will perform handover simultaneously due to the source cell switching to NES mode, is one technique that is found to be useful.

On the grouping of UEs, network can group UEs according to their measurement reports or any other feedback if deemed necessary e.g. mobility states, velocity, trajectory, UE capability so that UEs with the same characteristics can be grouped together. On the trigger from the network, it could be based on the NES state switching or a load balancing requirement, in order to achieve some energy saving gains.

Group HO is beneficial if it implies just a trigger and no more dedicated signalling is needed. But if the group-common signalling, as previously discussed during Rel-17 NTN, will include part of handover configuration and a further dedicated signalling is still needed, then the overhead reduction could be marginal.

L1/L2 signalling based mobility is beneficial to further reduce the signalling overhead as well as accelerate the handover execution.

NES-aware CHO which will define new CHO triggers based on the cell NES state is also beneficial.

#### 6.1.x.y	Assistance information from UE side

Not relevant.

Editor's note: will be updated once more agreements are made.

#### 6.1.x.z	Impacts on network interfaces

Editor's note: will be updated once more agreements are made.

-----------------------------------------------------------------TR 38.864------------------------------------------------------------------