3GPP TSG RAN WG2 Meeting #120               	R2-2211923

# Toulouse, France, 14th 18th November 2022

Agenda Item: 	8.5.2.2

Source:		Sony

Title:			Considerations on XR PDU prioritization

Document for:	Discussion & Decision

# Introduction

A new Rel-18 study item was approved at RANP#94 in December 2021, including the following objectives. The study is to be based on RAN1, Rel-17 TR 38.838 [2], SA4 work in SP-210043 [3], and SA2 work as per SP-220705 [4].

The study is to be based on Rel-17 TR 38.838, on corresponding Rel-17 work from SA4 (as per SP-210043) and on Rel-18 work from SA2 (as per SP-220705). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; Dynamic scheduling/grant enhancements

At RAN2#119e meeting [7], the following agreements were made:

- RAN2 should take SA2/SA4 work into account - RAN2 assumes that PDU Set based parameters and PDU Set related information may be used for better support of XR services. RAN2 can consider both UL and DL directions. - RAN2 will study PDU Set based parameters and PDU Set related information handling in Network and UE - RAN2 to adopt the current SA2 definition of PDU Set as an application media unit as working assumption, subjected to further guidance from SA2 and SA4. - XR awareness discussion in RAN2 should consider PDU set characteristics and how to use the information available on those (for UL and/or DL). Can also consider how to handle data bursts. - RAN2 can study e.g. periodicity, arrival time, jitter and frame-size variations for XR awareness to enable power savings and capacity enhancements. Can study also how often such parameters change (i.e. how dynamic they are). - RAN2 can consider how PDU sets can be mapped to DRBs (FFS if SA2 discussion on PDU set mapping to QoS (sub-)flows impacts this)

At RAN2#119bis meeting [8], the following agreements regarding XR awareness were made:

PDU sets:

From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL:

Semi-static information (from CN to RAN): At least PSER and PSDB.

Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries.

Discuss whether to send LS on AS re-ordering once we discuss bearer handling.

Capture the models 1a/b, 2a/b in TR and indicate what is possible in current specifications and how. FFS how LCH options work in each case

1. 	SDAP maps each data packet in a PDU set to a single PDCP SDU, as in legacy (i.e. each PDU is only mapped to a single SDU).

3.	HARQ and RLC re-/transmissions for XR traffic are done as in legacy (i.e. they are not based on XR PDU sets).

PDU prioritization;

No agreements

PDU discard;

1. For UE transmitter, the PDCP discard should be performed per PDU set basis.

2. For UE transmitter, The PDCP discard is managed per SDU for PDU set, the PDCP entity discards all PDCP SDUs associated with the PDU set.

Wait for SA2/SA4 to clarify this. No need for LS at this point.

Keep discard description at XR awareness (we assume it may impact both capacity and power saving). Can re-consider based on SI progress. Discuss in the next meeting how we handle overlaps between XR awareness and capacity/power saving enhancements.

In this contribution, we discuss potential solutions for XR awareness, particularly related to awareness on PDU set information and PDU prioritization. , We propose that RAN shall be informed about different reliability levels of PDUs within a PDU set and that RAN can be informed about PDUs containing application layer FEC redundancy information..

# Discussions on XR awarness, PDU prioritization

## Background

In SA2, the study FS_XRM is ongoing, TS 23.700-60 [5], including KI#4 (PDU Set integrated packet handling) and KI#5 (Differentiated PDU Set Handling). In KI#4 it is discussed what information and how the application will signal this to the Core Network, and also what information to forward to RAN and to the UE respectively. In KI#5 the PDU set is defined including how the importance information is signalled, enabling differentiated PDU Set handling, and to which entity it is sent.

This SA2 study in Rel-18 is ongoing, they are still in the study phase discussing the evaluation of proposed solution which will be finalized in conclusions. Theconclusions will soon be made regarding which proposals that are agreed and therefore will be specified in normative stage-2 specifications.

SA2 has agreed the following definition of a PDU Set

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g., a frame or video slice for XRM Services), which are of same importance requirement at application layer. All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In some cases, the application layer can still recover parts of the information unit, when some PDUs are missing.

## Discussion on Priority of PDUs within a PDU set.

XR transmission is typically involving a video transmission. A video transmission, particularly a video transmission with high quality, may require large packet size. A large packet size is typically divided into sub-packets or PDUs, belonging to a PDU set. Each sub-packet is transmitted from the source application over the internet via the UPF and the RAN connection to the application in the UE.

Assuming a PDU Set contains the same type of content/importance, then typically for XRM service, a PDU Set needs to be delivered as a whole and if some pieces are not delivered in time, then the complete PDU Set should be dropped. i.e. if not all PDU´s within the PDU set are delivered, then the packets delivered successfully, are transmitted in vain.

Observation 1: In order to not waste radio resources, if the initial packets in a PDU set are delivered successfully, the delivery of the remaining of the PDUs should be delivered with increasingly priority, in order to not have wasted the initial packets, if not the remaining packets were delivered.

Hence from a radio resource/bandwidth perspective, the last PDU´s within a PDU set should have higher importance compared to the first or initial PDU´s within the same PDU Set.

If the PDU Set contains multiple sub-types of flows such as video, audio, haptic data or sensor data as defined in some solutions then the same is valid for each of the sub-types.

Observation 2: In case there are different types of PDUs the different types shall be handled separately with different priorities.

During normal transmission the NG-RAN calculates the needed modulation or coding rate to achieve a specific BLER (Block Error Rate) to fulfill the desired QoS of the PDU. The existing solutions expect the QoS to be equal for all PDUs with the same importance within the PDU Set.

There is a need to improve the resource efficiency in 5GS, e.g. in NG-RAN by allowing NG-RAN to be aware of where within the PDU Set it transmit PDUs in order for NG-RAN/gNB to be able to adjust the reliability level within the PDU Set so that the reliability is less in the beginning compared to the end in the same PDU Set.

Proposal 1: NG-RAN/gNB should be informed about potential differentiation of reliability levels within a PDU set from Core Network nodes, including AMF and UPF.

Depending on solution, this may be mainly to be discussed by SA2 and RAN3 for CN-gNB signalling, in the last RAN Plenary it was discussed that RAN2-SA2 coordination is needed. Potential impact for RAN2, may be related to new ways to handle QoS and that RAN needs to be aware of the PDU set since RAN should be able to handle different reliability in the beginning and end of the PDU Set.

Observation 3: RAN needs to be aware of the PDU set and that RAN2 is notified if any RAN2 impact is identified by SA2.

## Prioritization of Application Layer FEC redundancy bits

In SA2 and SA4 it is discussed that the application data for the streamed media in a PDU Set may be error correction encoded by a FEC on the application layer, similar as in MBS using the FLUTE protocol, this is e.g described in TS 26.346 [6]. The application layer FEC redundancy bits are according to [6] transmitted separated from the information bits from the application to the UPF.

The redundancy bits are only needed in the UE in case there are errors in the message after it has been received by the application layer in the UE. In case the Information part of the PDU Set is received without any errors the redundancy bits are not needed and thereby it is not necessary to send them over the Uu interface. Therefore in case the information about where in the PDU set the FEC bits are transmitted is known to RAN, RAN could check if the whole PDU Set is correctly received on RAN level. In that case the FEC bits are not needed and thereby the load on the Uu interface can be reduced without any degradation.

The prioritization of the FEC redundancy bits depends thereby on how successful the transmission of the information bits is. In case the redundancy bits e.g. are sent in a different QoS flow from the UPF, that can be used to detect and delay this part of the PDU Set until the other bits are transmitted

Observation 4: In case the application layer uses FEC error correction and it is known by RAN where in the PDU set it is sent it is possible for RAN to reduce the load on the Uu interface by not transmitting the FEC redundancy bits.

Proposal 2: Send LS to SA2 that RAN2 is interested in information regarding the position of the FEC redundancy bits.

# Summary

In this contribution, we have discussed our view on potential XRxxxx in NR. Our observation and proposals are listed below:

Observation 1: In order to not waste radio resources, if the initial packets in a PDU set are delivered successfully, the delivery of the remaining of the PDUs should be delivered with increasingly priority, in order to not have wasted the initial packets, if not the remaining packets were delivered.

Observation 2: In case there are different types of PDUs the different types shall be handled separately with different priorities.

Proposal 1: NG-RAN/gNB should be informed about potential differentiation of reliability levels within a PDU set from Core Network nodes, including AMF and UPF

Observation 3: : RAN needs to be aware of the PDU set and that RAN2 is notified if any RAN2 impact is identified by SA2.

Observation 4: In case the application layer uses FEC error correction and it is known by RAN where in the PDU set it is sent it is possible for RAN to reduce the load on the Uu interface by not transmitting the FEC redundancy bits.

Proposal 2: Send LS to SA2 that RAN2 is interested in information regarding the position of the FEC redundancy bits.

# References

- RP-213587, “Study on XR Enhancements for NR””, RANP #94e, December 2021

- TR38.838,

- SP-210043, Revised SID on Feasibility Study of Typical Traffic Characteristics for XR Services and other Media, SA4

- SP-220705, SID update on Study on architecture enhancement for XR and media services, SA2

- TR 23.700-60, SA2, Study on XR (Extended Reality) and media services

- TS 26.346, Multimedia Broadcast/Multicast Service (MBMS)Protocols and codecs

- Minutes RAN2#119e meeting

- Minutes RAN2#119bis-e meeting