3GPP TSG-RAN WG2 Meeting #120	R2-2211950

Toulouse, France, November 14 – 18, 2022

Agenda Item:	8.15.2

Source: 	Xiaomi

Title:  	Discussion on LBT for sidelink operation on unlicensed spectrum

Document for:	Discussion and Decision

1	Introduction

In last RAN2 meeting, RAN2 discussed the consistent LBT for SL-U and achieved some agreements [1].

In this contribution, we would like to discuss some remaining issues related to LBT for sidelink operation on unlicensed spectrum and have corresponding proposals.

2	Discussion

Transmission type for LBT

After standardization of LTE-LAA, eLAA and feLAA, 3GPP had discussed to support New Radio over Unlicensed (NR-U) frequency bands Since Rel-16. As unlicensed frequencies are also be used by other networks, like WiFi, both gNB and UEs in NR-U need to perform Listen-Before-Talk (LBT) for every transmission. In the current MAC specification, it is stated that:

The lower layer may perform an LBT procedure, see TS 37.213 [18], according to which a transmission is not performed by lower layers if the channel is identified as being occupied. When lower layer performs an LBT procedure before a transmission and the transmission is not performed, an LBT failure indication is sent to the MAC entity from lower layers. Unless otherwise specified, when LBT procedure is performed for a transmission, actions as specified in this specification are performed regardless of if an LBT failure indication is received from lower layers. When LBT is not performed by the lower layers, LBT failure indication is not received from lower layers.

To support sidelink on unlicensed spectrum (SL-U), LBT needs to be performed before transmission on sidelink and the mechanisms adopted in NR-U can be used as a baseline. In NR-U, from the UE’s perspective, LBT needs to be carried out before each UL transmission, no matter of which type of UL transmission. Therefore, similar principle should apply to SL-U, i.e., UE needs to perform LBT before each SL transmission including PSSCH, PSCCH and PSFCH.

Proposal 1: In SL-U, LBT needs to be performed before each transmission on sidelink including PSSCH, PSCCH and PSFCH.

Usage of LBT failure indication

In last meeting, RAN2 has achieved the following agreement.

- SL-specific LBT failure indication from PHY is needed for SL-specific consistent LBT failure detection in the MAC. How/whether it is used for other purposes can be further discussed.

We think besides the support of consistent LBT failure detection and recovery, there are other usage of the LBT failure indication, i.e., handling of counters and timers. Actually in NR-U, if the transmission is not performed due to LBT failure, some counters may not be increased and some timers may not be started/stopped.

When it comes to SL-U, similar principle should apply. For CG, we have already introduced sl-MaxTransNum to control the number of transmissions and upon reaching the maximum value, UE needs to flush the HARQ buffer. Therefore in Rel-18, we need to discuss whether the transmission number is incremented or not when LBT failure happens. In addition, with the introduction of SL DRX, quite many timers are maintained in MAC and upon reception of LBT failure indication, some timers may not be (re-)started while some timers may need to be (re-)started regardless of LBT failure. So from this perspective, the acknowledgement of the LBT outcome is necessary to the MAC layer and the LBT failure indication needs to be indicated to MAC for other purposes.

Proposal 2: In SL-U, LBT failure indication is provided from lower layer to MAC layer to maintain the timers and counters.

SL-specific consistent LBT failure detection

In last RAN2 meeting, there was some discussion on the granularity of SL-specific consistent LBT failure detection and one proposed solution is per DST or per link or per cast type. There was not too much support on this option and a WA is proposed to rule this option out.

- Proposal 3-2 (15/21): RAN2 to discuss whether to make the working assumption that SL-specific consistent LBT failure detection at per cast type/per DST/per unicast link level is not supported in Rel-18 SL-U (unless the motivation/necessity can be fully justified). - Continue the discussion next meeting.

However, we think this option is also feasible considering SL is for the transmission between UEs. For this option, UE counts LBT failure for all SL transmissions targeted at a certain destination no matter in which resource pool/RB set the SL transmission is performed, and upon the counter reaching the maximum configured value, consistent LBT failure is triggered for this destination. With this operation, the impact caused by consistent LBT failure detection is minimized compared with per RP/BWP solution since if consistent LBT failure is detected on a RP/BWP then all the SL transmissions carried on this RP/BWP will be impacted. Therefore, we do see some benefit for this option. In addition, we think it is too early to have WA to rule this solution out since we anyway needs to wait for the reply LS from RAN1 before we can determine the granularity of SL-specific consistent LBT failure detection. So we propose RAN2 to further investigate all the candidate solutions until we get the reply LS from RAN1.

Proposal 3: Regarding the granularity of SL-specific consistent LBT failure detection, RAN2 shall not rule any candidate solutions out before we get the reply LS from RAN1.

SL-specific consistent LBT failure recovery

Even though the granularity of SL-specific consistent LBT failure detection cannot be determined for now, we can try to discuss some general rules for SL-specific consistent LBT failure recovery in SL-U. In NR-U, if consistent LBT failure is detected on SpCell, the UE switches to another BWP that has not triggered consistent LBT failure and initiates RACH. If consistent LBT failure is detected on SCell or SpCell and not cancelled, UE triggers consistent LBT failure report to the network to indicate the serving cell that occurred consistent LBT failure.

For UEs operating in mode 2, there were some proposals to support autonomous switching to other set of resources if SL-specific consistent LBT failure is detected. However we think it is too early to agree with this proposal since whether we can have this autonomous switching or not depends on the granularity of SL-specific consistent LBT failure detection. For example, if we finally agree per BWP granularity for SL-specific consistent LBT failure detection, then when SL-specific consistent LBT failure happens, since only one BWP is configured for SL, the UE has no chance for autonomous recovery. So we think we should not start the discussion on autonomous switching before we determine the granularity of SL-specific consistent LBT failure detection.

Proposal 4: RAN2 does not agree to support autonomous switching for SL-specific consistent LBT failure recovery before we determine the granularity of SL-specific consistent LBT failure detection.

In last RAN2 meeting, RAN2 has achieved the following agreement for consistent LBT failure report.

- Support the mechanism that a mode-1 UE can indicate the SL-specific consistent LBT failure to the gNB. FFS on a mode-2 UE in RRC_CONNECTED.

We think for a mode-2 UE in RRC_CONNECTED, some information to the network to assist the network to recover the UE from SL-specific consistent LBT failure is also needed. For example, with this information, the network can reconfigure the set of resources that occurred consistent LBT failure or reconfigure the UE from mode 2 to mode 1 to recovery from the SL-specific consistent LBT failure. There is no strong motivation to require a mode-2 UE to enter RRC-CONNECTED state to report the SL-specific consistent LBT failure if the mode-2 UE is not operating in RRC_CONNECTED state, but there is also no need to restrict a mode-2 UE in RRC_CONNECTED from reporting this information to the network.

Proposal 5: In SL-U, support the mechanism that a mode-2 UE in RRC_CONNECTED can indicate the SL-specific consistent LBT failure to the gNB.

Regarding the detailed design of the SL-specific consistent LBT failure report, there was some discussion last meeting and a majority of companies support to reuse the NR-U mechanism, i.e., consistent LBT failure MAC CE. We also support to agree with this general concept and leave the detailed content design which depends on the granularity of SL-specific consistent LBT failure detection as FFS.

Proposal 6: In SL-U, the MAC CE based signalling can be supported to signal the SL-specific consistent LBT failure to the gNB. FFS the detailed design of this MAC CE.

Cancellation of SL-specific consistent LBT failure

In NR-U, UE needs to cancel the consistent LBT failure on the serving cells if any in the following cases.

- Case 1: upon deactivation of the SCell, cancel, if any, triggered consistent LBT failure for the SCell.

- Case 2: upon MAC reset, cancel, if any, triggered consistent LBT failure

- Case 3: upon reception of a PDCCH for BWP switching, cancel, if any, triggered consistent LBT failure for this Serving Cell

- Case 4: upon successful transmission of the consistent LBT failure MAC CE, cancel all the triggered consistent LBT failure(s) in SCell(s) for which consistent LBT failure was indicated

- Case 5: upon complete of RACH on the SpCell which has triggered consistent LBT failure, cancel all the triggered consistent LBT failure(s) in the SpCell

- Case 6: upon reconfiguration of the consistent LBT failure related parameters for this serving cell, cancel all the triggered consistent LBT failure(s) in this serving cell

When it comes to SL-U, generally we should reuse the NR-U cancellation mechanism as much as possible. However since we have not determined the granularity of SL-specific consistent LBT failure detection as well as the configuration granularity of the parameters used for SL-specific consistent LBT failure detection, it seems too early to agree with some of the cancellations. However, we think at least the first two cancellation conditions which are not related to any other open issues can be discussed and agreed.

Proposal 7: In SL-U, the SL-specific consistent LBT failure is cancelled in the following cases. FFS other cancellation cases.

- Case 1: upon deactivation of the serving cell, cancel, if any, triggered SL-specific consistent LBT failure for the serving cell.

- Case 2: upon MAC reset, cancel, if any, triggered SL-specific consistent LBT failure

LBT impact on DRX operation

During Rel-17, SL DRX feature was introduced for unicast, groupcast and broadcast and sl-drx-HARQ-RTT-Timer was defined for unicast and groupcast during which time UE is allowed to fall asleep if there is no other active timer running. Based on TS 38.321 [2], this timer is started in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback or in the slot following the end of PSSCH transmission under different conditions.

3> if PSFCH resource is configured for the SL grant associated to the SCI: 4> if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to unicast; or 4> if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to groupcast and positive-negative acknowledgement is selected; 5> start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or ….. 4> if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is scheduled in the SCI: 5> start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

Then the next issue is when PSFCH transmission or PSSCH transmission is blocked by LBT, whether this timer should be started or not? Since either the network (mode 1) or the TX UE (mode 2) will schedule this UE for retransmission in subsequent resource, if this timer is not started, sl-drx-RetransmissionTimer will not be started. In that case the UE may fall asleep if there is no other active timer running and miss the subsequent retransmmissions. To avoid this issue and also aligned with NR-U, this timer should be started regardless of LBT outcome.

Proposal 8: In SL-U, sl-drx-HARQ-RTT-Timer is started regardless of LBT outcome of PSFCH transmission and PSSCH transmission.

In last RAN2 meeting, there were some proposals to discuss about the impact of multiple PSFCH resources. However we think according to the following RAN1 agreement, multiple PSFCH resources is still under discussion. Therefore we suggest RAN2 to not start the corresponding discussion until RAN1 concludes.

Agreement To address PSFCH transmission dropping due to LBT failure, the followings are to be studied: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Alt 2: PSFCH resources are dynamically indicated - Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE - Alt 4: drop PSFCH transmission - Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB - Combination of above alternatives are not precluded - FFS details of above alternatives

Proposal 9: RAN2 shall not start the discussion on the impact of multiple PSFCH resources until RAN1 concludes on this.

LBT impact on SL-RLF

In Rel-16, we have introduced HARQ-based Sidelink RLF detection which is used to detect Sidelink RLF based on a number of consecutive DTX on PSFCH reception occasions for a PC5-RRC connection. UE maintains a variable which needs to be incremented by 1 in case PSFCH reception is absent on the PSFCH reception occasion and upon this variable reaching the maximum number, HARQ-based Sidelink RLF is detected.

When it comes to SL-U, with the impact of LBT, PSFCH may be not available due to LBT failure, without the knowledge of the reason for the absent of PSFCH, UE may increase the variable blindly and SL-RLF may be triggered frequently especially when the channel conditions are not good or the sl-maxNumConsecutiveDTX is configued with a relatively low value. Therefore, we think some additional mechanism needs to be introduced to assist the UE to distinguish between LBT failure and real PSFCH absent to overcome the LBT impact on HARQ-based Sidelink RLF.

Proposal 10: RAN2 to discuss how to overcome the LBT impact on HARQ-based Sidelink RLF.

3	Conclusion

In this contribution, we discussed about LBT for sidelink operation on unlicensed spectrum and provide corresponding proposals:

Proposal 1: In SL-U, LBT needs to be performed before each transmission on sidelink including PSSCH, PSCCH and PSFCH.

Proposal 2: In SL-U, LBT failure indication is provided from lower layer to MAC layer to maintain the timers and counters.

Proposal 3: Regarding the granularity of SL-specific consistent LBT failure detection, RAN2 shall not rule any candidate solutions out before we get the reply LS from RAN1.

Proposal 4: RAN2 does not agree to support autonomous switching for SL-specific consistent LBT failure recovery before we determine the granularity of SL-specific consistent LBT failure detection.

Proposal 5: In SL-U, support the mechanism that a mode-2 UE in RRC_CONNECTED can indicate the SL-specific consistent LBT failure to the gNB.

Proposal 6: In SL-U, the MAC CE based signalling can be supported to signal the SL-specific consistent LBT failure to the gNB. FFS the detailed design of this MAC CE.

Proposal 7: In SL-U, the SL-specific consistent LBT failure is cancelled in the following cases. FFS other cancellation cases.

- Case 1: upon deactivation of the serving cell, cancel, if any, triggered SL-specific consistent LBT failure for the serving cell.

- Case 2: upon MAC reset, cancel, if any, triggered SL-specific consistent LBT failure

Proposal 8: In SL-U, sl-drx-HARQ-RTT-Timer is started regardless of LBT outcome of PSFCH transmission and PSSCH transmission.

Proposal 9: RAN2 shall not start the discussion on the impact of multiple PSFCH resources until RAN1 concludes on this.

Proposal 10: RAN2 to discuss how to overcome the LBT impact on HARQ-based Sidelink RLF.

4	Reference

[1] RAN2#119 bis-e meeting minutes.