3GPP TSG-RAN WG2 Meeting #120 	R2-2212095

Toulouse, France, 14th - 18th Nov., 2022

Agenda item:	6.12.3

Source:	Huawei, HiSilicon, vivo

Title:	Mismatch issue on RAR reception on RedCap specific initial DL BWP

Document for:	Discussion and Decision

- Introduction

In RAN2#119-e, the below text proposal is agreed. However, there are still some cases not captured by current text. In this paper, we will further discuss some identification related issues for RedCap UEs.

Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is a RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall: 1> if initialUplinkBWP-RedCap is configured: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap. 1> else: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP. 1> if initialDownlinkBWP-RedCap is configured: 2> If the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier; 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP. 2> else: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap. 1> else: 2> monitor the PDCCH on the BWP configured by initialDownlinkBWP.

- Discussion

According to current spec, if initialDownlinkBWP-RedCap is configured, and only when the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier, the RedCap UE will monitor the PDCCH on the BWP configured by initialDownlinkBWP, for other cases the RedCap UE will monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap. In fact, some other cases still have the similar identification problem. We discuss a scenario/assumption which results to the misalignment between UE and NW on the understanding of DL BWP to monitor RAR.

Observation 1: [Issue Scenario] It is a valid configuration that only RedCap-specific initial DL BWP is configured but RedCap-specific initial UL BWP is NOT configured.

Under this configuration case in observation 1, there are several cases where NW may not be able to identify the RedCap UE.

Case 1a: No RACH partitioning for RedCap feature configured on initialUplinkBWP:

In this case, since no RACH partitioning for RedCap feature is configured, the RedCap UE will use the RA resource which is shared with the legacy NR UEs in the legacy initial UL BWP. And the RedCap UE will monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap according to the current MAC spec. While NW cannot identify the UE is a RedCap UE or a NR UE, NW may only send RAR on legacy initial DL BWP.

Case 1b: RACH partitioning for RedCap feature is configured on initialUplinkBWP, but RedCap feature is not set as the highest priority:

If a dedicated RACH resource is configured for RedCap but its feature priority is not set to the highest one, e.g. the Slicing feature is set to the highest priority, and the RedCap feature is set to a lower priority.

In the RACH partitioning, how to set the priority for different features is up to NW implementation. When this case is configured by NW, the RedCap UE may select the random access resource which is configured for slicing not for RedCap, thus the similar case happens. NW cannot recognize it is a RedCap UE and may not send the RAR in initialDownlinkBWP-RedCap.

Case 2a: Msg1 based SI request on initialUplinkBWP if NUL is selected:

Since the RedCap-specific initial UL BWP is not configured, si-RequestConfigRedcap is not configured either. According to TS 38.331, RedCap UE will initiate the Random Access procedure on normal uplink using the configuration in si-RequestConfig on legacy initial UL BWP. Then NW is also not able to identify the RedCap UE.

Case 2b: Msg1 based SI request on initialUplinkBWP if SUL is selected:

This is similar to case 2a. And this case was already addressed by current spec, i.e. no RAR mismatch issue any more.

Observation 2: There could be following cases, where the NW is not able to identify the CBRA Msg1 is from RedCap UE, and then does not know whether to transmit the corresponding RAR on initialDownlinkBWP-RedCap or initialDownlinkBWP:

Case 1a: No RACH partitioning for RedCap feature is configured on initialUplinkBWP;

Case 1b: RACH partitioning for RedCap feature is configured on initialUplinkBWP, but RedCap feature is not set as the highest priority;

Case 2a: Msg1 based SI request on initialUplinkBWP if NUL is selected;

Case 2b: Msg1 based SI request on initialUplinkBWP if SUL is selected. [Already addressed by last meeting CR]

To address the issue where UE monitoring and NW sending RAR on different DL BWPs, we consider different options for RAN2 to discussion.

Option 1 UE based solution: (see the option1 MAC TP) - Only if RedCap UE uses the RedCap-specific RACH resource for CBRA, the RedCap UE receives RAR on the configured initialDownlinkBWP-RedCap. Otherwise, the UE monitors RAR on initialDownlinkBWP for IDLE/INACTIVE case, or switches to initialDownlinkBWP for CONNECTED case if the bandwidth of initialDownlinkBWP is supported.

The MAC TP is given as below:

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell: 1> if PRACH occasions are not configured for the active UL BWP: 2> if the UE is a RedCap UE; and 2> if initialUplinkBWP-RedCap is configured: 3> switch the active UL BWP to BWP indicated by initialUplinkBWP-RedCap. 2> else: 3> switch the active UL BWP to BWP indicated by initialUplinkBWP. 2> if the Serving Cell is an SpCell: 3> if the UE is a RedCap UE; and 3> if initialDownlinkBWP-RedCap is configured: 4> if RedCap UE selects the RedCap specific RA resource or the bandwidth of initialDownlinkBWP is not supported by RedCap: 5> switch the active DL BWP to BWP indicated by initialDownlinkBWP-RedCap. 4> else: 5> switch the active DL BWP to BWP indicated by initialDownlinkBWP. 3> else: 4> switch the active DL BWP to BWP indicated by initialDownlinkBWP. 1> else: 2> if the Serving Cell is an SpCell: 3> if the active DL BWP does not have the same bwp-Id as the active UL BWP: 4> switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP. ---------------------------------------------------Skiped Text-------------------------------------------- Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is a RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall: 1> if initialUplinkBWP-RedCap is configured for the selected carrier: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap. 1> else: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP. 1> if initialDownlinkBWP-RedCap is configured: 2> if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier, or 2> if RedCap UE does not select the RedCap specific RA resource: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP. 2> else: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap. 1> else: 2> monitor the PDCCH on the BWP configured by initialDownlinkBWP.

However, this solution doesn’t work well when the identification issue happens to UE in RRC_CONNECTED state. If the bandwidth of legacy initial DL BWP is wider than the maximum Bandwidth of RedCap UE, UE is not able to switch to the legacy initial DL BWP. Then, the NW does not know that the RAR should be sent on initialDownlinkBWP-RedCap since the NW is not aware of this Msg1 is from a RedCap UE.

Observation 3: The pure UE based solution can not address the case where the bandwidth of initialDownlinkBWP is NOT supported by RedCap UE, i.e. option 1 does not fully solve the issue.

Then, another option2 to rely on NW implementation is proposed, which requires some recommendations to the NW side and keep the MAC spec as it is. Then all the above cases can be solved by this NW implementation.

Option 2 NW based solution: (Capture below both two candidate recommendations in the RAN2 minutes/or as a NOTE in MAC specification). - If the network is not able to identify the RedCap UE by Msg1, there are two recommendations: - if initialDownlinkBWP-RedCap is configured, NW should transmit the corresponding RAR on both initialDownlinkBWP-RedCap and initialDownlinkBWP, or - The initialDownlinkBWP-RedCap should be absent (if the bandwidth of initialDownlinkBWP is supported by RedCap UE).

In conclusion, we give the following proposal:

Proposal 1: To address the mismatch issue on RAR reception on RedCap specific initial DL BWP, RAN2 agree the option 2 below:

Option 1 UE based solution: (with the MAC TP)

- Only if RedCap UE uses the RedCap-specific RACH resource for CBRA, the RedCap UE receives RAR on the configured initialDownlinkBWP-RedCap. Otherwise, the UE monitors RAR on initialDownlinkBWP for IDLE/INACTIVE case, or switches to initialDownlinkBWP for CONNECTED case if the bandwidth of initialDownlinkBWP is supported.

Option 2 NW based solution: (Capture below both two candidate recommendations in the RAN2 minutes/or as a NOTE in MAC specification).

- If the network is not able to identify the RedCap UE by Msg1, there are two recommendations:

- if initialDownlinkBWP-RedCap is configured, NW should transmit the corresponding RAR on both initialDownlinkBWP-RedCap and initialDownlinkBWP, or

- the initialDownlinkBWP-RedCap should be absent (if the bandwidth of initialDownlinkBWP is supported by RedCap UE).

- Conclusion

The contribution focuses on MAC issues for RedCap UEs. Corresponding proposals are listed as below:

Observation 1: [Issue Scenario] It is a valid configuration that only RedCap-specific initial DL BWP is configured but RedCap-specific initial UL BWP is NOT configured.

Observation 2: There could be following cases, where the NW is not able to identify the CBRA Msg1 is from RedCap UE and then does not know whether to transmit the corresponding RAR on initialDownlinkBWP-RedCap or initialDownlinkBWP:

- Case 1a: No RACH partitioning for RedCap feature is configured on initialUplinkBWP;

- Case 1b: RACH partitioning for RedCap feature is configured on initialUplinkBWP, but RedCap feature is not set as the highest priority;

- Case 2a: Msg1 based SI request on initialUplinkBWP if NUL is selected;

- Case 2b: Msg1 based SI request on initialUplinkBWP if SUL is selected. [Already addressed by last meeting CR]

Observation 3: The pure UE based solution can not address the case where the bandwidth of initialDownlinkBWP is NOT supported by RedCap UE, i.e. option 1 does not fully solve the issue.

Proposal 1: To address the mismatch issue on RAR reception on RedCap specific initial DL BWP, RAN2 agree the option 2 below:

Option 1 UE based solution: (with the MAC TP)

- Only if RedCap UE uses the RedCap-specific RACH resource for CBRA, the RedCap UE receives RAR on the configured initialDownlinkBWP-RedCap. Otherwise, the UE monitors RAR on initialDownlinkBWP for IDLE/INACTIVE case, or switches to initialDownlinkBWP for CONNECTED case if the bandwidth of initialDownlinkBWP is supported.

Option 2 NW based solution: (Capture below both two candidate recommendations in the RAN2 minutes or as a NOTE in MAC specification).

- If the network is not able to identify the RedCap UE by Msg1, there are two recommendations:

- if initialDownlinkBWP-RedCap is configured, NW should transmit the corresponding RAR on both initialDownlinkBWP-RedCap and initialDownlinkBWP, or

- the initialDownlinkBWP-RedCap should be absent (if the bandwidth of initialDownlinkBWP is supported by RedCap UE).