3GPP TSG RAN WG2 Meeting #120                                                            R2-2212192

# Toulouse, France, 14th - 18th November, 2022

Agenda item:		8.14.2

Source:			Huawei, HiSilicon

Title:		Discussion on QoE measurements for MBS broadcast services

Document for:	 	Discussion and decision

# Introduction

In the WID [1], one of the objectives is to specify a support for QoE measurements in RRC_IDLE and RRC_Inactive states for MBS Broadcast services. Both RAN2 and RAN3 made some initial agreements on this topic during the WG meetings in August and October, as summarized below:

RAN2 agreements - The gNB can send the QoE configuration for MBS broadcast service to UE by RRC message in RRC_CONNECTED via dedicated signalling. The UE stores the configuration for QoE and performs the application layer measurement for MBS broadcast service. - FFS if configuration can be done in IDLE/INACTIVE states. - FFS how does gNB determine which UEs can be configured with MBS QoE measurements - FFS if there is a new explicit indicator or new service type used for MBS QoE configuration in RRC_IDLE/RRC_INACTIVE. Wait for RAN3 progress and SA4 LS reply to RAN3. - The baseline principles for QoE measurement collection for MBS services in RRC_INACTIVE and RRC_IDLE states are: - 1) The UE is configured with IDLE/INACTIVE QoE via RRC. - 2) The UE buffers the QoE reports generated while in RRC IDLE/INACTIVE state. - 3) FFS if UE can setup/resume RRC connection just for QoE reporting, or whether the QoE reports are sent to the network when the UE moves to RRC CONNECTED state due to other reasons. - When the UE moves to RRC_CONNECTED state, the UE sends the QoE measurements availability indication to the gNB - For buffering of QoE reports generated in RRC IDLE/INACTIVE state, RAN2 should discuss at least the minimal memory size requirement. FFS if AS layer is responsible for storing the QoE reports (as in Rel-17).

RAN3 agreements In RAN3#117e - Both signalling based and management based QoE measurements in RRC INACTIVE/IDLE mode shall be supported in Rel-18. - UE handles area scope checking for QoE measurements in RRC INACTIVE/IDLE mode. - Whether UE AS layer or UE APP layer handle the area scope is to be discussed based on RAN2 progress. - Support MBS broadcast service INACTIVE/IDLE QoE first - UE shall keep the QoE configuration for MBS broadcast service configured in RRC_CONNECTED even when UE switches to RRC_IDLE and RRC_INACTIVE. - If the UE receives the configuration in RRC connected state, a common QoE configuration mechanism is used to support QoE measurement configuration pertaining to MBS broadcast service for all RRC states, where the Rel-17 QoE configuration mechanism is adopted as baseline. In RAN3#117b-e - Whether UE can only report the INACTIVE/IDLE QoE reports to gNB when the UE has entered to the RRC_CONNECTED due to other reasons is pending to RAN2 discussion.

Based on RAN2 and RAN3 progress made in previous meetings, in the subsequent sections, we provide our views on the issues related to QoE collection for MBS broadcast that remain to be solved.

# QoE measurements for MBS broadcast

## Overall configuration procedure

The following agreements were made by RAN2 and RAN3 relevant for configuration procedure of QoE measurements for MBS Broadcast service:

- RAN2 agreements:

  - The gNB can send the QoE configuration for MBS broadcast service to UE by RRC message in RRC_CONNECTED via dedicated signalling. The UE stores the configuration for QoE and performs the application layer measurement for MBS broadcast service.

  - FFS if configuration can be done in IDLE/INACTIVE states.

- RAN3 agreements:

  - Both signalling based and management based QoE measurements in RRC INACTIVE/IDLE mode shall be supported in Rel-18.

  - UE shall keep the QoE configuration for MBS broadcast service configured in RRC_CONNECTED even when UE switches to RRC_IDLE and RRC_INACTIVE.

  - If the UE receives the configuration in RRC connected state, a common QoE configuration mechanism is used to support QoE measurement configuration pertaining to MBS broadcast service for all RRC states, where the Rel-17 QoE configuration mechanism is adopted as baseline.

During the previous meeting RAN2 discussed whether to use RRC Reconfiguration or RRC Release message to configure QoE measurements for MBS broadcast. In our view, reusing RRC Reconfiguration message to configure QoE measurements for MBS broadcast is the simplest approach and is also aligned with RAN3 agreement to adopt Rel-17 QoE configuration mechanism as a baseline.

- QoE measurements for MBS broadcast are configured to the UE via RRC Reconfiguration message.

Furthermore, at RAN2#119-e meeting, some companies proposed to consider enabling QoE configurations via SIB or other broadcast means, e.g. MCCH channel. The argument in favour of such approach is that the UEs receiving MBS broadcast service may remain in RRC IDLE/INACTIVE state for the majority of the service reception time. In our opinion however, the benefits have to be weighed against the feasibility and additional complexity of such mechanism:

- Firstly, it should be noted that even though the UEs may receive MBS broadcast in RRC IDLE/INACTIVE states, they are expected to connect to the network from time to time for another purposes, e.g. for upper layer signaling, to exchange data related to other services etc.

- Moreover, it is sufficient for the network to configure only a limited number of UEs in a certain area for QoE collection to obtain a good representation of the QoE of the MBS service and there is no need to configure all the UEs in an area. Actually, doing that in the presence of potentially a very high number of MBS broadcast UEs, could lead to high and unnecessary signalling overhead generated due to QoE reporting.

- In addition, currently a QoE configuration container can be as big as 8000 bytes while the size limitation for SI message is currently 2976 bits due to physical layer design restrictions.

- MCCH channel does not have this limitation, but providing QoE configuration via MCCH would certainly have an impact on MCCH reception performance as well as on MBS UE’s power consumption in case MCCH change notification mechanism was to be reused when QoE configuration is provided.

- There are numerous aspects and issues which would have to be resolved in order to support QoE configuration via broadcast, i.e. signalling details, UE procedures, signaling overhead issues, impact to MBS UEs and MBS performance, coordination between dedicated and common configurations etc.

Observation 1: Only a limited number of UEs receiving MBS broadcast service needs to be configured for QoE measurements for the network to obtain a good representation of the service quality in a specific area.

Observation 2: There are numerous aspects and issues which would have to be resolved in order to support QoE configuration via broadcast, i.e. signalling details, UE procedures, signaling overhead issues, impact to MBS UEs and MBS performance, coordination between dedicated and common configurations etc.

According to the discussion in RAN3#117bis, most companies think the QoE measurement is configured when the UE is in RRC_CONNECTED.

Based on the observations and considerations above, we propose to agree on the following:

- QoE measurement configuration via broadcast signaling (e.g. System Information, MCCH/MTCH etc.) is not supported.

## QoE configuration storage

QoE configuration storage was tentatively discussed by RAN3 and it was rather considered it should be addressed by RAN2.

RAN3: Proposal 4: The following aspects shall be discussed by RAN2: which layer(e.g. app, as layer) is responsible to keep configured QoE configuration for MBS broadcast service at ue side when ue is in rrc_idle. how long UE shall keep the QoE configuration for MBS broadcast service.

The Rel-17 QoE principles with respect to QoE configuration storage are as follows:

- When the UE receives the QoE configuration from the network, the UE stores the configuration in both AS layer and APP layers. In AS layer the QoE container is not stored while other parameters are, i.e. measConfigAppLayerId-r17, serviceType-r17, pauseReporting-r17, transmissionOfSessionStartStop-r17 (we put RAN visible QoE aside for the moment)

- The configuration in AS and APP layers is synchronized in general, e.g. whenever the QoE configuration is released from AS layer, it is also released from APP layer.

- The UE clears the QoE configuration when the UE goes into RRC_IDLE.

For QoE for MBS broadcast service, it is expected that the UE should store the QoE configuration in all RRC states, including RRC_IDLE state.

- When the UE goes into RRC_IDLE, the UE AS layer stores QoE configuration for MBS broadcast (except for QoE container).

- When the UE goes into RRC_IDLE, the application layer stores QoE configuration for MBS broadcast and continues QoE measurements (if already ongoing), since it is not notified by the UE to release the QoE configuration.

In RAN3, it was also proposed that RAN2 should discuss how long the UE shall keep the QoE configuration for MBS broadcast service. In MDT framework, when the UE receives the logged MDT configuration, T330 is started with a value set to loggingDuration parameter. The UE considers the logged MDT configuration to be valid until the timer expires, upon which the configuration is released. The definition of T330 is copied below.

| - Timer | - Start | - Stop | - At expiry |
| --- | --- | --- | --- |
| T330 | Upon receiving LoggedMeasurementConfiguration message | Upon log volume exceeding the suitable UE memory, upon initiating the release of LoggedMeasurementConfiguration procedure | Perform the actions specified in 5.5a.1.4 |

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16                    OPTIONAL,  --Need R

plmn-IdentityList-r16                       PLMN-IdentityList2-r16                   OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need M

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need M

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need M

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16,

...

},

lateNonCriticalExtension                    OCTET STRING                             OPTIONAL,

nonCriticalExtension                        LoggedMeasurementConfiguration-v1700-IEs OPTIONAL

}

–	LoggingDuration

The LoggingDuration indicates the duration for which UE is requested to perform measurement logging. Value min10 corresponds to 10 minutes, value min20 corresponds to 20 minutes and so on.

LoggingDuration information element

-- ASN1START

-- TAG-LOGGINGDURATION-START

LoggingDuration-r16 ::=   ENUMERATED {

min10, min20, min40, min60, min90, min120, spare2, spare1}

-- TAG-LOGGINGDURATION-STOP

-- ASN1STOP

The intention of having the timer in MDT is to limit the impacts on UE power consumption. We think the T330 like timer is not needed for QoE measurements for MBS broadcast services, and the reasons are as below:

- It is hard to decide on the value from network point of view. For logged MDT, the UE is quite likely to do MDT measurements when going to idle/inactive, however, it is not true for QoE measurements. If following logged MDT, and the UE is configured with a T330 like timer, it is unclear when to start/stop the timer. In addition, it may happen that the timer is short but the MBS broadcast service may last for a long time, so the value of the timer is compromised.

- If there is no T330 like timer, the UE just performs QoE measurements and reporting, pending for the MBS broadcast service status in app layer.

- Timer based QoE configuration release is not supported, i.e. the UE stores the IDLE/INACTIVE QoE configuration until it is released by the network.

## QoE continuity during state transitions

In Rel-17, the UE receives the MBS configuration for a broadcast service via MBS Control Channel, i.e. MCCH, which is common for UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED states. When the UE performs MBS broadcast services in RRC_IDLE and RRC_INACTIVE, it is possible that the UE may move to RRC_CONNECTED (or vice versa) and the UE continues to receive MBS broadcast service after the state transition. Based on this, we think it is important to agree on the principle that the UE can continue to perform the QoE measurement for a certain MBS broadcast session when the UE switches between different RRC states.

- It should be possible for the UE to continue the MBS broadcast QoE measurements for a particular QoE measurement session after the UE changes its RRC state.

## Reporting procedure of the QoE measurements for MBS broadcast

As mentioned earlier, we assume that the UE receives QoE configuration for MBS broadcast while in RRC_Connected state, and subsequently:

- When the UE is in RRC_Connected state and initiates an MBS broadcast service, the UE starts QoE measurements and sends the reports to the network as soon as they are received from application layer in the same way as in Rel-17 QoE mechanism.

- When the UE goes into RRC_IDLE/INACTIVE and only then initiates an MBS broadcast service, the UE starts QoE measurements and stores the QoE configuration and sends the reports when the UE goes back to RRC Connected state.

For RRC_IDLE state it has been already agreed that the UE should store the QoE reports and indicate to the network it has some reports to send when the UE transitions into RRC_Connected state. The following FFS has also been captured: “FFS if UE can setup/resume RRC connection just for QoE reporting, or whether the QoE reports are sent to the network when the UE moves to RRC CONNECTED state due to other reasons.”

Currently, the UE initiates RRC connection setup or resume procedure only for specific purposes determined by the establishment/resume cause. The triggering conditions include service initiation, e.g. emergency, mo-Data, or the need to perform some network procedures, e.g. RNA update. In general, for the current state transitions, the triggering conditions are about services in higher layer or special reasons like RNA update and we think these conditions are critical. For the above FFS, if the UE has stored QoE measurements for MBS broadcast services in RRC_Idle/Inactive state, we think the UE should not initiate state transitions to connected state, the reasons are as below:

- If this is allowed, it will lead to frequent state transitions, which will bring impacts to signalling overhead and extra UE power consumption.

- QoE measurement reports are gathered by the OAM system and are not utilized in real-time mode, so there is no need to have them reported as soon as they are available.

- MBS broadcast service performance may be impacted if the UE has to transition to RRC_Connected state frequently, firstly due to service reception interruption during RRC connection setup/resume procedure and secondly because the network is not immediately aware of the UE receiving MBS broadcast and might schedule unicast data at the same time where MBS data is sent. An unnecessary impact on the MBS broadcast performance due to QoE should definitely be avoided.

- If triggering due to QoE was to be specified, it would require RAN2 to address many additional issues in order to control such triggering, e.g. thresholds, timer etc. and all of this introduces unnecessary complexity.

Observation 3: Resuming/setting up an RRC connection just for the sake of reporting QoE brings no benefits while it causes MBS broadcast service performance deterioration, increases signaling overhead, impacts UE battery life and brings additional complexity.

- The UE does not setup/resume RRC connection just for QoE reporting, i.e. the QoE reports are sent to the network when the UE moves to RRC_CONNECTED state due to other reasons.

The previous agreements made by RAN2 for QoE reporting for MBS broadcast concerned only the case where the UE is in RRC_IDLE/INACTIVE state. However, the QoE measurements for MBS broadcast can also take place while the UE is in RRC_Connected state. In this case, the reporting procedure from Rel-17 can be reused and no storing of reports is needed.

- If the UE is in RRC_Connected and receives QoE report for MBS broadcast from application layer, the UE sends the report according to QoE reporting procedure from Rel-17, i.e. the report is not stored but sent immediately (unless paused).

## Selection of UEs for MBS QoE configuration

RAN2 has captured the following FFS during the previous meeting:

FFS how does gNB determine which UEs can be configured with MBS QoE measurements

When the UE is in RRC_Connected state, the network will know the UE capability to perform QoE for MBS broadcast. The issue is how to know whether the UE is interested in receiving a specific MBS broadcast session/service. In R17 MBS, the UE in RRC_Connected state sends the MBS interest indication to inform the network it is receiving or interested to receive particular MBS broadcast sessions/service(s). However, according to the procedure text of MBS interest indication in TS 38.331, the UE reports the MBS interest indication only if the MBS session is ongoing or about to start. The UE will not report the MBS interest indication to inform the gNB that the UE will (or is likely to) receive an MBS session in the RRC_IDLE/INACTIVE in the future. In consequence, the network is not able to know which of the QoE configurations it should provide to the UE. The network can use blind configuration, i.e. the network may configure all MBS broadcast QoE configurations received from the OAM to all MBS broadcast QoE capable UEs which move to RRC_Connected state, but this means most of these configurations will never be used by the UEs. Also, the network does not really know how many of the configured UEs will actually perform the QoE measurements for a specific MBS session. Alternatively, the network could wait for the MBS session to start and when a UE receiving such session moves to RRC_Connected state, it could be configured with proper QoE configuration. However, most of the UEs receiving MBS broadcast session will remain in RRC_IDLE/INACTIVE as much as possible and will only connect to the network due to other reasons. This means that the network may have very limited opportunities to configure QoE measurements for an already ongoing MBS session and applying QoE measurements from the beginning of the session would also be very hard.

Observation 4: Forcing the gNB to utilize blind configuration of MBS broadcast QoE to all MBS capable UEs is sub-optimal for both the UE and the network in terms of signaling overhead, memory/storage requirements, predictability of receiving QoE measurements etc.

One way to address this issue would be to allow sending QoE configurations to the UEs that are in RRC_IDLE/INACTIVE state, but this approach presents many drawbacks, as mentioned in section 2.1 of this document. On the other hand, without any change, the network is forced to utilize blind configuration which is also very sub-optimal for both the UE and the network, as mentioned above. Therefore, we think RAN2 should investigate other possibilities to optimize the selection of the UEs for MBS broadcast configurations, e.g.:

- Lift the limitation in the MBS Interest Indication procedure that the UE reports only the MBS broadcast sessions, which are ongoing or “about to start”. E.g. based on knowing the MBS broadcast QoE capability of the UE, the network may request the UE to include in MII all the MBS broadcast sessions that upper layers configured it to receive (e.g. via USD), even if they are supposed to start at a later time, e.g. in an hour etc.

- Allow the network to indicate to the UE the IDs of MBS broadcast sessions for which it is interested in receiving QoE measurements (i.e. the ones for which OAM provided QoE configurations). Based on such indication, the UE could inform the network in case it is configured to receive this MBS session or when it is about to start receiving this session, so that the network may configure the UE with proper QoE configuration.

- RAN2 should investigate the means for the gNB to identify which UEs should be provided with MBS broadcast QoE configuration for a specific MBS session via, e.g.:

- Allowing the network to indicate to the UE the IDs of MBS broadcast sessions for which it is interested in receiving QoE measurements.

- The UE indicating to the network when the UE is configured with or receiving/starting to receive the indicated MBS sessions.

## Area scope checking

RAN3 has made the following agreement:

UE handles area scope checking for QoE measurements in RRC INACTIVE/IDLE mode. Whether UE AS layer or UE APP layer handle the area scope is to be discussed based on RAN2 progress.

For UE AS layer option, the network can send some area scope configurations to the UE and the UE needs to inform APP layer whenever it leaves or enters a certain area. Only based on such indication the APP layer may know whether a certain QoE job should be triggered or not.

For UE APP layer option, the scope is checked in the APP layer whenever the application is about to start. SA4 specifications has defined area scope information inside the QoE configuration container, i.e. LocationFilter. This option has been described to RAN2 by SA4 in the LS in [2]. What we can learn from this LS is that the application layer can determine UE’s location on per cell granularity and based on the LocationFilter provided in the QoE container, it may determine whether or not to trigger QoE measurements. Hence, this offers a ready solution to area scope handling for MBS broadcast service.

Observation 5: SA4 specifications already provide a readily available solution for handling QoE measurement area scope for MBS broadcast services.

- Area scope verification for QoE collection for MBS broadcast should be performed by application layer. Send an LS to SA4 informing about RAN2 assumption.

## UE buffering of QoE reports for broadcast service

RAN2 has made the following agreement during the previous meeting:

For buffering of QoE reports generated in RRC IDLE/INACTIVE state, RAN2 should discuss at least the minimal memory size requirement. FFS if AS layer is responsible for storing the QoE reports (as in Rel-17).

In Rel-17, for storing the QoE reports during RAN overload, RAN2 decided to use AS layer’s memory. However, the requirements for Rel-18 are very different from these in Rel-17:

- RAN overload is a temporary situation while MBS broadcast reception in RRC_IDLE/INACTIVE states is a normal mode of operation which can last even for the whole duration of the service.

- MBS broadcast can be used to deliver many services, including, e.g. video and VR services.

For both these reasons the memory requirements for MBS broadcast will be much higher than in case of pause/resume mechanism, e.g. SA4 indicated in the past in the LS in [4] that an exemplary QoE trace for VR service produced a QoE report with a size reaching 18 kBytes. As per RAN2 analysis in [3], application layer offers much more memory size than AS layer and this may be required in case of MBS broadcast. Furthermore, it should be noted that the AS memory which is used during pause may not always be available for reuse for storing the report in RRC INACTIVE as RAN2 agreed that the reports stored by the UE during pause are preserved when the UE moves to RRC INACTIVE state. This means that there will have to be an additional requirement for AS layer memory size at the UE which is known to be expensive.

Observation 6: The memory requirements for storing QoE reports generated for MBS broadcast in RRC_IDLE/INACTIVE states will be much higher than in case of pause due to RAN overload.

In addition, NR MBS QoE has not been defined in SA4 specifications yet. Since SA4 is aware that the MBS broadcast service can be received in RRC_IDLE/INACTIVE states, they may assume that the UE client will collect QoE measurements and store them in the application layer. In LTE, SA4 defined the MBMS QoE measurement. In our understanding, SA4 assumes the QoE results are stored in the application layer in MBMS QoE measurement.

Observation 7: The SA4 mechanism for QoE collection for MBS broadcast is unclear and it is unclear what are SA4 assumptions on storing the QoE reports for MBS broadcast.

Based on the above observations, we have doubts on whether AS layer can be the one to store MBS broadcast QoE reports generated in RRC_IDLE/INACTIVE state. Before RAN2 will be able to decide, RAN2 should consult SA4, therefore we propose the following:

- Send an LS to SA4 asking to clarify the following aspects:

- Whether SA4 plans to discuss storing of QoE reports generated for MBS broadcast while the UE is in RRC_IDLE/INACTIVE state.

- What is SA4 view on the memory requirements for storing QoE reports for MBS broadcast, e.g. depending on the service delivered via MBS broadcast or considering that the MBS broadcast service may be provided fully in RRC_IDLE/INACTIVE state.

# Conclusion

Based on the discussion in this paper, the following observations and proposals are made:

MBS broadcast QoE configuration

Observation 1: Only a limited number of UEs receiving MBS broadcast service needs to be configured for QoE measurements for the network to obtain a good representation of the service quality in a specific area.

Observation 2: There are numerous aspects and issues which would have to be resolved in order to support QoE configuration via broadcast, i.e. signalling details, UE procedures, signaling overhead issues, impact to MBS UEs and MBS performance, coordination between dedicated and common configurations etc.

- QoE measurements for MBS broadcast are configured to the UE via RRC Reconfiguration message.

- QoE measurement configuration via broadcast signaling (e.g. System Information, MCCH/MTCH etc.) is not supported.

- When the UE goes into RRC_IDLE, the UE AS layer stores QoE configuration for MBS broadcast (except for QoE container).

- When the UE goes into RRC_IDLE, the application layer stores QoE configuration for MBS broadcast and continues QoE measurements (if already ongoing), since it is not notified by the UE to release the QoE configuration.

- Timer based QoE configuration release is not supported, i.e. the UE stores the IDLE/INACTIVE QoE configuration until it is released by the network.

QoE measurements continuity

- It should be possible for the UE to continue the MBS broadcast QoE measurements for a particular QoE measurement session after the UE changes its RRC state.

QoE measurements reporting

Observation 3: Resuming/setting up an RRC connection just for the sake of reporting QoE brings no benefits while it causes MBS broadcast service performance deterioration, increases signaling overhead, impacts UE battery life and brings additional complexity.

- The UE does not setup/resume RRC connection just for QoE reporting, i.e. the QoE reports are sent to the network when the UE moves to RRC_CONNECTED state due to other reasons.

- If the UE is in RRC_Connected and receives QoE report for MBS broadcast from application layer, the UE sends the report according to QoE reporting procedure from Rel-17, i.e. the report is not stored but sent immediately (unless paused).

Selection of the UEs for MBS broadcast QoE configuration

Observation 4: Forcing the gNB to utilize blind configuration of MBS broadcast QoE to all MBS capable UEs is sub-optimal for both the UE and the network in terms of signaling overhead, memory/storage requirements, predictability of receiving QoE measurements etc.

- RAN2 should investigate the means for the gNB to identify which UEs should be provided with MBS broadcast QoE configuration for a specific MBS session via, e.g.:

- Allowing the network to indicate to the UE the IDs of MBS broadcast sessions for which it is interested in receiving QoE measurements.

- The UE indicating to the network when the UE is configured with or receiving/starting to receive the indicated MBS sessions.

Area scope handling

Observation 5: SA4 specifications already provide a readily available solution for handling QoE measurement area scope for MBS broadcast services.

- Area scope verification for QoE collection for MBS broadcast should be performed by application layer. Send an LS to SA4 informing about RAN2 assumption.

Storing of QoE reports in RRC IDLE/INACTIVE

Observation 6: The memory requirements for storing QoE reports generated for MBS broadcast in RRC_IDLE/INACTIVE states will be much higher than in case of pause due to RAN overload.

Observation 7: The SA4 mechanism for QoE collection for MBS broadcast is unclear and it is unclear what are SA4 assumptions on storing the QoE reports for MBS broadcast.

- Send an LS to SA4 asking to clarify the following aspects:

- Whether SA4 plans to discuss storing of QoE reports generated for MBS broadcast while the UE is in RRC_IDLE/INACTIVE state.

- What is SA4 view on the memory requirements for storing QoE reports for MBS broadcast, e.g. depending on the service delivered via MBS broadcast or considering that the MBS broadcast service may be provided fully in RRC_IDLE/INACTIVE state.

# References

[1] RP-221803, Enhancement on NR QoE management and optimizations for diverse services, China Unicom

[2] R2-2203846, LS Reply on SA4 requirements for QoE, SA4

[3] R2-2106775, LS on QoE report handling at QoE pause, RAN2

[4] R2-2109386, Reply LS on QoE configuration and reporting related issues, SA4