3GPP TSG-RAN WG2 Meeting #120                                                                   R2-2212330

Toulouse, France, November, 2022

Agenda Item	:	8.5.2.2

Title	:	Discussion on PDU prioritization

Source	:	Google Inc.

Work Item	:	FS_NR_XR_enh

Document for	:	Discussion and Decision

# 1		Introduction

SA2 has introduced the concept of PDU set and Data Burst [1] and defined them as below:

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services, as used in TR 26.926 [27]). In some implementations all PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In other implementations, the application layer can still recover parts all or of the information unit, when some PDUs are missing.

Data Burst: A set of data multiple PDUs generated and sent by the application in a short period of time.

NOTE: A Data Burst It can be composed by one or multiple PDU Sets.

Rel-18 is anticipated to specify an enhanced 5GS QoS framework to support different QoS handling of PDU Sets. PDU Sets can carry different content, e.g. I/B/P frames or slices/tiles within an I/B/P frame, etc

Hence, different PDU Sets have different importance and require differentiated QoS treatment to improve system efficiency, hence system capacity and improve user QoE.

# 2		PDU prioritization

PDU sets QoS parameters (PS Error Rate, PS Delay Budget, PDU Set Priority, intra and inter PDU Sets dependency, … ) are important for RAN to optimize the scheduling, the transmission and meet the QoS requirements of the XR service. The use of the concept of PDU set could also be useful for other non-real time media transmissions but they will not have the same requirements in terms of latency and reliability. Hence, using the same level of prioritization and the same prioritization rules is not needed for non-real time media transmission.

Proposal 1: New prioritization rules if defined are applicable to XR traffic only.

SA2 is currently discussing the following proposal to deliver PDU Set importance information to RAN:

- Option 1: use different QoS Flows with different priority level. PDU Set importance is mapped to existing QoS flow priority.

- Option 2: use one QoS flow for different PDU Set with different priority level

  - Option 2.1: use different sub-QoS Flow within one QoS Flow, and using sub-QoS flow Identifier in GTP-U header

  - Option 2.2: use PDU Set importance information in GTP-U header

If Option 1 is selected in SA2 there is no enhancement required at RAN and the existing QoS flow framework can be used as it is. Different PDU Sets of different importance would be mapped to different QoS flows and the current mechanism in RAN can be re-used

However, if Option 2 is selected, RAN enhancements are needed to support PDU Sets QoS requirements including enhancement to the LCP mechanism. In this case different PDU Sets of different importance could map to the same QoS flow and the QoS flow is no longer the finest granularity of QoS differentiation in the PDU Session. Hence, PDUs in the same QoS flow could be mapped to different DRBs.

The current LCP procedure does not take into consideration the remaining PS Delay Budget which could be used to ensure all PDUs in a PDU Set are delivered within the required time budget. Hence, PDUs with remaining delays close to the delay budget expiry should be treated with higher priority to ensure PDU Set integrity.

Delay Thresholds could be defined and when the remaining delays goes below these thresholds the PDUs priority level could be increased and different LCH could be used.

Also, PDU Sets with different priority could be served with different logical channels with different LCH priority and different restrictions.

Proposal 2: Consider PSDB and the remaining delay in the LCP mechanism.

Also, the current LCP parameters are configured via RRC and can be updated only via RRC reconfiguration which can add extra latency to adapt to the application adjustment of bit rate and fps.

Proposal 3: For reduced latency, use MAC CE to adapt LCP parameters.

Also, according to SA4 (S4-220505) and in some implementations, not all PDUs in a PDU-set are of similar importance. A first PDU in a PDU-set can have higher importance than the last PDU in a PDU-set. The first PDU can be carrying source information and the last PDU could be carrying repair/redundant information.

Intra-PDU-set scenarios could be considered for some implementations and awareness about dependency between PDUs in a PDU-set could be useful to assist in RAN scheduling, prioritization and efficient dropping of unnecessary PDUs.

However, in SA4 reply to SA2 [2] regarding whether the K/N FEC ratio is static for a specific XRM service, and whether application layer can provide such a ratio to 5GS. SA4 clarified that until now they have not done any analysis on applying FEC codes to XRM services.  SA4 mentioned that in real-time services, in particular with RTP and WebRTC as considered in Release 18 normative work in SA4, applying a “fixed” FEC scheme is quite often not possible as RTP source packets are typically not of identical size. Also, the reception of how many and which packets are necessary for recovery is quite dependent on a specific PDU Set.

SA4 discourages to apply “active” packet dropping to FEC protected information as it may negatively impact receiver operations (e.g., confuse the receiver (for example asking for even more FEC packets), result in additional delay, lead to wrong measurement of the network capacity, or harm fast decoding). Hence, SA4 think the 5G System should provide the requested/expected QoS and not rely on application layer FEC.

As a result, RAN2 should treat all PDUs in a PDU Set with the same level of priority and there is no need in Rel-18 to drop some PDUs in a PDU Set based on the FEC ratio.

Proposal 4:  All PDUs in a PDU Set are treated with the same priority.

# 3		Conclusions

In conclusion, we have the following proposals:

Proposal 1: New prioritization rules if defined are applicable to XR traffic only.

Proposal 2: Consider PSDB and the remaining delay in the LCP mechanism.

Proposal 3: For reduced latency, use MAC CE to adapt LCP parameters.

Proposal 4:  All PDUs in a PDU Set are treated with the same priority

# 4		References

- TR 23.700-60 v0.3.0, Study on XR (Extended Reality) and media services (Rel-18).

- S4aV220921 ‘Reply LS on further details on XR traffic’ Reply LS on further details on XR traffic, e-meeting, 11th October 2022.