3GPP TSG-RAN WG2 #120	Tdoc R2-2212392

Toulouse, November 14 –18, 2022

Agenda Item:	6.3

Source:	Ericsson

Title:	On conflict of UE preferred RRC state report

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#119bis-e, the following remained open:

R2-2209928   Corrections for the Conflict of UE Preferred RRC state Report      FGI       CR   Rel-17   38.331  17.2.0   3519     -            F   MUSIM

Moved from 6.24, not treated

This contribution further discusses the need for a correction.

# 2	Discussion

It is stated that the UE may be configured with both releasePreferenceConfig and musim-LeaveAssistanceConfig, which could then trigger a double report from the UE side. We think those techniques are used in different scenarios (i.e. releasePreferenceConfig for power saving and musim-LeaveAssistanceConfig for MUSIM) and it may not be likely that both features are configured together.

releasePreferenceConfig and musim-LeaveAssistanceConfig are used for different cases and may not be simultaneously configured.

Moreover, even if both features are configured together, it should be up to the UE implementation to make sure to not trigger both reports. For instance, if the UE would prefer to leave the current network for MUSIM purposes, it should then use the MUSIM report and not the one configured for power saving.

Triggering of double report for release preference can be avoided by UE implementation.

In RAN2#117-e. it was already agreed that such interaction between both reports would not be specified.

RAN2 does not specify any UE behavior on the interaction between power saving and MUSIM for leaving RRC connection i.e. no specification impact.

Hence, it could be further clarified that it is not expected that both reports would be triggered.

It is left up to UE implementation the interaction between releasePreferenceConfig and musim-LeaveAssistanceConfig, if both are configured. The trigger of a report for both features at the same time is not expected. No specification impact.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	releasePreferenceConfig and musim-LeaveAssistanceConfig are used for different cases and may not be simultaneously configured.

Observation 2	Triggering of double report for release preference can be avoided by UE implementation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	It is left up to UE implementation the interaction between releasePreferenceConfig and musim-LeaveAssistanceConfig, if both are configured. The trigger of a report for both features at the same time is not expected. No specification impact.